3GPP TSG-RAN WG4 Meeting #114bis	R4-2503833

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	 7.26.2.2

Source:	 vivo

Title:	Updates on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

Based on latest RAN4 agreements [1] and RAN1 agreements on signal design [2], this contribution update our simulation results for LP-WUS REFSENS, ASCS and ACS.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

RAN4 has agreed that AWGN channel model with 1% MDR is selected to sepecify RF requirements, and the target SNR is the performance without repetition. Besides, RAN4 also agreed to use OOK-4 M=4 to derive SNR value [1]:

Issue 2-1-1: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - Use worst case waveform of OOK-4 M=4 to derive SNR.

    - Some detailed aspects should be also confirmed, e.g., CRC bit (8), sequence length (8, 16), codepoint length of known part of the sequence)

In this contribution, based on latest signal design parameters in RAN1, we update the following simulation parameters for OOK-4 M=4:

- For OOK-based LP-WUS OOK-4 with M=4

- Length of information bits: [5] bits

- No CRC configured

- RM coding: Code block length 16bits or 20bits

In addition, our simulation results also consider the timing error (assume 0.9us) impacts on the SNR performance.

Figure 1: Updated SNR of Envelope receiver (OOK-4 M=4)

The correspondign LP-WUS SNR with AWGN model at 1%MDR is presented in Table 1.

Table 1 OOK-4 M=4 SNR@1% MDR with 4bit ADC under AWGN (no repetition)

| SNR | 16bit RM coding | 20bit RM coding |
| --- | --- | --- |
| Te = 0us | -6.5 dB | -7.5 dB |
| Te = 0.9us | -5.8 dB | -6.5 dB |

Observation 1: For OOK-4, M=4, the SNR value depends on RM coding, it is -6.5 dB under 16bit RM coding without Timing error [or 20bit RM coding without 0.9us timing error].

However, whether RM coding could be adopted for OFDM-based receiver is still under discussion in RAN1, current simulation results in our paper do not consider updated simulations for OFDM. According to latest analysis, SNR around -6.5dB should be considered at least for OOK-based. RAN4 can further discuss and decide whether a single SNR value is sufficient.

Proposal 1: Select -6.5dB for OOK-based [and OFDM-based] LP-WUR as target SNR when discuss REFSENS requirements.

  - ASCS requirements

Last meeting RAN4 had agreed the guard RB for ASCS requirements [2]:

Issue 2-3-2: in-band power level of ASCS Test case

Agreements:

- For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB].

- [0] guard RB is the baseline.

Using the same simulation parameters in section 2.1, the ASCS link level simulation results are updated in Figure 2.

Table 2 SNR performance degradation for ASCS (without guard RBs inside)

The degradation for ASCS without guard RB is small, as summarized in Table 2.

Table 2 ASCS degradation without guard RB

| SNR | 16bit RM coding | 20bit RM coding |
| --- | --- | --- |
| Without NR | -6.5 dB | -7.5 dB |
| With NR | -6.4 dB | -7.4 dB |

Even without guard RB, the impact of adjacent NR signal on the WUS sensitivity is negligible. However, consider the implementation margin of receiver, 0.5dB could be a reasonable value to specify ASCS requirements, i.e., allowable de-sense due to introduction of NR RBs is 0.5dB.

Proposal 2: Allowable ASCS de-sense due to introduction of NR RBs is 0.5dB. RAN4 can define the ASCS test parameters as following in Table 3.

Table 3: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | REFSENS+0.5dB |
| Pinterferer,NR | dBm | Same PSD as LP-WUS |
| Guard RB on each side of LP-WUS signal |  | 0 |
| NOTE: The NR signal occupies the remaining RB resources with the same PSD of LP-WUS within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs. |  |  |

  - ACS requirements

For ACS, we also update the simulation results, with both 16bits RM coding and 20bits RM coding, for OOK-4 M=4. The results are shown as following:

Figure 3 ACS performance with 16bit RM coding

Figure 4 ACS performance with 20bit RM coding

The simulations are performed under a similar condition of MR, i.e., a high LP-WUS SNR condition of 14dB+LP-WUS REFSENS. The summarized results are listed in Table 4.

Table 4: Adjacent channel selectivity for OOK-based receiver (OOK-4 M=4)

| ACS | RM coding = 16bit |  | RM coding = 20bit |  |
| --- | --- | --- | --- | --- |
|  | 3rd filter | 5th filter | 3rd filter | 5th filter |
| 0 Guard RB | 24.5 | 29.3 | 25.4 | 29.9 |
| 1 Guard RB | 26.9 | 30.5 | 27.5 | 31.5 |
| 2 Guard RB | 28.8 | 31.7 | 29.5 | 32.5 |
| 3 Guard RB | 30.2 | 32.5 | 31.2 | >33 |
| 4 Guard RB | 31.5 | >33 | 32.5 | >33 |
| 5 Guard RB | 32.6 | >33 | >33 | >33 |
| 6 Guard RB | >33 | >33 | >33 | >33 |

If we consider the SNR=-6.5dB, based on the agreed NF last meeting, the LR REFSENS would be 3.5dB worse, and the upper bound of ACS would be 28dB. Then, for set 1 NF value, it would require 1RB as guard RB under 3rd order filter, and 0RB as guard RB under 5th order filter. For set 2 NF, the LR REFSENS is even 1dB better than MR, then to meet the ACS of 31.5dB it would require 5RB with 3rd order filter and 3 RB with 5th order filter.

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR

Agreement:

- For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NF values are

    - Set 1: 18dB

    - Set 2: 13.5dB

- Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB)

Proposal 3: For set 1 NF, LP-WUS ACS requirements can be specified as: ACS 28dB with Guard RB as 1. The ACS requirements and test case is defined in Table 5 and Table 6.

Table 5: Adjacent channel selectivity

| Rx Parameter | Units | LP-WUS bandwidth (11 RB) |
| --- | --- | --- |
| ACS | dB | [28] |

Table 6: Test parameters for Adjacent channel selectivity, Case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 11 RB LP-WUS within 10 MHz NR CBW |
| Power in transmission bandwidth configuration | dBm | LR REFSENS + 14 dB |
| Guard RB |  | [1] |
| Pinterferer | dBm | LR REFSENS + [35.5] dB |
| BWinterferer | MHz | 5MHz |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: The guard RB is placed at single side of LP-WUS, between LP-WUS and interferer. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. |  |  |

Proposal 4: For set 2 NF, LP-WUS ACS requirements should be further discussed based on whether increase the wanted signal level or reduce the interference.

  - REFSENS requirements

If the SNR=-6.5dB is agreed, the REFSENS requirements based on set 1 and set 2 NF can be decided. Given the number of RB for LP-WUS is a fixed value, i.e., 11 RBs with 15kHz/30kHz SCS, then single REFSENS for each band at each SCS is needed. In our understanding, rather than discussing the REFSENS value per-band for LP-WUS, it is better to use 10 MHz MR REFSENS (for many bands, 10MHz is the min CBW with 30kHz SCS) as a reference to discuss the offset value, it could be a more efficient way to finalize requirements for all the bands.

Proposal 5: Specify the LP-WUS REFSENS based on MR REFSENS+offset value approach. FFS offset value.

The proposed spec update could be:

For an OOK-based LP-WUR UE equipped with 1 Rx antenna port, 10MHz reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-1 for LP-WUR at the applicable operating bands.

Table 7.3M.2-1: OOK-based LP-WUR Single antenna port reference sensitivity allowance ΔR1R

| Operating band | LP-WUS bandwidth (RB) | ΔR1R (dB) of 10MHz REFSENS |
| --- | --- | --- |
| TDD band | 11 | [TBD] |
| FDD band | 11 | [TBD] |

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Observation 1: For OOK-4, M=4, the SNR value depends on RM coding, it is -6.5 dB under 16bit RM coding without Timing error or 20bit RM coding without 0.9us timing error.

Proposal 1: Select -6.5dB for OOK-based [and OFDM-based] LP-WUR as target SNR when discuss REFSENS requirements.

Proposal 2: Allowable ASCS de-sense due to introduction of NR RBs is 0.5dB. RAN4 can define the ASCS test parameters as following in Table 3.

Table 3: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | REFSENS+0.5dB |
| Pinterferer,NR | dBm | Same PSD as LP-WUS |
| Guard RB on each side of LP-WUS signal |  | 0 |
| NOTE: The NR signal occupies the remaining RB resources with the same PSD of LP-WUS within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs. |  |  |

Proposal 3: For set 1 NF, LP-WUS ACS requirements can be specified as: ACS 28dB with Guard RB as 1. The ACS requirements and test case is defined in Table 5 and Table 6.

Table 5: Adjacent channel selectivity

| Rx Parameter | Units | LP-WUS bandwidth (11 RB) |
| --- | --- | --- |
| ACS | dB | [28] |

Table 6: Test parameters for Adjacent channel selectivity, Case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 11 RB LP-WUS within 10 MHz NR CBW |
| Power in transmission bandwidth configuration | dBm | LR REFSENS + 14 dB |
| Guard RB |  | [1] |
| Pinterferer | dBm | LR REFSENS + [35.5] dB |
| BWinterferer | MHz | 5MHz |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: The guard RB is placed at single side of LP-WUS, between LP-WUS and interferer. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. |  |  |

Proposal 4: For set 2 NF, LP-WUS ACS requirements should be further discussed based on whether increase the wanted signal level or reduce the interference.

Proposal 5: Specify the LP-WUS REFSENS based on MR REFSENS+offset value approach. FFS offset value.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo, RAN4#113

- R4-2503028, WF on UE RF requirements for LP-WUS, vivo, RAN4#114