3GPP TSG-RAN4 Meeting #114bis	R4-2503839

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.1.1.1.1

Source:	vivo

Title:	Discussion on HPUE MSD simplification

Document for:	Discussion

- Introduction

In RAN4#114 meeting, the WF[1] captures the guidelines and assumptions to provide 1UL and 2UL HPUE MSD simplification rules. Even though the WF was not approved in the end, some agreements were still reached. In this paper, we propose the corresponding 1UL and 2UL simplification schemes based on it.

Agreement: - Eligibility criteria to HPUE MSD requirements specified using LUTs: Moderator: to be updated to accommodate 2UL - A valid 1UL HPUE configuration is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and, - A default power class 3 (1Tx) 1UL MSD test point is specified, and, - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4b, Table 7.3A.6-1a-1, or Table 7.3A.6-1b.

- Discussion

  - 1UL HPUE MSD simplification

It could be seen that lots of 1UL MSD values are first specified in TR38.850, and also captured in TS38.101-1. In this TR, a computational logic for obtaining the HPUE MSD based on the PC3 MSD is mentioned several times, and the band combinations involved are: CA_n25-n77, CA_n25-n71, CA_n66-n77, and so on. Similarly, in TR38.880, this derivation logic is also used to calculate the MSD for CA_n71A-n77A IMD. And the corresponding equations are excerpted below:

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach: MSD due to interference power I is given by: where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known, then we have: If I is increased by X dB, then MSD(X) is given by

In all, the corresponding simplification formula is quoted as follows:

MSD（x）=10*log10(1+10x10(10MSD10-1))                     (1)

MSD(x) belongs to the MSD of HPUE, while MSD belongs to the current PC3 MSD in the spec. x is the increment of interference power.

Observation1: The PC2 1TX MSD values for PC2 harmonic and harmonic mixing for some band combinations are derived directly from the PC3 MSD based on the formula MSD（x）=10*log10(1+10x10(10MSD10-1)). MSD belongs to the current PC3 MSD in the spec, x is the increment of interference power.

### UL harmonic

For 1UL harmonic, in [2] the measurement results show that the power difference between MPR 0 PC3 and MPR 0 PC2 for the fundamental, 2nd and 3rd order harmonic products, etc., is basically around 3dB. So, the answer to the question in [3] about whether a unified PCx could be applicable for all harmonic orders is yes. For 1UL harmonic MSD with 1TX architecture, the interference level increases from PC3 to PC2, which means the fundamental power rises by 3, and the nth order harmonic products rise by 3n. And considering the PA’s linearity(IPn) improving, the total interference power increment could be roughly considered as 3dB, i.e., x=3 dB.

However, for 2TX, the architecture is changed with 1PA per chain. The MSD shall be evaluated when both main path UL and diversity path UL are transmitting, so both the main path and diversity path Rx inputs are subject to the combined nth harmonic power from the main path nth harmonic and diversity path nth harmonic[4,5]. Consequently, the calculation logic is modified, resulting in a non-symmetrical power increment for the two chains, i.e., equation (1) should not be employed for the calculation of PC2 2TX or PC1.5 2TX.

Fig1. 1UL HPUE MSD's 2TX Architecture Computational Logic from [5]

Comparing the spec’s MSD of PC2 1TX to that obtained from the simplification equation(1), the fitting performance is good in most cases, with deviations not exceeding 0.4 dB. For the combination CA_n8-n79 with the worst fit, the PC3 MSD corresponding to the PC2 MSD = 6.2 recorded in TR 38.850 is actually 4.4, but for some reason the PC3 MSD in TS38.101 is corrected to 3.8, which is one of the factors that contributed to the deterioration of the fit.

Table 1. PC2 harmonic MSD with 1TX

| UL band | DL band | MSD of PC3 | Spec’s MSD of PC2, 1TX | Spec’s ∆PC2, 1TX | equation’s MSD of PC2, 1TX | equation’s ∆PC2, 1TX | equation’s ∆PC2 - Spec’s ∆PC2, 1TX | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) |  |  |  |  |  |  |
| n3 | n78 | 23.9 | 27.1 | 3.2 | 26.9 | 3 | -0.2 | UL2/DL1 direct-hit |
| n8 | n41 | 13 | 15.9 | 2.9 | 15.9 | 2.9 | 0 | UL3/DL1 direct-hit |
| n8 | n41 | 4.5 | 6.5 | 2 | 6.7 | 2.2 | 0.2 | UL3/DL1 direct-hit |
| n8 | n79 | 12 | 14.9 | 2.9 | 14.9 | 2.9 | 0 | UL5/DL1 direct-hit |
| n8 | n79 | 3.8 | 6.2 | 2.4 | 5.8 | 2 | -0.4 | UL5/DL1 direct-hit |
| n25 | n77 | 23.9 | 26.9 | 3 | 26.9 | 3 | 0 | UL2/DL1 direct-hit |
| n66 | n77 | 23.9 | 26.9 | 3 | 26.9 | 3 | 0 | UL2/DL1 direct-hit |
| n71 | n2510,11 | 6.9 | 9.4 | 2.5 | 9.4 | 2.5 | 0 | UL3/DL1 direct-hit |
| n71 | n41 | 10.8 | 13.6 | 2.8 | 13.6 | 2.8 | 0 | UL4/DL1 direct-hit |
| n71 | n41 | 3.1 | 5.1 | 2 | 4.9 | 1.8 | -0.2 | UL4/DL1 direct-hit |
| n71 | n77 | 10.4 | 13.2 | 2.8 | 13.2 | 2.8 | 0 | UL5/DL1 direct-hit |

Observation2: The simplified equation(1) shows a good fit for the UL harmonic MSD of PC2 1TX, with a deviation of no more than 0.4 dB from the MSD present in the spec.

### Harmonic mixing

For 1UL harmonic mixing, the computational process is similar but the details are different. For 1TX based on the analysis of [6], the power imbalance (or antenna isolation) value has almost no impact on the accuracy of PC2 1TX MSD, which means with the increase of power class, the increment of both chains is the same and could also be roughly considered as 3dB for PC2.

Comparing the spec’s MSD to that obtained from the simplification equation(1), the fitting performance is good in most cases for PC2 1TX, with deviations not exceeding 0.3dB. Only two band combinations have a relatively large computational deviation of about 1.1dB.

Table 2. PC2 harmonic mixing MSD with 1TX

| UL band | DL band | MSD of PC3 | Spec’s MSD of PC2, 1TX | Spec’s ∆PC2, 1TX | equation’s MSD of PC2, 1TX | equation’s ∆PC2, 1TX | equation’s ∆PC2 - Spec’s ∆PC2, 1TX | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) |  |  |  |  |  |  |
| n25 | n41 | 1.3 | 2.2 | 0.9 | 2.3 | 1 | 0.1 | UL4/DL3 |
| n25 | n713 | 26.8 | 29.5 | 2.7 | 29.8 | 3 | 0.3 | UL1/DL3 |
| n39 | n41 | 2.2 | 4.9 | 2.7 | 3.7 | 1.5 | -1.2 | UL4/DL3 |
| n41 | n186 | 26.3 | 29.3 | 3 | 29.3 | 3 | 0 | UL1/DL3 |
| n41 | n186 | 21.3 | 24.3 | 3 | 24.3 | 3 | 0 | UL1/DL3 |
| n41 | n39 | 4.3 | 6.1 | 1.8 | 6.4 | 2.1 | 0.3 | UL3/DL4 |
| n41 | n39 | 0.8 | 1.5 | 0.7 | 1.5 | 0.7 | 0 | UL3/DL4 |
| n41 | n77 | 8.9 | 11.6 | 2.7 | 11.6 | 2.7 | 0 | UL4/DL3 |
| n41 | n77 | 2.1 | 3.6 | 1.5 | 3.5 | 1.4 | -0.1 | UL4/DL3 |
| n41 | n78 | 9.3 | 12.1 | 2.8 | 12 | 2.7 | -0.1 | UL4/DL3 |
| n41 | n78 | 2.3 | 3.8 | 1.5 | 3.8 | 1.5 | 0 | UL4/DL3 |
| n77 | n2 | 6.7 | 9.2 | 2.5 | 9.2 | 2.5 | 0 | UL1/DL2 |
| n77 | n2 | 2.8 | 4.4 | 1.6 | 4.5 | 1.7 | 0 | UL1/DL2 |
| n77 | n3 | 5.7 | 8.1 | 2.4 | 8.1 | 2.4 | 0 | UL1/DL2 |
| n77 | n5 | 5.7 | 8.1 | 2.4 | 8.1 | 2.4 | 0 | UL1/DL4 |
| n77 | n5 | 0.8 | 1.5 | 0.7 | 1.5 | 0.7 | 0 | UL1/DL4 |
| n77 | n7 | 14.7 | 17.7 | 3 | 17.6 | 2.9 | -0.1 | UL2/DL3 |
| n77 | n7 | 2.2 | 3.6 | 1.4 | 3.6 | 1.4 | 0 | UL2/DL3 |
| n77 | n8 | 5.7 | 8.1 | 2.4 | 8.1 | 2.4 | 0 | UL1/DL4 |
| n77 | n8 | 0.8 | 1.5 | 0.7 | 1.5 | 0.7 | 0 | UL1/DL4 |
| n77 | n12 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n77 | n12 | 18 | 21 | 3 | 21 | 3 | 0 | UL1/DL5 |
| n77 | n13 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n77 | n13 | 27.8 | 30.8 | 3 | 30.8 | 3 | 0 | UL1/DL5 |
| n77 | n14 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n77 | n14 | 27.8 | 30.8 | 3 | 30.8 | 3 | 0 | UL1/DL5 |
| n77 | n25 | 5.6 | 7.9 | 2.3 | 8.0 | 2.4 | 0.1 | UL1/DL2 |
| n77 | n25 | 0.3 | 0.5 | 0.2 | 0.6 | 0.3 | 0.1 | UL1/DL2 |
| n77 | n28 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n77 | n28 | 11.4 | 14.1 | 2.7 | 14.2 | 2.8 | 0.1 | UL1/DL5 |
| n772 | n29 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n772 | n29 | 27.8 | 30.8 | 3 | 30.8 | 3 | 0 | UL1/DL5 |
| n77 | n30 | 10.4 | 13.2 | 2.8 | 13.2 | 2.8 | 0 | UL2/DL3 |
| n77 | n30 | 7.6 | 10.2 | 2.6 | 10.2 | 2.6 | 0 | UL2/DL3 |
| n77 | n40 | 11.7 | 14.5 | 2.8 | 14.55 | 2.8 | 0 | UL2/DL3 |
| n77 | n40 | 3.6 | 5.5 | 1.9 | 5.5 | 1.9 | 0 | UL2/DL3 |
| n77 | n41 | 11.7 | 14.5 | 2.8 | 14.55 | 2.8 | 0 | UL2/DL3 |
| n77 | n41 | 3.6 | 5.5 | 1.9 | 5.5 | 1.9 | 0 | UL2/DL3 |
| n77 | n85 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n77 | n85 | 18 | 21 | 3 | 21 | 3 | 0 | UL1/DL5 |
| n78 | n3 | 5.7 | 8.1 | 2.4 | 8.1 | 2.4 | 0 | UL1/DL2 |
| n78 | n5 | 5.7 | [8.1] | 2.4 | 8.1 | 2.4 | 0 | UL1/DL4 |
| n78 | n8 | 5.7 | 8.1 | 2.4 | 8.1 | 2.4 | 0 | UL1/DL4 |
| n78 | n28 | 31 | 34 | 3 | 34 | 3 | 0 | UL1/DL5 |
| n78 | n40 | 3.6 | 4.4 | 0.8 | 5.5 | 1.9 | 1.1 | UL2/DL3 |
| n79 | n8 | 25 | 28.0 | 3 | 28.0 | 3 | 0 | UL1/DL5 |

Observation3: The simplified equation(1) shows a good fit for the harmonic mixing MSD of PC2 1TX, with a deviation of no more than 0.3 dB from the MSD present in the spec, except for two extreme cases.

In total, the equation (1) could be used to simplify the calculation of PC2 1TX MSD harmonic and harmonic mixing by dividing the granularity in the LUT based on the existing values in the spec. And then the upper limit of the range of values of each row of PC3 MSD could be taken to obtain the delta PC2 1TX values in the following table.

Proposal 1: The simplified equation(1): MSD（x）=10*log10(1+10x10(10MSD10-1)) could be used to simplify the PC2 1TX MSD for 1UL harmonic and harmonic mixing, and the LUTs obtained accordingly are as follows:

Table3: 1UL PC2 1TX LUT for harmonic and harmonic mixing

| PC3 MSD(dB) | PC2 1TX (dB) |
| --- | --- |
| <1 | 0.8 |
| [1,2) | 1.4 |
| [2~4) | 2.1 |
| [4~8) | 2.7 |
| >=8dB | 3 |

Also, even for PC1.5 with 2TX, as clarified in [6], the corresponding MSD values are inherited from Rel-17, which appear to assume a 0 dB interference imbalance for 1Tx Main/Div. So most of the PC1.5 MSD values in the spec can also be kept in good agreement with the values obtained from the simplified equation (1), x=6 dB. Compared to the MSD values in the spec, the fitting deviation does not exceed 0.4dB, except for one extreme case.

Table 4. PC1.5 harmonic mixing MSD with 2TX

| UL band | DL band | MSD of PC3 | Spec’s MSD of PC1.5, 1TX | Spec’s ∆PC1.5, 1TX | equation’s MSD of PC1.5, 1TX | equation’s ∆PC1.5, 1TX | equation’s ∆PC1.5 - Spec’s ∆PC1.5, 1TX | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) |  |  |  |  |  |  |
| n41 | n39 | 4.3 | 10.5 | 6.2 | 8.9 | 4.5 | -1.7 | UL3/DL4 |
| n41 | n39 | 0.8 | 3.0 | 2.2 | 2.6 | 1.8 | -0.4 | UL3/DL4 |
| n41 | n77 | 8.9 | 14.5 | 5.6 | 14.5 | 5.6 | 0 | UL4/DL3 |
| n41 | n77 | 2.1 | 5.5 | 3.4 | 5.4 | 3.3 | -0.1 | UL4/DL3 |
| n41 | n78 | 9.3 | 14.9 | 5.6 | 14.9 | 5.6 | 0 | UL4/DL3 |
| n41 | n78 | 2.3 | 5.8 | 3.5 | 5.8 | 3.5 | 0 | UL4/DL3 |
| n77 | n2 | 6.7 | 11.8 | 5.1 | 11.9 | 5.2 | 0.1 | UL1/DL2 |
| n77 | n2 | 2.8 | 6.5 | 3.7 | 6.6 | 3.8 | 0.1 | UL1/DL2 |
| n77 | n3 | 5.7 | 10.7 | 5 | 10.7 | 5 | 0 | UL1/DL2 |
| n77 | n5 | 5.7 | 10.7 | 5 | 10.7 | 5 | 0 | UL1/DL4 |
| n77 | n5 | 0.8 | 2.6 | 1.8 | 2.6 | 1.8 | 0 | UL1/DL4 |
| n77 | n12 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n77 | n12 | 18 | 24 | 6 | 24 | 6 | 0 | UL1/DL5 |
| n77 | n13 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n77 | n13 | 27.8 | 33.8 | 6 | 33.8 | 6 | 0 | UL1/DL5 |
| n77 | n14 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n77 | n14 | 27.8 | 33.8 | 6 | 33.8 | 6 | 0 | UL1/DL5 |
| n77 | n25 | 5.6 | 10.6 | 5 | 10.6 | 5 | 0 | UL1/DL2 |
| n77 | n25 | 0.3 | 0.8 | 0.5 | 1.1 | 0.5 | 0 | UL1/DL2 |
| n77 | n28 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n77 | n28 | 11.4 | 17 | 5.6 | 17.2 | 5.8 | 0.2 | UL1/DL5 |
| n772 | n29 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n772 | n29 | 27.8 | 33.8 | 6 | 33.8 | 6 | 0 | UL1/DL5 |
| n77 | n30 | 10.4 | 16.1 | 5.7 | 16.1 | 5.7 | 0 | UL2/DL3 |
| n77 | n30 | 7.6 | 13 | 5.4 | 13 | 5.4 | 0 | UL2/DL3 |
| n77 | n40 | 11.7 | 17.5 | 5.8 | 17.5 | 5.8 | 0 | UL2/DL3 |
| n77 | n40 | 3.6 | 7.9 | 4.3 | 7.9 | 4.3 | 0 | UL2/DL3 |
| n77 | n41 | 11.7 | 17.5 | 5.8 | 17.5 | 5.8 | 0 | UL2/DL3 |
| n77 | n41 | 3.6 | 7.9 | 4.3 | 7.9 | 4.3 | 0 | UL2/DL3 |
| n77 | n85 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n77 | n85 | 18 | 24 | 6 | 24 | 6 | 0 | UL1/DL5 |
| n78 | n3 | 5.7 | 10.7 | 5 | 10.7 | 5 | 0 | UL1/DL2 |
| n78 | n8 | 5.7 | 10.6 | 4.9 | 10.7 | 4.9 | 0 | UL1/DL4 |
| n78 | n28 | 31 | 37 | 6 | 37 | 6 | 0 | UL1/DL5 |
| n78 | n40 | 3.6 | 7.9 | 4.3 | 7.9 | 4.3 | 0 | UL2/DL3 |
| n79 | n8 | 25 | 31 | 6 | 31 | 6 | 0 | UL1/DL5 |

Observation 4: Simplified Equation (1) shows a good fit for the UL harmonic MSD of PC1.5 2TX. Compared to the MSD values in the spec, the fitting deviation does not exceed 0.4dB, except for one extreme case.

However, since the assumption of Main/Div power imbalance introduced by PC2 2TX is the key evaluation parameter, the PC1.5 MSD inherited from Rel-17 should also be modified according to this parameter. The lower MSD of PC1.5 should not be used to tighten the MSD value of PC2 2TX.

As can be seen that most of ∆PC2 2TX MSDs for harmonic & harmonic mixing in the spec are around 7~8dB, but the platform of ∆PC1.5 MSD for harmonic mixing in the spec is around 6dB. Therefore, specifying the power imbalance based on PC2 2TX would be a better solution to ensure that the PC2 MSD values in the existing spec are not tightened, while leaving some margin for PC1.5.

Proposal 2: The power imbalance metric should not be based on the spec value of PC1.5 MSD inherited from Rel-17, but should be based on the value of PC2 2TX MSD.

### Crossband isolation

However, as for crossband isolation, considering that the sources of interference are mainly spectral regeneration and harmonic distortion caused by device nonlinearities, the level of interference is also related to the frequency offset from the main carrier. The suppression effect of different filters varies with bandwidth configuration, frequency band, and other factors. Therefore, it is not appropriate to simplify the crossband isolation MSD by using the same scheme as UL harmonic and harmonic mixing.

Proposal 3: For the HPUE MSD of crossband isolation, it should be simplified independent of harmonic and harmonic mixing.

  - 2UL HPUE MSD simplification

The IMD products’ power is mainly influenced by factors such as changes in transmit power, variations in the power class of each PA, the number of TX paths, and the IMD index of the frequency bands involved in intermodulation distortion. So, in total, from PC3 to PC2, the increase in transmitting power naturally increases the IMD product’s power. The improvement in linearity brought about by the increase in PA power class will in turn lead to a reduction in product power. It can be roughly assumed that an increase in PA power level by one level corresponds to a 3 dB increase in IIPn. For IMD products of the same order, the coefficient distribution ratio of the two frequency bands will also affect the product power, especially when the transmitting power or PA of the two UL are different.

From our paper [7] presented in #111, some analysis of the simplification of 2UL MSD is carried out and the corresponding simplification formula is quoted from TR38.880 as follows:

MSD'=10*log10(1+10x10(10MSD10-1))                     (1)

MSD’ belongs to the MSD of HPUE, while MSD belongs to current PC3 MSD in the spec. x is the increment of interference power. Assuming all the component linearity assumptions keep unchanged, the MSD of PC2/PC1.5, i.e., MSD’, could be obtained from the deformation of PC3 MSD.

The formula (1) could be transformed into the following form:

MSD'=10lg⁡[10x10(10MSD10+(10-X10-1))]

=x+10lg10MSD10+10-X10-1

=MSD+x+Δ                                       (2)

where  Δ=10lg10MSD10+10-X10-1-MSD

When MSD is higher than 10dB, 10MSD10 is also higher than 10, which is 10 times that of (10-X10-1), so the (10-X10-1) item could be omitted. And furthermore, Δis approximately equal to 0 and could be omitted either.

And the MSD’ in (2) could be simplified to

MSD'≈MSD+x

So when PC3 MSD>10dB, PC2/PC1.5 MSD = PC3 MSD + x, x is the increment of interference power for PC2/PC1.5.

The original intermodulation product power of PC3 is: a and b is index of band1 and 2 respectively.

PIMDn=aP1+bP2-n-1IIPn,  where a+b=n

After power enhancement, for 2TX PC2, the transmitting power will change from 20dBm+20dBm to 23dBm+23dBm, and the intermodulation product power of PC2 is:

P'IMDn=aP1+3+b(P2+3)-(n-1)IIPn

- When IIPn is constant, it is simplified to:

P'IMDn=PIMDn+a+b3

=PIMDn+3n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+a+b3-3(n-1)

=PIMDn+3

Similarly, for 2TX PC1.5 with 26 dBm +26 dBm, the intermodulation product power of PC1.5 is:

P'IMDn=aP1+6+b(P2+6)-(n-1)IIPn

- When IIPn is constant (This is unrealistic because two PC3 PA cannot support the cross-modulation production of PC1.5), it is simplified to:

P'IMDn=PIMDn+a+b6

=PIMDn+6n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+a+b6-3(n-1)

=PIMDn+3+3n

For PC1.5 with 23dBm+27.8dBm, the intermodulation product power of PC1.5 is:

P'IMDn=aP1+3+b(P2+7.8)-(n-1)IIPn

- When IIPn is constant, it is simplified to:

P'IMDn=PIMDn+b*4.8+3n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+b*4.8+3n-3(n-1)

=PIMDn+b*4.8+3

From (2) it could be seen that x is the increment of interference power. Therefore, from PC3 to PC2 with 23+23, ∆PC2= 3n with constant PC3 PA, ∆PC2= 3 for 2 PC2 PA. From PC3 to PC1.5 with 26+26, ∆PC1.5= 6n with constant PC3 PA(non-existent), ∆PC1.5= 3+3n for 2 PC2 PA.

Observation 5: When PC3 MSD>10dB,

- For PC2 of 23dBm+23dBm, PC2=PC3 PA+PC3 PA, 2TX MSD = PC3 MSD +3n, n is the order of IMD product.

- For PC2 of 23dBm+23dBm, PC2=PC2 PA+PC2 PA, 2TX MSD = PC3 MSD +3.

- For PC1.5 of 26dBm+26dBm, PC1.5= PC2 PA+PC2 PA, 2TX MSD = PC3 MSD +3n +3, n is the order of IMD product.

Above discussion is based on PC3 MSD>10dB, however for low PC3 MSD cases, the fitting of the data is more complicated. So in the spirit of simplifying the discussion and not adding burden to the UE, it is suggested that the above equation can be generalized to all PC2 & PC1.5 MSD simplification scenarios.

For other power combinations, the corresponding simplified formulas are unified in the following table, which is based on the various cases obtained from the WF of the 113 meeting, and is a relatively complete analysis of the various power and PA combinations of HPUE 2UL MSD.

Proposal 4: For 2UL HPUE MSD, the simplified equation table is recommended as follows:

Table 5. HPUE MSD simplification equation table

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.8 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.8¦b¦+3n | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3order |  |  | 3dB |  | +3dB +3xorder |  | 4.8¦b¦+3n | +3dB +3xorder |

From the table above, case1 is the baseline of PC3 MSD. Cases 2~6 are PC2 ∆MSD, cases 7~10 are PC1.5 ∆MSD.

Case 2 compared to case1, only the transmit power of each band is increased by 3dB, and the power category of PA is unchanged, ∆PC2=3n.

Case 3 compared to case1, the transmit power of each band is increased by 3dB, and the PA’s power class of band2 is increased from PC3 to PC2, so the overall IMD product increase is less than 3n, i.e., ∆PC2<3n. If only one band’s PA power class is increased, the overall delta MSD will be smaller than all the PA power classes remain unchanged.

Observation 6: When the transmit power of each link remains unchanged, ∆MSD of the case where all PA power classes are raised is the largest, ∆MSD of the case where all PA power classes remain unchanged is the smallest, and ∆MSD of the case where only some PA power classes are raised is an intermediate value.

Case 4 compared to case3, the main difference is that the architecture of band2 is divided into 2TX:

- When the index of band2 is 1, ignoring the effect of MRC, the increment of IMD products in the two parts can be considered equal.

- When the index of band2 is not 1, the IMD products of the two parts are different. To simplify the details of the classification, ∆PC2 of 2TX could be reused.

Proposal 5: For IMD cases of R17/18 PC2 with 2TX or 3TX (case2~4), PC2 MSD = PC3 MSD + 3* IMD order.

Case 5 compared to case 1, the transmitting power of each band is increased by 3dB, and the PA power class of each band is increased from PC3 to PC2, the overall IMD product is increased by = 3n-3 (n-1) =3, i.e., ∆PC2=3dB.

Case 6 compared to case 5, the main difference is that the architecture of band2 is divided into 2TX, ignoring the influence of MRC, it can be considered that the increment of IMD products in the two parts is equal, i.e., ∆PC2=3dB.

Proposal 6: For IMD cases of R19 PC2 with 2TX or 3TX (case5~6), PC2 MSD = PC3 MSD + 3.

Case 7 compared to case 1, the power of both bands is increased by 6dB, and the PA is changed from PC3 to PC2, so the increment is 6n-3(n-1)=3n+3, i.e., ∆PC1.5=3n+3, n is the order of IMD product.

Case 8 compared to case 7, the main difference is that the architecture of band2 is divided into 2TX, ignoring the influence of MRC, it can be considered that the increment of IMD products in the two parts is equal i.e., ∆PC1.5=3n+3, n is the order of IMD product.

Case 9: For PC1.5's 23+24.8+24.8 R18 architecture, it is necessary to consider both the index and order of the IMD product to calculate ∆MSD. Compared to case1, the power of band 1 and 2 increases by 3dB and 7.7dB respectively, and the PA in band2 is also upgraded by one power class level, which means its linearity is also optimized. So, if ignoring the MRC difference between 2TX and 3TX, for band 1, ∆MSD = 3n+b*|4.8|; for band2, ∆MSD = 3n+b*|4.8| - 3*(n-1)=|b|*4.8+3. Totally, ∆PC1.5<3n+|b|*4.8, b is index of band2. For simplicity, it could be equal to 3n+|b|*4.8.

Case10: Compared to case4, the power of band1 and 2 both increases by 3dB, and the PA in band1 and band2 is also upgraded by one power class level, so MSD increment from case 4 is 3 ignoring MRC issue, i.e.,  ∆PC1.5<3n+3, n is the order of IMD product. It could also be merged with case7&8 to leave some margin for calculation.

Proposal 7: For IMD cases of R19 PC1.5 with 2TX or 3TX (case7~8,10), PC1.5 MSD = PC3 MSD + 3 + 3* IMD order.

Proposal 8: For IMD cases of R18 PC1.5 with 3TX (case9), PC1.5 MSD = PC3 MSD + 3* IMD order+4.8*|IMD index of PC1.5 band|.

The following table is a comparison of the calculated ∆MSD and spec ∆MSD of PC2 and PC1.5. The calculated MSD uses the equation of PC2 MSD = PC3 MSD + 3* IMD order, and PC1.5 MSD = PC3 MSD + 3* IMD order+4.8*|IMD index of PC1.5 band|.

It could be seen from Table 6 that, except for very few configurations (n8-n79), the equation for PC2 is still a good fit for most band combinations while leaving some margin for realization, max (spec ∆PC2- calculated ∆PC2) < 1dB. For PC1.5, except for very few configurations (n2 - n77), the equation for PC1.5 is also a good fit for most band configurations, max (spec ∆PC1.5- calculated ∆PC1.5) <1dB.

Table 6: PC2 and PC1.5 HPUE MSD for 2DL/2UL config

| NR CA config |  | index ¦a¦+¦b¦ | PC3 MSD | PC2 MSD | PC1.5 MSD | Source of IMD | PC2 Delta MSD |  | PC1.5 Delta MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  | 20+20 | 23+ 23/2x20 | 23+ (2x24.8) |  | spec | vivo calc | spec | vivo calc |
| n1-n77 | n1-n77 |  | 29.8 | 35.8 |  | 2B_IMD2 | 6 | 6 | N/A | N/A |
| n2-n77 | n2-n77 | 1+1 | 26 | 32.1 | 35.2 | 2B_IMD2 | 6.1 | 6 | 9.2 | 10.7 |
| n3-n77 | n3-n77 |  | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n3-n78 | n3-n78 |  | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n25-n77 | n25-n77 | 1+1 | 26 | 32.1 | 37.5 | 2B_IMD2 | 6.1 | 6 | 11.5 | 10.7 |
| n25-n78 | n25-n78 |  | 26 | 32.1 |  | 2B_IMD2 | 6.1 | 6 | N/A | N/A |
| n66-n77 | n66-n77 | 1+1 | 31 | 34.33 | 40 | 2B_IMD2 | 3.33 | 6 | 9 | 10.7 |
| n70-n77 | n70-n77 | 1+1 | 31 | 37 | 41.8 | 2B_IMD2 | 6 | 6 | 10.8 | 10.7 |
| n18-n41 | n18-n41 | 2+1 | 15.6 | 24.6 |  | 2B_IMD3 | 9 | 9 | N/A | N/A |
| n3-n41C | n3-n41C |  | 15.3 | 23.3 |  | 3CC_IMD3 | 8 | 9 | N/A | N/A |
| n1-n77 | n1-n77 |  | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 12 | N/A | N/A |
| n1-n78 | n1-n78 |  | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 12 | N/A | N/A |
| n2-n77 | n2-n77 | 3+1 | 8 | 19.1 | 26.4 | 2B_IMD4 | 11.1 | 12 | 18.4 | 16.7 |
| n3-n41 | n3-n41 |  | 8.2 | 18.4 |  | 2B_IMD4 | 10.2 | 12 | N/A | N/A |
| n3-n77 | n3-n77 |  | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 12 | N/A | N/A |
| n3-n78 | n3-n78 |  | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 12 | N/A | N/A |
| n5-n77 | n5-n77 | 3+1 | 8.3 | 18.6 | 25.5 | 2B_IMD4 | 10.3 | 12 | 17.2 | 16.7 |
| n5-n78 | n5-n78 |  | 8.3 | 18.6 |  | 2B_IMD4 | 10.3 | 12 | N/A | N/A |
| n7-n77 | n7-n77 | 2+2 | 7.1 | 15.8 | 29.7 | 2B_IMD4 | 8.7 | 12 | 22.6 | 21.4 |
| n8-n78 | n8-n78 | 3+1 | 8.3 | 15.5 |  | 2B_IMD4 | 7.2 | 12 | N/A | N/A |
| n18-n77 | n18-n77 |  | 8.4 | 18.4 |  | 2B_IMD4 | 10 | 12 | N/A | N/A |
| n26-n78 | n26-n78 | 3+1 | 11.1 | 23.8 |  | 2B_IMD4 | 12.7 | 12 | N/A | N/A |
| n30-n77 | n30-n77 |  | 8 | 17.6 |  | 2B_IMD4 | 9.6 | 12 | N/A | N/A |
| n41-n71 | n41-n71 | 3+1 | 11 | 16.3 | 25.4 | 2B_IMD4 | 5.3 | 12 | 14.4 | 16.7 |
| n2-n77 | n2-n77 | 3+2 | 5 | 20 | 28 | 2B_IMD5 | 15 | 15 | 23 | 24.4 |
| n8-n79 | n8-n79 |  | 4.8 | 21.5 |  | 2B_IMD5 | 16.7 | 15 | N/A | N/A |
| n12-n77 | n12-n77 |  | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 15 | N/A | N/A |
| n13-n77 | n13-n77 |  | 5.5 | 20.5 |  | 2B_IMD5 | 15 | 15 | N/A | N/A |
| n14-n77 | n14-n77 |  | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 15 | N/A | N/A |
| n18-n77 | n18-n77 |  | 4.5 | 11.7 |  | 2B_IMD5 | 7.2 | 15 | N/A | N/A |
| n28-n77 | n28-n77 |  | 5.5 | 19.2 |  | 2B_IMD5 | 13.7 | 15 | N/A | N/A |
| n66-n77 | n66-n77 | 3+2 | 5 | 11.27 | 26 | 2B_IMD5 | 6.27 | 15 | 21 | 24.4 |
| n70-n77 | n70-n77 | 3+2 | 5 | 18.4 | 27.9 | 2B_IMD5 | 13.4 | 15 | 22.9 | 24.4 |
| n71-n78 | n71-n78 |  | 5.5 | 11.4 |  | 2B_IMD5 | 5.9 | 15 | N/A | N/A |

- Conclusion

Observation1: The PC2 1TX MSD values for PC2 harmonic and harmonic mixing for some band combinations are derived directly from the PC3 MSD based on the formula MSD（x）=10*log10(1+10x10(10MSD10-1)). MSD belongs to the current PC3 MSD in the spec, x is the increment of interference power.

Observation 2: The simplified equation(1) shows a good fit for the UL harmonic MSD of PC2 1TX, with a deviation of no more than 0.4 dB from the MSD present in the spec.

Observation 3: The simplified equation(1) shows a good fit for the harmonic mixing MSD of PC2 1TX, with a deviation of no more than 0.3 dB from the MSD present in the spec, except for two extreme cases.

Proposal 1: The simplified equation(1): MSD（x）=10*log10(1+10x10(10MSD10-1)) could be used to simplify the PC2 1TX MSD for 1UL harmonic and harmonic mixing, and the LUTs obtained accordingly are as follows:

Table: 1UL PC2 1TX LUT for harmonic and harmonic mixing

| PC3 MSD(dB) | PC2 1TX (dB) |
| --- | --- |
| <1 | 0.8 |
| [1,2) | 1.4 |
| [2~4) | 2.1 |
| [4~8) | 2.7 |
| >=8dB | 3 |

Observation 4: Simplified Equation (1) shows a good fit for the UL harmonic MSD of PC1.5 2TX. Compared to the MSD values in the spec, the fitting deviation does not exceed 0.4dB, except for one extreme case.

Proposal 2: The power imbalance metric should not be based on the spec value of PC1.5 MSD inherited from Rel-17, but should be based on the value of PC2 2TX MSD.

Proposal 3: For the HPUE MSD of crossband isolation, it should be simplified independent of harmonic and harmonic mixing.

Observation 5: When PC3 MSD>10dB,

- For PC2 of 23dBm+23dBm, PC2=PC3 PA+PC3 PA, 2TX MSD = PC3 MSD +3n, n is the order of IMD product.

- For PC2 of 23dBm+23dBm, PC2=PC2 PA+PC2 PA, 2TX MSD = PC3 MSD +3.

- For PC1.5 of 26dBm+26dBm, PC1.5= PC2 PA+PC2 PA, 2TX MSD = PC3 MSD +3n +3, n is the order of IMD product.

Proposal 4: For 2UL HPUE MSD, the simplified equation table is recommended as follows:

Table: HPUE MSD simplification equation table

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.8 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.8¦b¦+3n | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3order |  |  | 3dB |  | +3dB +3xorder |  | 4.8¦b¦+3n | +3dB +3xorder |

Observation 6: When the transmit power of each link remains unchanged, ∆MSD of the case where all PA power classes are raised is the largest, ∆MSD of the case where all PA power classes remain unchanged is the smallest, and ∆MSD of the case where only some PA power classes are raised is an intermediate value.

Proposal 5: For IMD cases of R17/18 PC2 with 2TX or 3TX (case2~4), PC2 MSD = PC3 MSD + 3* IMD order.

Proposal 6: For IMD cases of R19 PC2 with 2TX or 3TX (case5~6), PC2 MSD = PC3 MSD + 3.

Proposal 7: For IMD cases of R19 PC1.5 with 2TX or 3TX (case7~8,10), PC1.5 MSD = PC3 MSD + 3 + 3* IMD order.

Proposal 8: For IMD cases of R18 PC1.5 with 3TX (case9), PC1.5 MSD = PC3 MSD + 3* IMD order+4.8*|IMD index of PC1.5 band|.

- References

- R4-2502864, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#114.

- R4-2413068, Update on Harmonic MSD simplification, Skyworks Solutions, Inc. RAN4#112.

- R4-2500315, On Rel-19 HPUE CA DC MSD consideration, Apple, RAN4#114.

- R4-2215660, MSD analysis for CA_n3-n78 with PC2 n3 UL, Apple, RAN4#104bis-e.

- R4-2417784, Further discussion on HPUE MSD rules, Murata Manufacturing Co Ltd. RAN4#113.

- R4-2519172, 1UL HPUE NR-CA MSD Observations. RAN4#113.

- R4-2408131, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, #111.