3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503843

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Iot NTN HPUE Rx Requirements

Agenda Item:	7.10.2.3.2

Document for:	Approval

Introduction

The Rx requirements have been progressing slowly for a few meetings. Since it is only applicable to eMTC based with full duplex case for Iot-NTN, and not applicable to half-duplex cases, the scenarios have been confirmed for more than one meeting.

In RAN4#114, the WF has been reached in [2]. In this paper, some discussions are provided.

HPUE Impact for Iot-NTN RSD

General

As already agreed, for all the half duplex mode cases, no RSD is needed. Only for the case of eMTC based Iot NTN with full duplex, RSD is needed to be discussed.

Table 1: Status for requirements consideration and feasibility (eMTC based Iot NTN with full duplex)

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

For this particular case, actual the guidelines should be the same to NR-NTN, since similar reference components are expected. However, there are two main difference: Only 1Tx and 1Rx

Observation 1: For the RSD analysis of eMTC based Iot NTN with full duplex, the key differences from NR-NTN is only 1Tx and 1Rx be considered. Other conditions (reference components conditions, duplex gap) are similar.

PC2

For PC2, the current WF in [2] is as following:

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - 256:

    - 0 dB (vivo, oppo, Samsung, Ericsson)

    - 1.1 dB (Qualcomm)

  - 255:

    - 0 dB (oppo)

    - 1.2 dB (Qualcomm)

    - FFS (vivo)

  - 254:

    - 0 dB (Samsung, Ericsson)

- WF (Ad-hoc agreement)

  - 256: FFS

  - 255: FFS

  - 254: 0dB

Different from NR-NTN, for band 256, there is also proposal of non-zero value, and further discussion is needed. For band 255, there is not too much discussions yet. If more characteristics for eMTC can be identified, more reference with NR-NTN can also be considered.

In addition, no proposal has been made on possible differentiation of handheld and non-handheld yet, so this is likely to be what the requirements become.

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN, further study for band 255 and 256 is needed, and NR-NTN can be used as another reference and also considering that only 1Tx and 1Rx is applicable for eMTC.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

The current agreed WF is as following:

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for n254/n255/n256 (Ericsson,…)

  - Option 2: non-zero value (MediaTek)

  - Option 3: others

- WF (Ad-hoc agreement)

  - Keep earlier agreement [0] dB.

  - Further study needed

As discussed in NR-NTN, currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD. For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band 254/255/256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

For PC1 Iot-NTN, currently only non-handheld requirements would be defined for eMTC based full-duplex, as the duplexer for 1Tx PC1 UE is currently deemed as not that feasible. In this sense, more clear reference components is needed if non-zero value is proposed.

Proposal 3: For PC1 1Tx, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Any non-zero value proposal should be accompanied with more detailed assumptions of the reference components.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: For the RSD analysis of eMTC based Iot NTN with full duplex, the key differences from NR-NTN is only 1Tx and 1Rx be considered. Other conditions (reference components conditions, duplex gap) are similar.

PC2

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN, further study for band 255 and 256 is needed, and NR-NTN can be used as another reference and also considering that only 1Tx and 1Rx is applicable for eMTC.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

Proposal 3: For PC1 1Tx, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Any non-zero value proposal should be accompanied with more detailed assumptions of the reference components.

- References

[1] R4-2500681, Topic summary for [114][129] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114

[2] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114

[3] R4-2500765, Iot NTN HPUE Rx Requirements, vivo, RAN4#114