3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503844

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Further discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	7.22.2

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

In RAN4#112, an initial analysis and study [2] were made for the WI, and a WF [3] has been agreed. In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope.

In RAN4#112bis, the 3Tx issues and mTRP issues were discussed and document summary is in [6], a WF is agreed in [7] and an LS was sent out in [8].

In RAN4#113, further discussion was made. A discussion paper was submitted in [10]. The overall summary was in [11] and an WF was reached in [12]. In addition, an reply LS was received after the meeting in [13], in which the answers from RAN1 were included.

In RAN4#114, discussion continues. However, no major progress was made. The main difficulty is on the TxD support and whether FR2 CR is needed or not. A summary is in [15] and WF in [16].

In this paper, further discussion was provided. Since some questions remain unchanged, some of the observations and proposals are re-submission from last meeting.

Discussion

3Tx RF Impact

TxD issue

The following WF was in [16]:

<Way forward>: PC for 3Tx requirement

- For PC3 and PC2, specify requirements for FWA and handheld UE

- FFS on PC1.5 with 3Tx, will be decided in the next meeting

  - Option 1: Consider it in the next release

  - Option 2: Specify requirements in Rel-19

Current the key remaining issue is whether TxD should be allowed or not. There are strong objection on TxD capability, according to previous discussion and what is documented in [7]:

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

However, with this understanding and strictly enforced, even support of PC1.5 would be questionable.

In [17], it is proposed that if 2Tx TxD is allowed, then PC1.5 is still possible to be achieved based on the current understanding of the RF chain capability. However, it is admitted that with this allowance of 2Tx TxD, it might be complicated to do verification. E.g.

- UE has to declare which of the two antennas among the 3 RF chains.

- If multiple RF chains can be applied to 2Tx TxD, should all of them be verified.

Based on this condition, it was proposed to consider allowing 2Tx TxD to enable PC1.5 for 3Tx UE, if the verification complexity can be controlled. However, it was still challenged offline that a 2Tx TxD, even if allowed, is a fallback mode of 3Tx thus not suitable to be verified or even defined under 3Tx scope.

Proposal 1: Discuss whether 2Tx TxD can be enabled for 3Tx, and whether this is a valid scenario for 3Tx to be defined.

MPR for PC2

For PC2 MPR requirements, there is following WF:

<Way forward>: PC2 for 3Tx

- Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results

- Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

This is also related to TxD. If TxD can not be assumed in any means, then only 2*26dBm architecture can be assumed. In this case the MPR for 1Tx requirements would be reasonable.

Proposal 2: Decide the PC2 MPR requirements based on whether TxD can be supported or not.

UE types

As discussed in [17]. We tend to let PC1.5 only support FWA UE type if this power class is supported, since multiple high power UE RF chains is challenging.

Similarly, for 3Tx PC2 UE with full power mode 0 supported, since 26dBm *3 RF chains are assumed. It is also suggested to be restricted to FWA type. Although 3*26dBm is also being considered as an implementation in NR-NTN, it should be noted that that case is for even higher power class (PC1) and NTN is much more eager for higher power compared to TN.

Proposal 3: Consider to restrict the 3Tx UE type to FWA only for

- PC2 full power mode 0

- PC1.5 if defined

Asymmetric DL sTRP/UL mTRP

In last meeting, this issue was further discussed. An further updated and simplified draft CR was also listed in [16].

6.6K 	Beam correspondence for [UEs with different TCI states for UL and DL]

For a UE that supports [[future_sTRP_2TA_capability] and the IE unifiedTCI-StateType is configured as ‘separate’, and the QCL type D reference signals associated with the UL and DL TCI states are different,] the requirements of clause 6.6 apply. Side-conditions in clause 6.6 apply to each of the TCI states.

By continuous revision, now the verification may be restricted to certain scenario that a bit beyond what originally proposed. The configuration may be able to work.

However, it is still doubtful why we should purse such verification in such a late stage, and highly likely with no real RF impact. We would rather to choose postpone the whole set of tentative m-TRP related verification in next release or 6G than to trying to find difficult compromises for various small features

Proposal 4: Consider to postpone the m-TRP related verification in next release or 6G.

Conclusion

In this paper, more study on UE RF impact of MIMO phase 5 is provided:

3Tx RF Impact

Proposal 1: Discuss whether 2Tx TxD can be enabled for 3Tx, and whether this is a valid scenario for 3Tx to be defined.

Proposal 2: Decide the PC2 MPR requirements based on whether TxD can be supported or not.

Proposal 3: Consider to restrict the 3Tx UE type to FWA only for

- PC2 full power mode 0

- PC1.5 if defined

Asymmetric DL sTRP/UL mTRP

Proposal 4: Consider to postpone the m-TRP related verification in next release or 6G.

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R4-2412095, Discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112

[3] R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement, Samsung, RAN4#112

[4] RP-242394, WID revision: NR MIMO Phase 5, RAN#105

[5] R4-2413224, On asymmetric connection scenarios for NR MIMO Phase 5, Qualcomm, RAN4#112

[6] R4-2415232, Topic summary for [112bis][128] NR_MIMO_Ph5_UE, Samsung, RAN4#112bis

[7]R4-2417116, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#112bis

[8] R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, Huawei, RAN4#112bis

[9] R4-2415792, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112bis

[10] R4-2418204, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#113

[11] R4-2418153, Topic summary for [113][131] NR_MIMO_Ph5_UE, Moderator (Samsung), RAN4#113

[12] R4-2420400, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#113

[13] R4-2500005, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1#119

[14] R4-2417632, dCR on RF requirements for a UE with different TCI states for UL and DL, Qualcomm, RAN4#113

[15] R4-2500685, Topic summary for [114][133] NR_MIMO_Ph5_UE, Moderator(Samsung), RAN4#114

[16] R4-2503037, WF on UE RF requirements for MIMO enhancement phase 5, Samsung, RAN4#114

[17] R4-2500767, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#114