3GPP TSG-RAN WG4 Meeting # 114bis                                R4-2503854

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.27.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to layer3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will propose our views on RRM requirement for event triggered L1 reporting.

Discussion

According to the WID, RAN4 should discuss the impact on RRM requirements depends on the RAN1/2 specification introduced for Event triggered L1 measurement reporting. In the pervious meetings, we have reached consensus on Event-triggered L1 reporting delay requirement and accuracy requirement from high level and detail should be discussed in furture.

The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Event triggered reporting delay for L1 reporting

- Capabilities for Support of Event Triggering and Reporting Criteria

- whether and how to coordinate with R19 MIMO

- L1 filtering assumption in event evaluation

- Other potential impact of event-triggered L1 reporting

Below, we will express our views on the above issues.

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

In the last meeting, we have discussed how to define event triggered reporting delay for L1 reporting. According to the value of Treport, delay requirements are divided into two parts, one is except for periodic measurement report with the assumption of Treport is 0, and the other is for periodic measurement report with the network configuration on the reporting interval. The remain issue is which L1-RSRP measurement period on the cells corresponding should be used.

Option 1 proposes to use the maximum L1-RSRP measurement period on the cells corresponding to the event. Based on this, the event instance evaluation could be based on new results from both current and new beam. In our understanding, we should consider channel fading and it is risk to compare new results on beam1 and old results on beam2.

For option 2, it can be used for the case that only refer to serving cell or candidate cell, such as event LTM2, event LTM4 and event LTM5. But for LTM3 which involves both serving cell and candidate cell, we prefer to use the maximum L1-RSRP measurement period on the cells corresponding to the event.

Proposal 1: prefer to use option1 for Event triggered reporting delay in L1 reporting,

Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

- Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) - Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

For the first bullet, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to introduce the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133, which is similar as legacy event triggered L3 measurement reporting. As for how to define relevant capabilities to support of event triggering L1 measurement reporting, we prefer to define a separate capability for the number of L1 measurement reports triggered by supported events to avoid affecting L3 measurement reports.

Proposal 2: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 need to be updated to cover event triggered L1 measurement reporting. And RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

For the next issue, we should discuss whether and how to coordinate with R19 MIMO.

- Issue 2-7: whether and how to coordinate with R19 MIMO Candidate options: - Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC)

We understand that this is done to reduce repetitive discussions on similar issues and avoid wasting resources. However, both the current topic and MIMO are independent topics of R19, and coordinate with R19 MIMO maybe affect overall progress of R19 Mobility. In order not to affect the discussion of the topic, we support to define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: Define a separate capability for supported number of events triggered L1 measurement reporting.

- Issue 2-8: L1 filtering assumption in event evaluation Agreement: - RAN4 should start work assuming no L1 filtering. FFS whether to define requirement with L1 filtering assumed, taking into account the progress in RAN1/2.

As to the filtering assumption in event evaluation, RAN2 assumes filtering of the L1 measure results is needed for LTM event and it’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. For event triggered reporting for L3 measurement, an additional delay can be expected in event triggered measurement reporting delay when L3 filtering is used. If L1 filtering is used, there maybe an impact on report delay requirement. Based on this, RAN4 should follow up on other details of the event evaluation and report triggering. Therefore, we should wait for RAN1/RAN2’s progress before defining requirements.

Based on RAN1’s progress, no L1 specified filtering for time and spatial domain is introduced and robustness has already been able to be guaranteed by at least one of L3 filtering of L3 based event-triggered reporting and TTT of L1 based event-triggered reporting. Therefore, there is no impact on event triggered L1 measurement in mobility.

Conclusion in RAN1#120 meeting: There is no consensus in RAN1 to support specified filtering of L1-RSRP for LTM. Send an LS to RAN2 to inform that L1 filtering is up to UE implementation. Final LS in R1-2501577.

Proposal 4: Based on RAN1’s progress, do not consider L1 filter for requirements in event triggered L1 measurement in mobility.

Conclusions

In this contribution, we put forward the following proposals on R19 L1/L2 triggered mobility.

Proposal 1: prefer to use option1 for Event triggered reporting delay in L1 reporting,

Except for periodic MR:

  - The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

For periodic MR:

  - For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

Proposal 2: The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 need to be updated to cover event triggered L1 measurement reporting. And RAN4 should define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 4: Based on RAN1’s progress, do not consider L1 filter for requirements in event triggered L1 measurement in mobility.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple