3GPP TSG-RAN4 Meeting #114bis	R4-2503859

# Wuhan, Hubei, China, 07th – 11th April, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.133: test cases for NR channel BW less than 5MHz for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce test cases for NR channel BW less than 5MHz for FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test cases for NR channel BW less than 5MHz for FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements cannot be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.12.X1, A.6.5.12.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of the change1>

#### A.6.5.12.X1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

A.6.5.12.X1.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell change with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.11E.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.X and  Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X1 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X2 and handover with PSCell delay defined in clause A.6.5.12.X3.

A.6.5.12.X1.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.X1.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell1 is on radio channel 1 in FR1. Cell 2 and 3 are on radio channel 2 in FR1. General test parameters as specified in table A.6.5.12.1.2-2 with config 1 apply except those specified in table A.6.5.12.X1.2-2. Cell specific test parameters as specified in table A.6.5.12.1.2-3 with config 1 apply except those specified in table A.6.5.12.X1.2-3.

The test procedure specified in clause A.6.5.12.1.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.X1.2-1: Supported test configurations for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD, 3 MHz bandwidth with 12PRB CBW |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.12.X1.2-2: General Test Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2, Cell 3 | Neighbour cells with 12 PRB SSB bandwidth on RF channel number 2. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 2. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |

Table A.6.5.12.X1.2-3: Cell Specific Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 12 |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 12 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.2 FDD |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |
| IoNote3 | dBm/3 MHz | Config 1 | -75.45 | -66.81 | -62.46 | -75.45 | -66.81 | -62.46 |

A.6.5.12.X1.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 920+10+102 ms=1032 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+920+10+102 ms=1032 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<end of the change1>

<start of the change2>

A.6.5.12.X2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth

A.6.5.12.X2.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell addition with 12 PRB SSB bandwidth under NR-DC is within the requirements stated in clause 8.9C.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass one of addition and release delay of PSCell with 12 PRB SSB bandwidth defined in clause A.6.5.11.X and intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth in clause A.6.5.12.X1 and inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth defined in clause A.6.5.12.X2 and handover with PSCell delay defined in clause A.6.5.12.X3.

A.6.5.12.X2.2	Test Parameters

Supported test configurations are shown in table A.6.5.12.X2.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell 1 is on radio channel 1 in FR1. Cell 2 is on radio channel 2 in FR1. Cell 3 is on radio channel 3 in FR1.

General test parameters as specified in table A.6.5.12.2.2-2 with config 1 apply except those specified in table A.6.5.12.X2.2-2. Cell specific test parameters as specified in table A.6.5.12.2.2-3 with config 1 apply except those specified in table A.6.5.12.X2.2-3.

The test procedure specified in clause A.6.5.12.2.2 applies to this test. The Cell 3 is the target cell operating with 12 PRB SSB bandwidth.

Table A.6.5.12.X2.2-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD, 3 MHz bandwidth with 12PRB CBW |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.12.X2.2-2: General Test Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2. |
|  |  |  | Cell 3 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 3. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell with 12 PRB SSB bandwidth on RF channel number 3. |
|  | Neighbour Cell |  | Cell 2 | Neighbour cell with 12 PRB SSB bandwidth on RF channel number 2 |

Table A.6.5.12.X2.2-3: Cell Specific Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter | Unit | Test configuration | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Duplex mode |  | Config 1 | FDD |  |  |  |  |  |  |  |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 12 |  |  |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 12 |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.2 FDD |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.2 FDD |  |  |  |  |  |  |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  |  |  |  |  |  |  |
| IoNote3 | dBm/3 MHz | Config 1 | -62.4 | -59.4 | -62.4 | -62.4 | -62.4 | -62.4 | -62.4 | -59.4 |

A.6.5.12.X2.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 1040+10+102 ms=1152 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+1040+10+102 ms=1152 ms from the start of T4.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<end of the change2>