3GPP TSG-RAN WG4 Meeting #114bis	R4-2503862

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor was further discussed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on remaining issues.

# Discussion

The agreements achieved in last meeting are summarized as follows:

### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2. ### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue. ### Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC Agreement in RRM session (Monday): - No need to add additional processing time in the delay requirement for scenario 5. ### Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement Agreement: - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

Regarding whether and how to define a condition to trigger UE to quit multiple panel L3 measurement, there is discussion but without agreements. The discussion could be found in the summary [3]. The reason why the conditions is important is that the power consumption in FR2 with multiple panel is extremely high compared with FR1 measurement. We should try to avoid UE is force to perform multiple panel measurement forever. We can find the evaluation about UE power consumption in TR 38.840

#### 8.1.4.1 For intra-frequency measurements Table 22: UE power consumption for the RRM measurements N: Number of cells for intra-frequency measurement; Synchronous case; Asynchronous case. FR1; FR2; FR1; FR2. N=8; 150; 225; 170; 285. N=4; 120; 195; 140; 255. - All above values are slot-averaged power (Pfr1 or Pfr2 for FR1 or FR2 respectively). - Synchronous case means actual SSB transmissions from cells are time-aligned e.g., timing of SSBi from celli is aligned with timing of SSBi from cellj. - Asynchronous case means actual SSB transmissions from cells are not time-aligned e.g., timing of SSBi from celli is not aligned with timing of SSBi from cellj - The following maximum number of cells for intra-frequency measurement within a slot: Table 23: Maximun numer of cells for intra-frequency measurements FR1; FR2. Nmax = 19; Nmax = 9. For combined measurement and search: To obtain combined neighboring cell measurement and search power, add the difference between full neighboring cell search state power and SSB/CSI-RS proc. state power to the neighboring cell measurement power, as shown below: Table 24: UE power consumption of the combined neighbor cell measurements and cell search N: Number of cells for intra-frequency measurement & search; Synchronous case; Asynchronous case. FR1; FR2; FR1; FR2. N=8; 200; 320; 220; 380. N=4; 170; 290; 190; 350.

If we look at the power consumption model for intra-frequency measurement, the power consumption per slot for RRM measurement for FR2 is 1.5~1.85 times of that of FR1. Then, ratio is 6~7.4 times per second (30KHz FR1 and  120KHz FR2). Then, if we consider UE activating multiple panels for FR2 measurement, this ratio will be at least doubled since the most power consuming component is the RF part.

Observation 1: The UE power consumption of FR2 RRM measurement is several times more than that of FR1 RRM measurement. The value will be even larger (10+ times) when activating multiple panels simultaneously.

Another issue is whether to define conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”. From under understanding, it is essential to have a quit method from UE perspective. If the activation condition is only based on serving cell quality (e.g. RSRP and/or RSRQ threshold). If UE moves to the cell edge and stops for a long time, UE will activate multiple panels for fast L3 measurement for a long time. Another reason is that, the channel condition may vary from UE to UE. For instance, the threshold conditions are met due to some blocking but the UE is still in the cell centre. It does not necessarily require to UE to perform FBS all the time.

Observation 2: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

If without UE quit conditions, the above cases can not be avoided. Then most likely that the consequence will be UE will not support this feature. The reason is that how fast/how many Rx beams are used for RRM measurement is UE implementation issues and gNB does not needs to know. For instance, though the beam sweeping factor is defined as 8 in the spec, UE can off course use smaller beam sweeping factor for better performance since Rel-15. In other words, if the above case cannot be avoided, UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Observation 3: UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Proposal 1: UE autonomous quitting should be supported in Rel-19.

Based on the online/offline discussion, valuable comments are received that some cases are still unsolved. The questions are recorded as follows:

- Q1: For case#1, if UE is in the cell edge and stops, UE shall not quit fast measurement forever as UE may starting moving again at the cell edge.

- Q2: For case#2, UE will not have qualified neighbour cell measurement results, then UE is till performing fast measurement.

Based on the discussion and proposals in last meeting, we think a timer could be considered which can help solve the issues mentioned above. For instance, if the UE has performed fast measurement for at least T1, UE is allowed to fall back to normal measurement, which can solve the issue mentioned in Q2. And then, if UE quits the fast measurement for power saving, it shall reactivate fast measurement within T2, which can solve the issue mentioned in Q1.

Observation 4: A timer can solve the issues as mentioned in Q1 and Q2.

Based on the analysis above, details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Proposal 2: Details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Another issue is about the searcher assumption, and it is concluded that RAN4 will not limit the searcher number and introduce SSB processing time. As explained during searcher assumption discussion, enough flexibility shall be allowed for UE implementations. Supporting these features requires additional UE capability for both RF and baseband resources. And the capacity of RF/baseband resource varies for different band combinations. Based on the discussion during so far, many aspects are related to whether to support CA and DC/multiple FR2 bands. From our understanding, we suggest to only consider single carrier cases in FR2 and to be extend to other cases in further release. For single carrier case (i.e. only FR2-1 PCell), and UE is only configured with one target carrier for L3 measurement in the same FR2-1 bands, the most typical scenario is intra-frequency measurement, which means the target and serving carrier frequency are the same. Thus, it is proposed to only consider single carrier case (i.e. only FR2-1 PCell) and UE is only configured with one target carrier for L3 measurement in the same FR2-1 band.

Proposal 3: Only consider single carrier case in Rel-19, and other cases can be discussed in future release.

- Single carrier case (i.e. only FR2-1 PCell) and UE is only configured with one target carrier for L3 measurement in the same FR2-1 band.

- Define separate UE capability for intra-frequency and inter-frequency.

- The candidate value for beam sweeping factor are {4, 6}

# Conclusions

Observation 1: The UE power consumption of FR2 RRM measurement is several times more than that of FR1 RRM measurement. The value will be even larger (10+ times) when activating multiple panels simultaneously.

Observation 2: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

Observation 3: UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Proposal 1: UE autonomous quitting should be supported in Rel-19.

Observation 4: A timer can solve the issues as mentioned in Q1 and Q2.

Proposal 2: Details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Proposal 3: Only consider single carrier case in Rel-19, and other cases can be discussed in future release.

- Single carrier case (i.e. only FR2-1 PCell) and UE is only configured with one target carrier for L3 measurement in the same FR2-1 band.

- Define separate UE capability for intra-frequency and inter-frequency.

- The candidate value for beam sweeping factor are {4, 6}

# References

[1] R4-2502593 WF on RRM requirements for NR_RRM_Ph5_Part2

[2] R4-2502576 AdHoc minutes for [217]&[218]NR_RRM_Ph5

[2] R4-2415656 Topic summary for [112bis][209] NR_RRM_Ph5_Part1