3GPP TSG-RAN WG4 Meeting #114bis	R4-2503866

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:  7.25.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES OD-SSB

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

## Scenarios

In Oct RAN4 meeting, RAN4 send a LS to RAN1 for clarification on time/frequency/spatial domain relation between OD-SSB and AO-SSB for case#2. RAN1 discussed the question asked in the LS and provide answers as follows [2]:

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Besides the LS reply, RAN1 also achieved following agreements in Nov meeting:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

In last RAN1 meeting, RAN1 discuss the timing relation and sent a LS to RAN4 [3] as follows:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

Besides, RAN1 also achieve following agreements which can help further clarify the applicable scenario in RAN4 discussion.

Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Based on above RAN1 progress, we can have following observations for further scenario clarification.

Observation 1: For time domain, for both case#1 and case#2, UE can determine the SFN and half frame of OD-SSB burst by SFN offset and half frame index; for case#2, it is FFS time offset between always-on SSB and on-demand SSB can be used to indicate the time location of on-demand SSB.

Observation 2: For frequency domain, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB; If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB

Observation 3: For QCL relation, SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are QCLed. FFS whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Regarding the latest RAN1 agreements, RAN4 should discussed whether to define requirements for both Alt Time-C1/C2 or only one of them. For Alt Time C1, it is straightforward that RAN4 shall define requirements for this typical case. In addition, RAN4 also discuss a special case of Alt Time C1 (scenario 4 during RAN4 discussion) when the OD-SSB and AO-SSB are evenly distributed with same periodicity. During RAN4 discussion, it is agreed that the requirements are defined based on the effective measurement periodicity. Similarly, for Alt Time C1, same principle shall also apply.

Proposal 1: RAN4 to define requirements for Alt Time C1 per RAN1 LS, and the requirements is defined based on the effective periodicity of the union of AO-SSB and OD-SSB (for both deactivated SCell measurement and SCell activation and other applicable requirements if supported).

Proposal 2: RAN4 to discuss whether to define requirements for Alt Time C2.

RAN1 also agree to support the case when the OD-SSB and AO-SSB are in different frequency location. However, it is pre-mature to consider RAN4 requirements for this case as some essential conditions are not clear at the moment. For instance, what is the QCL assumption, and which SSB UE is required to measure since UE is no required to measure both.

Proposal 3: RAN4 to wait for more progress in RAN1 about the applicable conditions when AO-SSB and OD-SSB are in different frequency location, e.g. the QCL assumptions and which SSB UE is required to measure.

## OD-SSB processing time.

In last RAN4 meeting, RAN4 discussed and confirmed RAN1 working assumptions on OD-SSB processing timeline with following agreements:

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

From network point of views, what really matters is the time that UE is ready to processing OD-SSB. Otherwise, some of the SSB will be wasted, and there could be mismatch between UE and gNB. For additional processing time, we are not fully convinced on the necessary of having them. As pointed out at very beginning, in legacy SCell activation requirements, it is possible that the first SSB comes right after Tharq+3ms, and there is no additional processing time allowed. As explained by companies, one possible case is that UE may need to re-schedule the searcher in deactivated SCell measurement. Thus, one compromised solution is to only consider the additional processing time for deactivated SCell measurement.

Proposal 4: No additional UE processing/preparation for OD-SSB for SCell activation (i.e. Scenario 2A).

Proposal 5: RAN4 to discuss the value of additional UE processing/preparation for OD-SSB only for deactivated SCell L3 measurement (i.e. Scenario 2).

## Deactivated SCell L3 measurement (case 1)

For deactivated SCell, we start from case#1, and some issues are common for both case#1 and case#2. RAN4 achieved following agreements in last RAN4 meeting:

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

For measurement period in fast mode, it is agreed that UE shall perform measurement based on OD-SSB periodicity regardless of measCycleSCell and DRX cycle. The initial motivation to considering T2 is to avoid too frequency fast measurement to avoid severe power consumption issues, e.g. UE is performing deactivated SCell measurement based on each SSB.

Observation 4: The motivation of define a side conditions of T2 is to avoid too frequency fast measurement to avoid severe power consumption issues.

Thus, the very simply principles as discussed in previous meetings by several UE vendors is that UE is not required to perform “fast measurement” more frequently than the minimum interval. And the length can refer to the minimum measurement period defined in existing spec (measurement period with 160ms measCycle).

Proposal 6: UE is not required to perform “fast measurement” more frequently than the minimum interval. And the length is the minimum measurement period defined in existing spec (i.e. measurement period with 160ms measCycle).

From our understanding, with proposal 1, there is no need to define the starting point or end point of T2. Instead, it only works as the side conditions for fast measurement window. However, based on the options on the table, we see the issue is unnecessarily over complicated. For instance, there is options to bound the length of T2 with L3 reporting. From UE point of views, when UE receives the OD-SSB activation, it means UE will determine whether to perform fast measurement after checking the last L3 reporting time point. It will involve cross layer complexity and does not bring any benefits. And it is the same to gNB implementations. As it is to solve the issues about too frequency fast measurement window, for simplification, T2 should be define as the interval between the starting point of two first activated OD-SSB.

Proposal 7: For simplification, if the starting point ending point of T2 are to be defined, T2 is defined as the interval between the starting point of the first activated OD-SSB burst (e.g. the first OD-SSB burst after OD-SSB activation and the first OD-SSB burst for the next OD-SSB activation).

Based on the online discussion, there is also options on UE measurement enhancement that the cell search can be skipped. However, we should be clear that it serves completely different purpose than UE power saving. Taking FR1 as example, if T2 is defined as 5*160ms = 800ms, it means when the OD-SSB is triggered within 800ms, UE can skip cell search. However, we known that cell search timing can be maintained much longer than that.

Proposal 8: The following two cases shall be separately discussed which serve completely different purposes.

- UE is not required to perform too frequent “fast measurement”

- UE is required to perform “fast measurement” without cell search.

Another issue is whether to have lower bound for FMW, as proposed as 10ms by companies. We can understand the motivation for UE power saving. However, the situation here is not exactly the same. Firstly, legacy L3 measurement is periodically happening. For instance, the lower bound is 600/200ms for FR1 L3 intra-frequency measurement. Without the lower bound, if the SMTC is configured as 20ms, it means the measurement period is 100ms. However, back to OD-SSB scenario, UE is only required to perform fast measurement when OD-SSB is triggered. It is not realistic that gNB trigger OD-SSB every 100 ms. If a lower bound is defined, it is meaningless to configure an SSB with shorter periodicity. For instance, if the lower bound is defined as 400ms for FR1, it means there is no difference from gNB point of view to trigger a SSB with periodicity smaller than 80ms.

Observation 5: Lower bound on OD-SSB periodicity will implicitly restrict the OD-SSB periodicity configurations with may impact the network energy saving gain.

Based on above analysis, there is no need to define lower bound for fast mode measurement.

Proposal 9: No need to define lower bound for fast mode measurement.

RAN4 also discussed the multi carrier cases, which are divided into single OD-SSB carries and multiple OD-SSB carriers. The status is summarized as follows:

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers Agreement: RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. - FFS multiple OD-SSB carriers

From our understanding, if the feature is supported, it unreasonable to restrict that only one carrier can support OD-SSB and all other carriers are with legacy SSB. If network want to have energy saving gain, most likely all carries within the same bands will support OD-SSB; otherwise, the Rx chain is active the energy saving is negligible if only one of the carriers is supporting OD-SSB.

Observation 6: The energy saving gain will be severely impacted if it is restricted that only one carrier can use OD-SSB.

And if we look at pervious RAN2 agreements, it is already supported by RAN2 that NW can send OD-SSB indication for multiple SCells simultaneously.

RAN2 Agreements @ RAN2#127bis NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Thus, multiple OD-SSB carriers shall be supported with/without legacy SSB carriers, and whether to prioritize OD-SSB carrier(s) measurement shall be discussed separately with a unified an approach.

Proposal 10: RAN4 to define requirements for multiple OD-SSB carriers with/without legacy SSB carriers, and whether to prioritize OD-SSB carrier(s) measurement shall be discussed separately with a unified approach.

The follow-up question is whether to prioritize OD-SSB carriers(s) and how to calculate CSSF when there is OD-SSB carriers(s). The simplest way to just to reuse the legacy CSSF calculation that all carriers shall be scaled by CSSF regardless of OD-SSB carriers or legacy SSB carriers. Thus, for OD-SSB carriers, there is no need to further restrict whether FMW is overlapped or not, as the measurement is scaled by CSSF and UE is not required to perform fast measurement simultaneously.

Observation 7: If legacy CSSF calculation applies that all carriers shall be scaled by CSSF regardless of OD-SSB carriers or legacy SSB carriers, there is no need to further restrict whether FMW is overlapped or not.

Some further enhancement to be considered is:

- Q1: Whether to prioritize OD-SSB carriers

- Q2: How to CSSF can be reduced for OD-SSB carriers

For Q1, it is straightforward that prioritizing OD-SSB carrier means gNB only needs to transmit minimum number of OD-SSB required for measurement, which will have maximum power saving gain the shortest measurement delay for the target OD-SSB measurement. However, there is also drawbacks of always prioritizing OD-SSB measurements. Prioritizing OD-SSB measurements means the measurement on other legacy SSB carriers are delayed. For instance, if there is also legacy SSB carrier measurement for mobility purpose, delayed the measurement on such carrier may impact the mobility performance.

Observation 8: The benefits/drawback of prioritizing OD-SSB carries measurement are as follows:

- Benefits: Maximum power saving gain the shortest measurement delay for the target OD-SSB measurement

- Drawback: Delayed the measurement on other carrier may impact the mobility performance.

From our understanding, to most safe way is to let gNB to decide whether to prioritize the OD-SSB carrier measurement since gNB in charge of the usage of OD-SSB measurement and is aware of the deployments. But we also recognize it will bring extra complexity to RAN4/2.

Proposal 11: Regarding whether to prioritize OD-SSB carrier(s), RAN4 to discuss following options:

- Option 1: Always prioritize OD-SSB carrier(s) FMW

- Option 2: No prioritizations and CSSF of OD-SSB carrier(s) are counted as legacy

- Option 3: Up to gNB to decide whether to prioritize OD-SSB carrier(s) FMW

For Q2, cases are discussed in last meeting when OD-SSB FMW are overlapped. As commented in last meeting, when UE is configured to monitor multiple OD-SSB carriers, the measurement needs to be scaled as long as activated OD-SSB is overlapped even FMW is non-overlapped (Fig.2). As illustrated in Fig.1, when the time period with activated OD-SSB are non-overlapped, when calculating the CSSF for these two carriers, they can be considered as one carriers.

Fig. 1 OD-SSB are non-overlapped

Fig. 2 OD-SSB are overlapped

Proposal 12: For CSSF calculation for multiple OD-SSB carriers, for carriers whose time period with activated OD-SSB are non-overlapping, these carriers are considered as one carrier in CSSF calculation.

We give an example in Fig. 3, UE is configured with PCC and OD-SSB CC1, OD-SSB CC2, OD-SSB CC3 and legacy SSB CC4. When calculating the CSSF for CC1~4, as OD-SSB CC1 and OD-SSB2 are overlapped, and OD-SSB CC1 and OD-SSB CC3 are non-overlapped. The CSSF can be calculated as 3. One can take CC1 and CC3 are one carrier or CC2 and CC3 are one carrier since they are both non-overlapping, but the final result of CSSF calculation is the same.

Fig. 3 multiple OD-SSB carriers.

## Deactivated SCell L3 measurement (case 2)

For deactivated SCell L3 measurement in case 2, RAN4 achieved following agreements in last meeting:

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Based on the discussion in last meeting, we can understand the motivation of option 2 from UE perspective. However, from gNB perspective, gNB is not aware of the known or unknown conditions of the UE. For instance, if there is no L3 report, how does gNB know how often gNB needs to the activated OD-SSB and how many OD-SSB bursts needs to be transmitted. One promising use case of having OD-SSB in case#2 is to gNB configure a large measCycle length (e.g. 1280ms) to guarantee a baseline mobility performance. And gNB can activate OD-SSB occasionally to check the status of the SCell and to obtain fresh measurement results for SCell activation. In this way, both gNB and UE can have power saving gains.

Observation 9: If UE always follow legacy requirements based on measCycleSCell for case 2, it is meaningless for gNB to trigger OD-SSB.

Observation 10: gNB has no idea of whether known or unknown conditions are met at UE side, thus gNB does not know how often gNB needs to the activated OD-SSB and how many OD-SSB bursts needs to be transmitted

Proposal 13: For case 2, RAN4 specify same UE measurement behavior/requirements as case 1.

## Deactivated SCell L3 measurement – other issues

For neighbor cell measurement, RAN4 achieved following agreements in last meeting:

Issue 7-2: OD-SSB based Neighbour cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

The high-level principle is that 1) UE is not aware of whether neighbor cell is OD-SSB or not 2) UE just follows configured SMTC for neighbor cell measurement. For the case when serving Cell is OD-SSB SCell, the FFS point is for FR2 measurements. The reason is that since the serving Cell measurement is separately performed from neighbor cell measurement, then UE may have different Rx beam pattern (even different beams) for Serving Cell and neighbor cell. Thus, the reasonable approach is to prioritize the serving Cell measurement in FMW, and the measurement delay for neighbor cell measurement could be extended.

Proposal 14: When the serving Cell is OD-SSB serving Cell, FR2 neighbour cell measurement in the same MO could be extended during FMW.

For neighbor cell measurement, we identified another issue based on the agreements in last meeting. For some use cases, gNB may triggered OD-SSB to check the status of the SCell or for SCell activation. On the same frequency layers, there may be no other neighbour cells or neighbour cells are also OD-SSB SCell and the SSB is muted. In that case, gNB is not expected neighbour cell measurement on the frequency layers. However, when the MO is configured to the UE, UE has to perform measurement on the frequency layer all the time. Especially for case#1, gNB may only triggered OD-SSB occasionally (e.g. just before SCell activation), but UE needs to perform neighbour cell measurement all the time. In RAN4#112 meeting, RAN4 achieved the agreements that UE is not expected to perform SSB based deactivated SCell L3 measurement before OD-SSB is triggered. It is straightforward for the deactivated SCell, but UE has no choice but to perform neighbour cell measurement all the time.

@RAN4#112 R4-2413885 Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

Observation 11: Based on current framework, gNB can not configure the UE to only measure the deactivated SCell. For case#1, UE has to perform measurement on the frequency layer even the OD-SSB is rarely triggered.

Proposal 15: RAN4 to discuss whether to support that gNB configured the UE to only measure deactivated SCell (i.e. no need to measure neighbour cell in the MO) for Case#1.

Another issue is whether to introduce OD-SSB based L3 measurement reporting. UE can trigger the measurement report if there is valid measurement report and UE can stop fast measurement. The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window. By legacy case, what we can imagine is gNB to artificially configured a very low threshold to trigger the event and reconfigure it after report to avoid UL overhead and wrongly triggered event. The is very clumsy and situation will be more severe for case#2. We think we can have similar approach as introduced in Rel-18 RRM “report on SCell activation”, and the workload is manageable in RAN2 since framework can be reused.

Issue 7-3: OD-SSB based L3 reporting Wayforward: - RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

Observation 12: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 16: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

## OD-SSB based SCell activation

For OD-SSB based SCell activation, RAN4 achieved following agreements in last meeting.

Agreement: - RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion.

From our understanding, the impact of OD-SSB for SCell activation is quite common regardless of SCell activation case.

Proposal 17: RAN4 to define OD-SSB based SCell activation requirements for:

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation with the L3 reporting during activation

Another issue identified in last meeting is about interruption due to OD-SSB based deactivated SCell measurement. From our understanding, it is reasonable to allow interruption upon OD-SSB is activated or activated for RF tuning ON/OFF. Apart from the activation and deactivation, legacy interruption requirements can apply. One thing needs to be further considered is when there is activated SCell in the same band, longer interruption duration is allowed for intra-band CA and it is assumed that RSs for activated serving Cells and deactivated SCell are available in the same slot. For OD-SSB, it is not a very typical case that there is active serving cell in the same band and the deactivated SCell is OD-SSB SCell. The reason is that there is not much energy saving gain from NW point of views. In additional, as OD-SSB can be triggered very flexibly in time domain, it cannot be guaranteed that OD-SSB is overlapping with SSB of other serving Cells in the same band.

Proposal 18: For interruption due to OD-SSB based deactivated SCell measurement:

- When there is no active serving Cells in the same band, in addition to allowed interruption requirements (in terms of probability and interruption length), additional interruption upon OD-SSB is activated or activated is allowed.

- When there is active serving Cells in the same band, existing requirements apply with following updating:

  - TSMTC_duration to duration of OD-SSB burst duration

  - No restriction on “cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot”

# Conclusions

Observation 1: For time domain, for both case#1 and case#2, UE can determine the SFN and half frame of OD-SSB burst by SFN offset and half frame index; for case#2, it is FFS time offset between always-on SSB and on-demand SSB can be used to indicate the time location of on-demand SSB.

Observation 2: For frequency domain, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB; If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB

Observation 3: For QCL relation, SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are QCLed. FFS whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Observation 4: The motivation of define a side conditions of T2 is to avoid too frequency fast measurement to avoid severe power consumption issues.

Observation 5: Lower bound on OD-SSB periodicity will implicitly restrict the OD-SSB periodicity configurations with may impact the network energy saving gain.

Observation 6: The energy saving gain will be severely impacted if it is restricted that only one carrier can use OD-SSB.

Observation 7: If legacy CSSF calculation applies that all carriers shall be scaled by CSSF regardless of OD-SSB carriers or legacy SSB carriers, there is no need to further restrict whether FMW is overlapped or not.

Observation 8: The benefits/drawback of prioritizing OD-SSB carries measurement are as follows:

- Benefits: Maximum power saving gain the shortest measurement delay for the target OD-SSB measurement

- Drawback: Delayed the measurement on other carrier may impact the mobility performance.

Observation 9: If UE always follow legacy requirements based on measCycleSCell for case 2, it is meaningless for gNB to trigger OD-SSB.

Observation 10: gNB has no idea of whether known or unknown conditions are met at UE side, thus gNB does not know how often gNB needs to the activated OD-SSB and how many OD-SSB bursts needs to be transmitted

Observation 11: Based on current framework, gNB can not configure the UE to only measure the deactivated SCell. For case#1, UE has to perform measurement on the frequency layer even the OD-SSB is rarely triggered.

Observation 12: There is no existing method to trigger L3 report after OD-SSB FMW to report latest report and help gNB to deactivated OD-SSB.

Proposal 1: RAN4 to define requirements for Alt Time C1 per RAN1 LS, and the requirements is defined based on the effective periodicity of the union of AO-SSB and OD-SSB (for both deactivated SCell measurement and SCell activation and other applicable requirements if supported).

Proposal 2: RAN4 to discuss whether to define requirements for Alt Time C2.

Proposal 3: RAN4 to wait for more progress in RAN1 about the applicable conditions when AO-SSB and OD-SSB are in different frequency location, e.g. the QCL assumptions and which SSB UE is required to measure.

Proposal 4: No additional UE processing/preparation for OD-SSB for SCell activation (i.e. Scenario 2A).

Proposal 5: RAN4 to discuss the value of additional UE processing/preparation for OD-SSB only for deactivated SCell L3 measurement (i.e. Scenario 2).

Proposal 6: UE is not required to perform “fast measurement” more frequently than the minimum interval. And the length is the minimum measurement period defined in existing spec (i.e. measurement period with 160ms measCycle).

Proposal 7: For simplification, if the starting point ending point of T2 are to be defined, T2 is defined as the interval between the starting point of the first activated OD-SSB burst (e.g. the first OD-SSB burst after OD-SSB activation and the first OD-SSB burst for the next OD-SSB activation).

Proposal 8: The following two cases shall be separately discussed which serve completely different purposes.

- UE is not required to perform too frequent “fast measurement”

- UE is required to perform “fast measurement” without cell search.

Proposal 9: No need to define lower bound for fast mode measurement.

Proposal 10: RAN4 to define requirements for multiple OD-SSB carriers with/without legacy SSB carriers, and whether to prioritize OD-SSB carrier(s) measurement shall be discussed separately with a unified approach.

Proposal 11: Regarding whether to prioritize OD-SSB carrier(s), RAN4 to discuss following options:

- Option 1: Always prioritize OD-SSB carrier(s) FMW

- Option 2: No prioritizations and CSSF of OD-SSB carrier(s) are counted as legacy

- Option 3: Up to gNB to decide whether to prioritize OD-SSB carrier(s) FMW

Proposal 12: For CSSF calculation for multiple OD-SSB carriers, for carriers whose time period with activated OD-SSB are non-overlapping, these carriers are considered as one carrier in CSSF calculation.

Proposal 13: For case 2, RAN4 specify same UE measurement behavior/requirements as case 1.

Proposal 14: When the serving Cell is OD-SSB serving Cell, FR2 neighbour cell measurement in the same MO could be extended during FMW.

Proposal 15: RAN4 to discuss whether to support that gNB configured the UE to only measure deactivated SCell (i.e. no need to measure neighbour cell in the MO) for Case#1.

Proposal 16: RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation that UE triggers report after measurement based on OD-SSB FMW.

Proposal 17: RAN4 to define OD-SSB based SCell activation requirements for:

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation with the L3 reporting during activation

Proposal 18: For interruption due to OD-SSB based deactivated SCell measurement:

- When there is no active serving Cells in the same band, in addition to allowed interruption requirements (in terms of probability and interruption length), additional interruption upon OD-SSB is activated or activated is allowed.

- When there is active serving Cells in the same band, existing requirements apply with following updating:

  - TSMTC_duration to duration of OD-SSB burst duration

  - No restriction on “cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot”

# References

[1] R4-2502598 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R1-2410917 Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell

[3] R1-2501633 LS on time location of on-demand SSB for SCell