3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503867

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.25.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adaptation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discussed the RRM impacts of SSB adaption in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

In last RAN1 meeting, RAN1 further achieved following agreements and progress, which are summarized as follows:

Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

The agreements and progress in last RAN2 meetings are summarized as follows:

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Based on latest RAN1/RAN2 progress, we can observe that:

- Only SSB periodicity adaptation with legacy SSB periodicity values are supported.

- DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported.

- RAN2 preference on L3 measurement:

  - SMTC based SCell measurement with additional SMTC configurations

  - SMTC based neighbour cell measurement with legacy SMTC

Observation 1: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported.

Observation 2: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported.

Observation 3: RAN2 preference on L3 measurement:

- SMTC based SCell measurement with additional SMTC configurations

- SMTC based neighbour cell measurement with legacy SMTC

## SSB adaptation for L1 measurement

For L1 measurement, RAN4 achieved following agreements in last meeting:

Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

Based on above agreements, we can have unified/clear UE behaviour and requirements when SSB adaption happens. And the detailed spec impact can be further discussed with more progress from other working group in CR stage. Another issue we would like to discuss is the potential impacts on measurement restrictions. Taking L1-RSRP as example, when SSB-based L1-RSRP and CSI-RS are overlapped, UE is only required to measure one of them.

#### 9.5.5.1 Measurement restriction for SSB based L1-RSRP For FR1, when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, - If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If SSB and CSI-RS have different SCS, - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair. #### 9.5.5.2 Measurement restriction for CSI-RS based L1-RSRP For both FR1 and FR2, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB. For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement without restrictions. For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities: - If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions. - If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined. For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction. For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

Based on existing requirements, it is up to UE to choose either SSB or CSI-RS for measurement. However, for SSB adaption (maybe the same situation for OD-SSB), SSB are occasionally triggered by MAC-CR or DCI. Since the Cell specific CSI-RS is configured based on normal SSB pattern without collision, the newly triggered the SSB may colliding the CSI-RS and gNB may not guarantee that both SSB and CSI-RS are available to UE when they are overlapping. Thus, if UE choose CSI-RS for measurement, it may cause wrong measurement results (e.g. wrong L1-RSRP results/ wrongly triggered BFD). Thus, it should be discussed whether legacy measurement restriction can apply when adapted SSB colliding with CSI-RS.

Observation 4: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour.

Proposal 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

## SSB adaptation for L3 measurement

For SSB adaptation for L3 measurement, RAN4 send a LS to RAN2 to ask for the potential impacts on SMTC [3]. Based on the discussion in last meeting, RAN4’s common understanding is that the L3 measurement is still based on SMTC, which means UE is still expected to detect/measure Cells within SMTC. Regarding whether there is any impact on SMTC (i.e. new SMTC or SMTC adaptation as mentioned by companies) is up to RAN2 discussion. In last RAN4 meeting, it is proposed to discuss whether to follow the same measurement behaviour and requirements as defined for OD-SSB, and the status are as follows:

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

Based on the discussion in last RAN2 meeting, RAN2 achieved following agreements [4]:

RAN2 thanks RAN4 for the LS in R4-2416911 on SSB adaptation. Regarding the question of SMTC impacts due to SSB adaptation, RAN2 made the following agreements: Agreements on SSB adaptation - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

From our understanding, it means NW will configured a serving Cell specific SMTC configurations, and for neighbour cell measurement, UE will follow legacy SMTC. From RAN4 requirements perspective, there is less impact on requirements since there is already requirements for different SMTC configurations within the same frequency layer (e.g. smtc1/2). Another issue is that there could be dynamic change on SMTC, and RNA4 may needs to consider whether to define requirements at transition based on further RAN2 progress.

Proposal 2: RAN4 to confirm the RAN2 agreements on L3 measurement framework on SSB adaptation. And RAN4 to further discuss the RAN 4 spec impact based on further RAN2 progress. (e.g. whether to define requirements at transition of SMTC change)

## SSB adaptation for SCell activation

Based on latest progress on SSB adaption scenario, it is almost the same as OD-SSB which is only for SCell, and at least SSB periodicity adaption is supported. Based on that, RAN4 achieves following agreements in last meeting:

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG

One remaining issue is how to define the case that SSB adaptation command comes together with SCell activation command. Based on latest agreements in RAN1 about the triggering command, at least group common DCI is supported and whether MAC CE is supported is up to RAN2 to discuss.

Proposal 3: For SCell activation requirements for SSB adaptation, RAN4 to wait for more progress from RAN1/2 about triggering command and discuss the conditions that “SSB adaption command and SCell activation command comes together.

Another issue is also discussed in OD-SSB about the applicable requirements for SCell activation. As also commented in last meeting, the technical aspect and spec impact are almost the same regardless of normal SCell activation/direct SCell activation/PUCCH SCell activation. The change is very similar as the feature introduced in Rel-18 RRM enhancement shortMeasInterval which applies to difference SCell activation cases.

Proposal 4: Based on agreements in last RAN4 meeting, RAN4 to define SCell activation requirements for SSB adaption for single SCell activation, including:

- Normal SCell activation

- Direct SCell activation

- PUCCH SCell activation

- SCell activation with L3 reporting

The detailed spec impact can be discussed in CR stage.

FFS whether to define requirements for multiple SCells activation.

## CR work split

Considering the progress so far and the workplan, it is proposed to trigger the work split for CR submission for following meetings. For SSB adaption, the impacts on specification are identified as follows:

| Requirements | Remarks | Responsibility |
| --- | --- | --- |
| 9.5 L1-RSRP measurements for Reporting |  |  |
| 9.8 L1-SINR measurements for Reporting |  |  |
| 8.5.5 Requirements for SSB based candidate beam detection |  |  |
| 8.3 SCell activation requirements | FFS the impacted sub-clause |  |
| 9.2 NR intra-frequency measurement | FFS the impact on L3 measurement |  |
| 9.3 NR inter-frequency measurement | FFS the impact on L3 measurement |  |

Proposal 5: RAN4 to work on the CR split according to above table.

# Conclusions

Observation 1: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported.

Observation 2: DCI-based SSB adaption triggering is supported by RAN1. RAN2 will FFS whether MAC CE based SSB adaptation is supported.

Observation 3: RAN2 preference on L3 measurement:

- SMTC based SCell measurement with additional SMTC configurations

- SMTC based neighbour cell measurement with legacy SMTC

Observation 4: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour.

Proposal 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

Proposal 2: RAN4 to confirm the RAN2 agreements on L3 measurement framework on SSB adaptation. And RAN4 to further discuss RAN4 spec impact based on further RAN2 progress. (e.g. whether to define requirements at transition of SMTC change)

Proposal 3: For SCell activation requirements for SSB adaptation, RAN4 to wait for more progress from RAN1/2 about triggering command and discuss the conditions that “SSB adaption command and SCell activation command comes together.

Proposal 4: Based on agreements in last RAN4 meeting, RAN4 to define SCell activation requirements for SSB adaption for single SCell activation, including:

- Normal SCell activation

- Direct SCell activation

- PUCCH SCell activation

- SCell activation with L3 reporting

The detailed spec impact can be discussed in CR stage.

FFS whether to define requirements for multiple SCells activation.

# References

[1] R4-2502689 WF on RRM requirements for Netw_Energy_NR_enh_Part2

[2] R2-2501484 Reply LS on SSB adaptation

[3] R4-2415663 Topic summary for [112bis] [216] Netw_Energy_NR_enh_Part2

[4] Reply LS on SSB adaptation