3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503868

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.25.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 OD-SIB1

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discussed the RRM impacts of OD SIB1 in last meeting with the guidance captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

It is agreed in last meeting that RAN4 to discuss the potential RRM impacts for OD-SIB1.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

Based on the latest progress in RAN1/2 discussion, when UE decides to perform cell reselection to an NES Cell, it will first transmit WUS(PRACH) to NES Cell based on the WUS configuration obtained from Cell A, and receives SIB1 transmitted from NES Cell within a configured time window. Thus, for RRM requirements impacts, the paging interruption may need to be updated to include the potential paging interruption caused by WUS transmission. The existing paging interruption requirements are shown as follows:

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

One way to clarify the impact due to OD-SIB1 is to that: When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

Proposal 1: For RRM requirements impacts of OD-SIB1, RAN4 to update the paging interruption requirements as:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

## CR work split

Considering the progress so far and the workplan, it is proposed to trigger the work split for CR submission for following meetings. For SSB OD-SIB1, there is only one CR identified as follows:

| Requirements | Remarks | Responsibility |
| --- | --- | --- |
| 4.2.2.6 Maximum interruption in paging reception |  |  |

# Conclusions

Proposal 1: For RRM requirements impacts of OD-SIB1, RAN4 to update the paging interruption requirements as:

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. When the target Cell is an [NES OD-SIB1 Cell], TSI-NR also includes the time for UL WUS transmission and RAR reception for OD-SSB1 reception.

# References

[1] R4-2502689 WF on RRM requirements for Netw_Energy_NR_enh_Part2