3GPP TSG-RAN WG4 Meeting # 114bis		                                         R4-2503874

Wuhan, China, 07 - 11, April, 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during RAN4 #114. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

Solutions

In the last meeting, the companies agreed to introduce the solution 1 and the solution 3. For these two solutions, there are still some parts that need to be discussed. There are two points we would like to provide our views which are whether solution 1 is possible to extend to the FR1 and FFS of solution 3.

Solution 1

Whether this solution can be extended to the FR1

The following content has been extracted from the last meeting’s WF[1].

Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

Also, in the RAN4 #113 meeting, we reached an agreement to discuss whether the concluded solution can be extended to FR1. The content is captured below from the meeting’s WF[2].

Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

Solution 1 is to let the network enable this enhancement that allows the UE to measure single serving CC per band when multiple serving CCs are configured. However, this solution is based on a technical assumption that FR2 CCs experience minimal interference due to the use of directional beams for signal transmission and reception. In contrast, FR1 signal transmission and reception typically rely on omnidirectional antennas, which are more susceptible to interference than antenna arrays. As a result, even within the same band, different CCs may experience varying levels of interference in FR1. Therefore, in FR1, a single CC cannot accurately represent the measurement results for the entire band.

Proposal 1: Solution 1 is not able to be extended to the FR1.

Solution 3:

In the last meeting, the following ad hoc agreement about 3-searcher scenarios is captured in WF[1]:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not

Issue of NR-DC

The first thing that needs to be discussed is whether the NR-DC is able to be included. NR-DC have three kinds of CC that need to be measured in FR1+FR2 (FR1 PCell and FR2 PSCell) which are PCC, SCCs and PSCC. The original rule of CSSF value based on the 2-searcher solution, the FR1 PCC occupied one searcher and PSCC shared the second searcher with the FR1 and FR2 SCCs. If using Option 3 mentioned in the agreement for NR-DC the solution will be: FR1 PCC occupied one searcher, FR2 PSCC occupied one searcher and the FR1 and FR2 SCCs shared one searcher.

According to this, no implementation issue let the 3-searcher solution apply to the NR-DC scenario.

The first point that needs to be discussed is whether NR-DC can be included. In the FR1+FR2 NR-DC scenario (with an FR1 PCell and an FR2 PSCell), three types of CCs need to be measured: PCC, SCCs, and PSCC.

The original CSSF configuration rule was based on a 2-searcher solution, where the FR1 PCC occupied one searcher, and the PSCC shared the second searcher with FR1 and FR2 SCCs. If Option 3, as mentioned in the agreement, is applied to NR-DC, the solution would be as follows: the FR1 PCC occupies one searcher, the FR2 PSCC occupies another searcher, and the FR1 and FR2 SCCs share the third searcher.

Based on this, our view is that no implementation issues are preventing the application of the 3-searcher solution to the NR-DC scenario.

Proposal 2: No implementation issues are preventing the application of the 3-searcher solution to the NR-DC scenario.

Issue of NW indication for 3-searcher solution

Secondly, regarding the introduction of a network indication for a specific SCC to use the third searcher, allocating the third searcher to a single SCC may lead to two issues. On one hand, one SCC would be measured faster than another SCC that requires neighbor cell measurement, which is not logical. Additionally, there is no significant benefit in accelerating the measurement of a single SCC to such an extent.

On the other hand, if the network indicated SCC is the one requiring neighbour cell measurement, it would occupy a total of 1.5 searchers—0.5 searcher shared between FR1 and FR2 SCCs, and the third searcher dedicated to the network indicated SCC.

Overall, this network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Observation 1:  This network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Issue of capabilities

Over the last several meetings, we have provided our analysis on the relationship between the 3-searcher solution and per-FR GAP. In our view, if the UE is configured with per-FR GAP, it may trigger at least three searchers for measurement in certain situations. For example, one searcher would be used for FR1 PCC measurement, a second searcher for FR1 SCC measurement, and an additional searcher for FR2 GAP measurement.

In the last meeting, companies thought linking the 3-searcher solution to the capability of per-FR GAP was confusing and unsuitable. As a result, a new capability for the 3-searcher solution has been approved. However, configuring this new capability together with the per-FR GAP capability may exceed the UE's measurement processing capacity. For instance, if three measurements outside the GAP in FR1 and one within the GAP measurement in FR2 occur simultaneously, the UE would effectively trigger four searchers for measurement. This could impose higher performance requirements on the UE's measurement processing capacity than originally intended. In our view, these two capabilities should be restricted so that only one can be configured at once.

Proposal 3: Per-FR gap and new 3-searcher capabilities should be restricted so that only one can be configured at once.

3-searcher solution details

Last time we discussed the design of 3-searcher solution, the third searcher will utilise the same as the second searcher. The agreement from the last meeting is captured below:

Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

From our perspective, the priority for measurement acceleration should be given to FR1 SCC over FR2 SCC. The reason why is that, as discussed in Solution 1, MOs in FR2 can be measured one CC per band under NW indications. This demonstrates that FR2 intra-frequency measurements can be accelerated by using a single CC measurement result to represent the entire band but FR1 CCs cannot be sped up by this method. Therefore, one of three searchers should primarily be used to accelerate FR1 SCC measurement. To implement this design efficiently, it is preferable to allocate separate searchers for FR1 and FR2 SCCs’ measurements. The third searcher will be used to do the FR1 SCC measurement. And remaining, the FR1 PCC occupied one searcher and FR2 all SCCs shared one searcher, for example. To summarize, the 3-searcher solution can be defined as follows:

| One of 3 searchers | Measurement Objectives |  |  |
| --- | --- | --- | --- |
| Searcher 1 | FR1 PCC |  |  |
| Searcher 2 | FR1 SCC(s) | FR1 inter-frequency MO(s) with no measurement gap | inter-RAT MO(s) with no measurement gap |
| Searcher 3 | FR2 SCC where neighbour cell measurement is required and | FR2 SCC(s) where neighbour cell measurement is not required. | FR2 inter-frequency MO(s) with no measurement gap |

Note: For the searcher used to measure the FR2 MOs, the FR2 SCC that requires neighbour cell measurement shares 50% of the searcher, while the remaining 50% is shared between the FR2 SCCs that do not require neighbour cell measurement and the FR2 inter-frequency MOs with no measurement gap.

Proposal 4: Due to the design of Solution 1 and the analysis of FR1 and FR2 measurement characteristics. The 3-searcher solution can be defined as follows ():

| One of 3 searchers | Measurement Objectives |  |  |
| --- | --- | --- | --- |
| Searcher 1 | FR1 PCC |  |  |
| Searcher 2 | FR1 SCC(s) | FR1 inter-frequency MO(s) with no measurement gap | inter-RAT MO(s) with no measurement gap |
| Searcher 3 | FR2 SCC where neighbour cell measurement is required and | FR2 SCC(s) where neighbour cell measurement is not required. | FR2 inter-frequency MO(s) with no measurement gap |

Note: For the searcher used to measure the FR2 MOs, the FR2 SCC that requires neighbour cell measurement shares 50% of the searcher, while the remaining 50% is shared between the FR2 SCCs that do not require neighbour cell measurement and the FR2 inter-frequency MOs with no measurement gap.

Conclusions

Proposal 1: Solution 1 is not able to be extended to the FR1.

Proposal 2: No implementation issues are preventing the application of the 3-searcher solution to the NR-DC scenario.

Observation 1:  This network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Proposal 3: Per-FR gap and new 3-searcher capabilities should be restricted so that only one can be configured at once.

Proposal 4: Due to the design of Solution 1 and the analysis of FR1 and FR2 measurement characteristics. The 3-searcher solution can be defined as follows ():

| One of 3 searchers | Measurement Objectives |  |  |
| --- | --- | --- | --- |
| Searcher 1 | FR1 PCC |  |  |
| Searcher 2 | FR1 SCC(s) | FR1 inter-frequency MO(s) with no measurement gap | inter-RAT MO(s) with no measurement gap |
| Searcher 3 | FR2 SCC where neighbour cell measurement is required and | FR2 SCC(s) where neighbour cell measurement is not required. | FR2 inter-frequency MO(s) with no measurement gap |

Note: For the searcher used to measure the FR2 MOs, the FR2 SCC that requires neighbour cell measurement shares 50% of the searcher, while the remaining 50% is shared between the FR2 SCCs that do not require neighbour cell measurement and the FR2 inter-frequency MOs with no measurement gap.

References

- R4-2502592 WF on RRM requirements for NR_RRM_Ph5_Part1

- R4-2420093 WF on RRM requirements for NR_RRM_Ph5_Part1