3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503877

Wuhan, China, 07 - 11, April, 2025

Agenda item:	7.22.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on simulation results of CJT calibration

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], In this contribution, we further provide our simulation results of CJT calibration.

- Discussion

UE reporting enhancement for CJT calibration

Last meeting we had an additional conclusion that updated the simulation assumption.

Agreement: - RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters - Report resolution (MD = 128, MFO = 128). - Offset range (AD and AFO) - AD = CP - AFO = 0.2 ppm - Time offset between the two TRPs: 0.5 CP - Frequency offset between the two TRPs: 0.1 ppm - Note that report resolution and offset range are network configuration via RRC signalling. - Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

In this paper, we provided the simulation based on the FO: 0.1 ppm and DO: 0.5 CP. The results will be both error value and level. The error is between two TRPs and the level is error values in 128 levels of AD and AFO. However, according to the offline e-mail discussion, the MFO is better to align with the RAN1 discussion, MFO=256. The error values are chosen at the maximum absolute value between 5% and 95% of CDF.

The simulation results are shown in the following parts.

Frequency offset simulation results

For the OF CDTJ simulation, the simulation assumption is:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm |
| Offset range (AFO) | 0.2ppm |
| Report resolution (MFO) | 256 |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

| Frequency offset: 0.1 ppm SCS: 15k Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR | 1 Sample |  | 3 Samples |  | 5 Samples |  |
|  | Error (kHz) | Levels (Max 256) | Error (kHz) | Levels (Max 256) | Error (kHz) | Levels (Max 256) |
| -3 | 0.20 | 144 | 0.12 | 85 | 0.09 | 66 |
| 0 | 0.11 | 78 | 0.09 | 66 | 0.04 | 31 |
| 3 | 0.09 | 65 | 0.06 | 41 | 0.04 | 26 |
| Frequency offset: 0.1 ppm SCS: 30k Hz |  |  |  |  |  |  |
| SNR | 1 Sample |  | 3 Samples |  | 5 Samples |  |
|  | Error (kHz) | Levels (Max 256) | Error (kHz) | Levels (Max 256) | Error (kHz) | Levels (Max 256) |
| -3 | 0.41 | 296 | 0.18 | 134 | 0.16 | 118 |
| 0 | 0.28 | 207 | 0.16 | 115 | 0.09 | 68 |
| 3 | 0.18 | 135 | 0.09 | 68 | 0.07 | 55 |

Delay offset simulation results

For the OF CDTJ simulation, the simulation assumption is:

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP |
| Offset range (AD) | CP |
| Report resolution (MD) | 128 |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

| Delay offset: 0.5 CP SCS: 15k Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR | 1 Sample |  | 3 Samples |  | 5 Samples |  |
|  | Error (us) | Levels (Max 128) | Error (us) | Levels (Max 128) | Error (us) | Levels (Max 128) |
| -3 | 0.50 | 28 | 0.36 | 20 | 0.30 | 17 |
| 0 | 0.29 | 16 | 0.20 | 11 | 0.17 | 10 |
| 3 | 0.18 | 11 | 0.14 | 8 | 0.09 | 5 |
| Delay offset: 0.5 CP SCS: 30k Hz |  |  |  |  |  |  |
| SNR | 1 Sample |  | 3 Samples |  | 5 Samples |  |
|  | Error (us) | Levels (Max 128) | Error (us) | Levels (Max 128) | Error (us) | Levels (Max 128) |
| -3 | 0.28 | 16 | 0.16 | 9 | 0.13 | 8 |
| 0 | 0.16 | 9 | 0.14 | 8 | 0.10 | 6 |
| 3 | 0.11 | 7 | 0.08 | 5 | 0.06 | 4 |

References

[1] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2