3GPP TSG-RAN WG4 #114bis	R4-2503879

Wuhan, China, 07 – 11 April, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM performance requirements for Rel-19 ATG

Agenda Item:	7.12.5

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The TU schedule of #114bis shows that the performance part for Rel-19 ATG is starting. This paper presents our observations and views on RRM impacts of performance parts on ATG CA.

RRM performance requirement analysis

In this part, we provide our analysis of the performance requirements to support CA in case by case manner in the following part.

Interruption performance requirements

In the core part discussion, we have the conclusion about the interruption requirements that we need to define in Rel-19. The SRS antenna port switching is introduced so we need the corresponding performance requirement for this. The following shows the agreement on the Rel-19 ATG core part.

Issue 1-3-1-1 Whether Interruptions at NR SRS antenna port switching requirements need to be introduced Agreement: - Introduce interruption requirement at SRS antenna port switching, and legacy requirements can be reused - Only DL interruption is considered based on legacy interruption requirement.

According to this, the performance requirement of interruption at NR SRS antenna port switching needs to be defined.

Proposal 1: Performance requirement of interruption at NR SRS antenna port switching needs to be defined.

Meanwhile, the SRS carrier based switching is still discussing, this part also needs the performance requirements as well. Waiting for the core part conclusion of SRS carrier based switching.

Proposal 2: The performance requirement of interruption at NR SRS antenna port switching needs to be defined, if the core part discussion agrees to introduce this interruption requirement.

SCell activation of SSB-less

In the core part discussion, the SSB-less SCell activation requirements are agreed that can be applied to Rel-19 ATG. The following part is the agreement of the core part.

Issue 1-3-2: SCell Activation and Deactivation Delay Agreement: - In SCell activation delay requirement, the definition of measurement related component (e.g, TL1-RSRP, measure) should refer to the ATG Clause. - Inter-band SSB-less SCell activation requirements can be applied to Rel-19 ATG.

Regarding using the antenna array type UE to do the SSB-less requirement without QCL type-D reference in FR1, only obtaining the beam info by location reference is different from the general SSB-less requirements. The performance requirement may need to be discussed.

Proposal 3: Further discuss whether or not need to define the performance requirement of SCell Activation of SSB-less SCell in FR1 inter-band CA for ATG.

Scell deactivation measurement

The Rel-19 ATG mainly discusses the CA enhancement, the SCell related requirement needs the performance requirement. The SCell activation and deactivation delay need the performance requirements. The following performance requirements may be a reference for RRM to discuss:

| SCell Activation and deactivation performance requirements | Need or Not |
| --- | --- |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| Direct SCell activation at SCell addition of known SCell in FR1 |  |
| Direct SCell activation at handover with known SCell in FR1 |  |
| PUCCH SCell Activation and deactivation of known SCell in FR1 |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command |  |
| SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX |  |
| Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX |  |
| SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |

Proposal 4: The SCell activation and deactivation delay need the performance requirements. The following performance requirements table is a reference for RRM to discuss:

| SCell Activation and deactivation performance requirements | Need or Not |
| --- | --- |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| Direct SCell activation at SCell addition of known SCell in FR1 |  |
| Direct SCell activation at handover with known SCell in FR1 |  |
| PUCCH SCell Activation and deactivation of known SCell in FR1 |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command |  |
| SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX |  |
| Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX |  |
| SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |

Conclusion

Proposal 1: Performance requirement of interruption at NR SRS antenna port switching needs to be defined.

Proposal 2: The performance requirement of interruption at NR SRS antenna port switching needs to be defined, if the core part discussion agrees to introduce this interruption requirement.

Proposal 3: Further discuss whether or not need to define the performance requirement of SCell Activation of SSB-less SCell in FR1 inter-band CA for ATG.

Proposal 4: The SCell activation and deactivation delay need the performance requirements. The following performance requirements table is a reference for RRM to discuss:

| SCell Activation and deactivation performance requirements | Need or Not |
| --- | --- |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| Direct SCell activation at SCell addition of known SCell in FR1 |  |
| Direct SCell activation at handover with known SCell in FR1 |  |
| PUCCH SCell Activation and deactivation of known SCell in FR1 |  |
| SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |  |
| SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command |  |
| SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX |  |
| Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
| SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX |  |
| SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
| TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |

References

[1] R4-2502591 WF on RRM requirements for NR_ATG_enh