3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503888

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussion on impacts for measurement event prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.21.2.2

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In, In RAN4 started measurement prediction is carried out since RAN4#112bis meeting, and measurement event prediction is to be discussed in the upcoming RAN4#114bis meeting [2]. This contribution focuses on measurement event prediction for AI/ML mobility.

Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

Discussion

In RAN2, there are two methods to predict measurement event, i.e., indirect and direct measurement event prediction. The general functional framework for indirect and direct event prediction are illustrated in below figures (duplicated from TR38.744).

In indirect event prediction, measurement prediction AI/ML model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing. In direct event prediction, measurement event is predicted directly by an AI/ML model, no measurement prediction model is involved.

Figure 4.3-1: Indirect measurement event prediction [TR38.744]

Figure 4.3-2: Direct measurement event prediction [TR38.744]

Proposal 1: RAN4 to study the indirect event prediction and direct event prediction.

In RAN2 evaluation, performance metric F1 score is introduced [TR38.744]:

F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3)

In the following, we provide analyses on indirect event prediction and direct event prediction respectively.

Indirect event prediction

For indirect event prediction, the counter n1, n2 and n3 in the formula are defined as following [TR38.744]:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]

where ETD herein is the timing difference between the instance ground-truth event occurs and the instance predicted event occurs.

Figure 5.3.1-1: illustration of ETD [TR38.744]

From defining requirements perspective, there are some options.

Option 1: Define accuracy for indirect event prediction

Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that measurement event is predicted within certain ETD.

Option 2: Not define accuracy requirement for indirect event prediction

In indirect event prediction, measurement prediction model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing without further involvement of an AI/ML model. In a sense, as long as the measurement prediction accuracy is able to be satisfied, the event prediction accuracy can be guaranteed as well.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

Option 1: Define accuracy for indirect event prediction

Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD.

Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

Direct event prediction

In direct event prediction, the input of the model with direct prediction is the same as indirect prediction, and measurement event is predicted directly by an AI/ML model, and no measurement prediction AI model is involved. The output is a likelihood of an event prediction.

For direct event prediction, the counter n1, n2 and n3 in the formula are defined as following [TR38.744]:

- Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

- Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

- Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold

For direct event prediction method, a measurement event could be predicted within an occurrence window starting from current time instance i.e. t0 and future time instance t1 with a probability X% [TR38.744].

Figure 5.3.1-2: occurrence window of direct prediction method [TR38.744]

We think the accuracy of direct event prediction can be defined in below options:

Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within a time duration [t0, t1].

Proposal 3: RAN4 to discuss the following options for defining accuracy for direct event prediction:

Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: RAN4 to study the indirect event prediction and direct event prediction.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

Option 1: Define accuracy for indirect event prediction

Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD.

Option 3: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

Proposal 2: RAN4 to discuss the following options for defining accuracy for direct event prediction:

Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2502594, WF on AI/ML Mobility, Nokia

- TR38.744