3GPP TSG-RAN4 Meeting #114	R4-2503895

# Wuhan, China, 7 – 11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TC 3-2 and TC 4-1 for less than 5MHz PH2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to WF R4-2502586, the following test case need to be introduced for less than 5MHz Ph2. - TC3-2: SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used - TC4-1: SA IDLE mode CA/DC measurements on target SSB with 12 PRB bandwidth |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the following test case for less than 5MHz Ph2. - TC3-2: SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used - TC4-1: SA IDLE mode CA/DC measurements on target SSB with 12 PRB bandwidth Note: the test purpose of TC3-2 is not fully clear from WF R4-2502586. The test case is defined for event trigger reporting for SCC with activated SCell under DRX with SSB index reading and 12 PRB SSB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM performance of less than 5MHz Ph2 is not fully verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.1.X (new), A.6.6.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.6.6.1.X	SA event triggered reporting tests for SCC with activated SCell under DRX with SSB index reading and 12 PRB SSB

##### A.6.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] makes correct reporting of an event when the intra-frequency target cell on the SCC is transmitting 12 PRB SSB, when the SCell is activated. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.X.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), a FR1 activated SCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2). Supported test configurations for PCell and SCell are given in Table A.6.6.1.X.2-1 and Table A.6.6.1.X.2-1A, respectively. The general parameters are given in Table A.6.6.1.X.2-2, and cell specific test parameters are given in Table A.6.6.1.X.2-3.

In the measurement control information, a measurement object is configured for the frequency of the SCell (SCC), and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.X.2-1: Supported PCell test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with the smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.6.1.X.2-1A: Supported SCell test configurations

| ConfigurationSCell | Description |
| --- | --- |
| 1A | 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. NOTE 3: NR Cell 3 has the same SCS, BW and duplex mode as NR Cell 2. |  |

Table A.6.6.1.X.2-2: General test parameters for SA intra-frequency event triggered reporting for SCC with activated SCell under DRX with SSB index reading and 12 PRB SSB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Activated SCell |  | Cell 2 |  |
| Neighbour cell |  | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 2: Cell 2 and 3 |  |
| SSB configuration |  | SSB.13 FR1 |  |
| SMTC configuration |  | SMTC.2 |  |
| A6-Offset | dB | -4.5 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 |  |
| measCycleSCell |  | 640 ms |  |
| Time offset between serving and neighbour cells |  | 3 ms | Asynchronous cells. The timing of Cell 3 is 3 ms later than the timing of Cell 2. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.6.6.1.X.2-3: Cell specific test parameters for SA intra-frequency event triggered reporting for SCC with activated SCell under DRX with SSB index reading and 12 PRB SSB

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | N/A |  | N/A |  | N/A |  |
| PDSCH RMC configuration |  | SR.1.2 FDD |  | N/A |  | N/A |  |
| RMSI CORESET RMC configuration |  | CR.1.3 FDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | CCR.1.7 FDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | TRS.1.1 FDD |  | N/A |  | N/A |  |
| IInitial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | SSB |  | N/A |  | N/A |  |
| Note 2 | dBm/SCS | -98 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 4 | 4 | 4 | 4 | -Infinity | 4 |
| SSB_RP Note 3 | dBm/SCS kHz | -94 | -94 | -94 | -94 | -Infinity | -94 |
| Io | dBm/2.7 MHz | -69.99 | -69.99 | -69.99 | -67.65 | -69.99 | -67.65 |
| Propagation Condition |  | AWGN |  |  |  |  |  |

A.6.6.1.X.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 8160 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>

<Start of Change 2>

#### A.6.6.9.X	SA Idle mode CA/DC measurement for FR1 with 12RB SSB

##### A.6.6.9.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode, with 12 RB SSB on the target carrier. This test will partly verify the Idle mode CA/DC measurements requirements in clause 4.4.

The test environment in clause A.6.6.9.X.1 apply to this test, except that beam level reporting for early measurements is configured. Supported test configuration is given in table A.6.6.9.X.1-1. General test parameters as specified in table A.6.6.9.X.1-2 with config 1 apply to this test. Cell specific test parameters as specified in table A.6.6.9.X.1-3 with config 1 apply to this test, except those specified in table A.6.6.9.X.1-2.

Table A.6.6.9.X.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.X.1-3: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| BWP BW | MHz | Config 1 | 3: NPRB,c = 15 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 | CR.1.3 FDD |  |  | N/A |  |  |
| SSB parameters |  | Config 1 | SSB.1 FR1 |  |  | SSB.13 FR1 |  |  |
| IoNote3 | dBm/2.7 MHz | Config 1 | -67.66 | -67.66 | -67.66 | -75.45 | -72.44 | -72.44 |

##### A.6.6.9.X.2	Test Requirements

The test requirements in clause A.6.6.9.X.2 apply to this test.

<End of Change 2>