3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503896

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM requirements for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.4

Document for:	Discussion

Introduction

RRM requirements for less than 5MHz for NTN are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- Measurement (SSB index detection requirements)

- CHO

In this paper, we will provide our views on RRM requirements for less than 5MHz for NTN.

Discussion

Measurement (SSB index detection requirements)

#### Issue 1-1-1: Measurement(SSB index detection requirements) Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR.

RAN4#114 discussed the number of SSB samples for SSB index reading in L3 measurement for less than 5MHz in NTN. In R18 TN requirements for less than 5MHz, the extension is 4 samples for intra-frequency and 3 samples for inter-frequency. The numbers assumed that side condition is -6dB for intra-frequency and -4dB for inter-frequency, and the simulation result showed that different numbers are required for different SINR conditions.

It is straightforward to re-use the same number of samples for NTN, but some companies are concerned with the possible long delay due to possible use of larger SSB periodicity in NTN. During the discussion in RAN4#114, it was found that the side condition for inter-frequency is in fact -6dB instead of -4dB in current spec. and it was not clear whether this -6dB is a typo or intentionally defined condition.

We checked the discussion during R15 on accuracy requirements, and we find that this -6dB is intentionally defined. In RAN4#89, draftCR [2] was endorsed which updated the side condition for inter-frequency RSRP, RSRQ and SINR from -4B to -6dB.

Based on the status, we see the following options to define SSB index reading requirements for less than 5MHz in NTN.

- Option 1: define both intra- and inter-frequency requirements at -6dB with 4 sample extension

- Option 2: define both intra- and inter-frequency requirements at -4dB with 3 sample extension

- Option 3: define intra-frequency requirements at -6dB with 4 sample extension, and inter-frequency requirements at -4dB with 3 sample extension

We do not have strong view among the options as they are all aligned with the simulation results, and the dependency of the sample number on the SINR condition is kept and aligned with R18 requirements, which is the important principle to follow for defining the requirements.

Proposal 1: RAN4 to consider the following options for L3 measurement requirements for less than 5MHz in NTN.

- Option 1: define both intra- and inter-frequency requirements at -6dB, with 4 sample extension for SSB index reading time

- Option 2: define both intra- and inter-frequency requirements at -4dB with 3 sample extension for SSB index reading time

- Option 3: define intra-frequency requirements at -6dB with 4 sample extension for SSB index reading time, and inter-frequency requirements at -4dB with 3 sample extension for SSB index reading time

CHO

#### Issue 1-1-2: CHO Agreements: - For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to [3] Trs.

In RAN4#114 agreement, the extension in CHO requirements is depending on the extension on the extension in L3 measurement requirements. The rationale of this dependency is not clear, as in HO the extension of T∆ is same for known and unknown target cell. The extension of T∆ is also same for intra- and inter-frequency HO, which have different L3 measurement requirements.

We suggest to follow the same principle as in TN. As discussed in earlier meetings, extension of T∆ is to for SSB index validation during HO execution which is similar to SSB index reading, and it has nothing to do with whether SSB index reading for L3 measurement is extended by 3 or 4 samples.

Proposal 2: RAN4 to confirm T∆ in CHO with L3 measurement is extended to 3 Trs regardless of the extension of Tmeasure.

Conclusions

In this paper we provided our views on RRM requirements for less than 5MHz for NTN.

Proposal 1: RAN4 to consider the following options for L3 measurement requirements for less than 5MHz in NTN.

- Option 1: define both intra- and inter-frequency requirements at -6dB, with 4 sample extension for SSB index reading time

- Option 2: define both intra- and inter-frequency requirements at -4dB with 3 sample extension for SSB index reading time

- Option 3: define intra-frequency requirements at -6dB with 4 sample extension for SSB index reading time, and inter-frequency requirements at -4dB with 3 sample extension for SSB index reading time

Proposal 2: RAN4 to confirm T∆ in CHO with L3 measurement is extended to 3 Trs regardless of the extension of Tmeasure.

Reference

- R4-2502589, WF on RRM requirements for [114][214] NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)

- R4-1816408, Draft CR for 38.133 on inter-frequency measurement accuracy (section 10.1.4.1, 10.1.9.1, 10.1.14.1), CMCC