3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503898

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM test cases for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.5

Document for:	Discussion

Introduction

RRM core requirements for less than 5MHz for NTN are almost finished in RAN4#114, and the draftCR is endorsed in [1]. According to the work plan, RAN4 should start performance part of the WI.

In this paper, we will provide our views on RRM test cases for less than 5MHz for NTN.

Discussion

In R18, for less than 5MHz in TN, following test cases are defined.

- Intra-frequency cell reselection

- Intra-frequency HO

- RLM OOS with DRX for 12RB

- RLM OOS without DRX for 15RB

- RLM IS without DRX for 15RB

- RLM IS with DRX for 12RB

- BFR without DRX

- Intra-frequency L3 measurement with SSB index reading without DRX

- Inter-frequency L3 measurement without SSB index reading with DRX

It is straightforward to use the TN test list as starting point for NTN.

One difference between TN and NTN is that in NTN only 15RB BW is supported, so the highlighted two test cases for 12RB BW is not needed. Another difference is that in NTN, less than 5MHz is considered in more mobility requirements including CHO and satellite switch.

For CHO in NTN, test cases are defined for time based CHO with L3 measurement, distance based CHO with L3 measurement, and time based CHO without L3 measurement. For each category, both intra- and inter-frequency HO are considered. We suggest to select one existing CHO test case and update it for less than 5MHz. We suggest to select inter-frequency time based CHO with L3 measurement to maximize the difference compared to normal HO test case which is for intra-frequency unknown target cell.

For satellite switch in NTN, test cases are defined for hard and soft switch. We suggest to update the hard switch test case for less than 5MHz because UE supporting soft switch will also support hard switch, so the test will be applicable for more UEs.

In summary, we suggest to define the following test cases as in Proposal 1.

Proposal 1: Define the following test cases for less than 5MHz in NTN.

- IDLE-1: Intra-frequency cell reselection

- HO-1: Intra-frequency HO, unknown cell

- CHO-1: Inter-frequency CHO, time based with L3 measurement

- SatSwitch-1: Hard switch

- RLM-1: RLM OOS without DRX for 15RB

- RLM-2: RLM IS without DRX for 15RB

- BFR-1: BFR without DRX

- MR-1: Intra-frequency with SSB index reading without DRX

- MR-2: Inter-frequency without SSB index reading with DRX

Conclusions

In this paper we provided our views on RRM test cases for less than 5MHz for NTN.

Proposal 1: Define the following test cases for less than 5MHz in NTN.

- IDLE-1: Intra-frequency cell reselection

- HO-1: Intra-frequency HO, unknown cell

- CHO-1: Inter-frequency CHO, time based with L3 measurement

- SatSwitch-1: Hard switch

- RLM-1: RLM OOS without DRX for 15RB

- RLM-2: RLM IS without DRX for 15RB

- BFR-1: BFR without DRX

- MR-1: Intra-frequency with SSB index reading without DRX

- MR-2: Inter-frequency without SSB index reading with DRX

Reference

- R4-2502615, Draft big CR on RRM requirements for NTN less than 5MHz in Rel-19, Xiaomi, Samsung