3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503899

Wuhan, China, 7 – 11 April, 2025

Title: 	Simulation assumption and additional simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement. In particular, a subset of binary sequences agreed in RAN1 is selected for LP-SS simulation in [1].

In this paper, we will provide our views on the remaining issues for simulation assumption and also provide updated simulation results.

Discussion

Simulation assumption

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation Agreement: Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

In our view, the ideal RSRP should be based on the serving cell signal after fading channel. By definition, it is the received power on certain time or time/frequency resources, and the received power is impacted by the channel fading, so the ideal RSRP should be based on actual channel fading instead of statistic attenuation. Besides, we understand it has been the principle in R15 simulation to derive the normal RRM accuracy.

Similarly, for ideal RSRQ, the RSSI part should be based on the actual total received power at the specific time or time/frequency resources instead an assumed total received power.

Proposal 1: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Issue 2-1-7: On OOK interference

Our simulation results are shown in Table 1 in section 2.2. Among all the agreed cases in [1], we find that the performance for M=1, L=6 is particularly poor, especially in TDL-C channel. The specific of this case is that the binary sequence for the serving cell and interfering cell are opposite to each other.

For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1]

To confirm the impact, we also simulated two additional cases with opposite sequence which was not included in [1]:

For M=1, L=4 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0] Interference cell: [0 1 0 1] For M=2, L=8 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 0 1 0 1 1 0] Interference cell: [0 1 1 0 1 0 0 1]

The simulation results for those two additional cases are also included in Table 1. It can be seen that for all cases with opposite sequence, the performance is very poor, and the target accuracy of 3.5dB cannot be reached even with 4 samples in TDL-C.

In our view, there are 2 approaches for to address such cases with opposite sequence.

- Alt 1: define requirements for all combinations, and the requirements are based those with opposite sequence (worst case)

- Alt 2: define the requirements without considering combinations with opposite sequence, and the requirements are not applicable for those with opposite sequence

We suggest to adopt Alt 2 because the combinations with opposite sequence are in minority, but they will drag down the requirements significantly (longer delay plus degraded accuracy).

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

  - Simulation results for LP-SS

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Sample number: 1, 2, 3, 4

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

- Interference type: LP-SS, interfering cell is fully synchronous to serving cell, binary sequence for LP-SS is independent for serving cell and interfering cell

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. Our updated simulation results for LP-SS are given in Table 1.

Table 1: Simulation results for LP-SS

| {M,L} | LP-SS Binary Sequence | Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
|  |  | X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| {1,4} | 1 | 2.70 | 1.98 | 1.92 | 1.89 | 2.40 | 1.71 | 1.64 | 1.59 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | LP-SS |
| {1,4} | Opposite sequence | 3.63 | 2.60 | 2.51 | 2.38 | 3.65 | 2.72 | 2.62 | 2.51 |  |  |  |
| {1,6} | Opposite sequence | 3.42 | 2.63 | 2.49 | 2.42 | 3.47 | 2.73 | 2.58 | 2.47 |  |  |  |
| {1,8} |  | 2.93 | 2.34 | 2.29 | 2.28 | 2.66 | 2.06 | 1.98 | 1.98 |  |  |  |
| {2,8} |  | 2.58 | 1.80 | 1.72 | 1.66 | 2.55 | 1.73 | 1.59 | 1.50 |  |  |  |
| {2,8} | Opposite sequence | 4.14 | 3.15 | 2.91 | 2.83 | 4.15 | 3.28 | 2.99 | 2.92 |  |  |  |
| {2,12} |  | 2.76 | 2.09 | 2.03 | 2.00 | 2.43 | 1.78 | 1.72 | 1.67 |  |  |  |
| {2,16} |  | 2.35 | 1.68 | 1.63 | 1.60 | 2.19 | 1.46 | 1.42 | 1.35 |  |  |  |
| {4,16} |  | 3.10 | 2.34 | 2.26 | 2.24 | 2.64 | 1.98 | 1.88 | 1.84 |  |  |  |
| {4,32} |  | 2.33 | 1.69 | 1.66 | 1.63 | 2.24 | 1.50 | 1.45 | 1.41 |  |  |  |
| {1,4} |  | 3.67 | 1.94 | 1.85 | 1.69 | 3.64 | 1.87 | 1.78 | 1.67 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | TDL-C | LP-SS |
| {1,4} | Opposite sequence | 30.56 | 6.53 | 5.05 | 4.51 | 30.73 | 6.68 | 5.25 | 4.60 |  |  |  |
| {1,6} | Opposite sequence | 30.39 | 6.31 | 4.68 | 4.07 | 30.65 | 6.46 | 5.25 | 4.22 |  |  |  |
| {1,8} |  | 4.21 | 2.57 | 2.35 | 2.17 | 3.93 | 2.33 | 2.22 | 1.99 |  |  |  |
| {2,8} |  | 4.20 | 2.35 | 2.10 | 1.93 | 4.13 | 2.22 | 2.02 | 1.85 |  |  |  |
| {2,8} | Opposite sequence | 30.06 | 6.83 | 5.40 | 4.26 | 29.96 | 7.34 | 5.58 | 4.55 |  |  |  |
| {2,12} |  | 3.94 | 2.23 | 1.95 | 1.94 | 3.89 | 2.10 | 1.88 | 1.74 |  |  |  |
| {2,16} |  | 3.56 | 1.95 | 1.91 | 1.72 | 3.52 | 1.91 | 1.83 | 1.69 |  |  |  |
| {4,16} |  | 4.06 | 2.53 | 2.23 | 2.10 | 3.85 | 2.28 | 2.05 | 1.87 |  |  |  |
| {4,32} |  | 3.63 | 2.11 | 1.95 | 1.73 | 3.63 | 2.09 | 1.96 | 1.81 |  |  |  |

It can be seen that without considering the cases with opposite sequence for serving and interfering cell as discussed in section 2.1, the target accuracy of 3.5dB can be met for all {M,L} combinations with 2 samples, for both AWGN and TDL-C channels. We suggest to take number of samples for measurement X as [2].

Proposal 3: RAN4 to take samples for LP-SS measurement X as [2].

Conclusions

In this paper we provided our views on simulation assumption and also provided updated simulation results.

Proposal 1: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

Proposal 3: RAN4 to take samples for LP-SS measurement X as [2].

Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, Moderator (vivo)