3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503902

Wuhan, China, 7 – 11 April, 2025

Title: 	Additional simulation results for UE CLI measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4#112-bis, link level simulation assumption is approved in [1] for evaluating the measurement performance of L1-SRS-RSRP, e.g. with different sample number, different timing error condition and different Es/Iot. In [2], we have provided our initial simulation results for basic setup, i.e. with SRS Es/Iot = 1dB and SRS PRB number = 48. In [3], we have provided our additional simulation results for SRS Es/Iot = 1dB and 3dB and SRS PRB number = 24 and 48, under AWGN.

In this paper, we will provide additional simulation results for L1-SRS-RSRP for SRS Es/Iot = 1dB and 3dB and SRS PRB number = 24 and 48, under fading channel.

Discussion

In this section, we will provide simulation results for the following configurations in [2].

- Number of SRS PRBs: 24 and 48

- SRS Es/Iot: 1dB and 3dB

- Channel: fading channel TDL-A and TDL-C

Case 1

The measurement performance for Case 1 are shown in Table 1-2 for different fading channel conditions.

Table 1a: L1-SRS-RSRP performance for Case 1 in TDL-A @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.59 | 0.30 |
|  |  | 3 | -1.13 | -0.20 |
|  | 48 | 1 | -1.4 | 0.02 |
|  |  | 3 | -1 | -0.31 |
| 30 | 24 | 1 | -2.39 | -0.37 |
|  |  | 3 | -1.90 | -0.81 |
|  | 48 | 1 | -2.03 | -0.61 |
|  |  | 3 | -1.75 | -0.92 |
| 60 | 24 | 1 | -4.31 | -1.45 |
|  |  | 3 | -3.57 | -2.01 |
|  | 48 | 1 | -3.61 | -1.76 |
|  |  | 3 | -3.29 | -2.22 |
| 60 | 24 | 1 | -3.26 | -0.99 |
|  |  | 3 | -2.73 | -1.44 |
|  | 48 | 1 | -2.84 | -1.25 |
|  |  | 3 | -2.47 | -1.55 |
| 120 | 24 | 1 | -7.16 | -3.14 |
|  |  | 3 | -6.19 | -3.91 |
|  | 48 | 1 | -6.4 | -3.49 |
|  |  | 3 | -5.85 | -4.11 |

Table 1b: L1-SRS-RSRP performance for Case 1 in TDL-A @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.44 | 0.07 |
|  |  | 3 | -1.09 | -0.27 |
|  | 48 | 1 | -1.18 | -0.17 |
|  |  | 3 | -0.92 | -0.36 |
| -30 | 24 | 1 | -2.16 | -0.53 |
|  |  | 3 | -1.85 | -0.85 |
|  | 48 | 1 | -1.97 | -0.75 |
|  |  | 3 | -1.64 | -0.95 |
| 60 | 24 | 1 | -3.99 | -1.73 |
|  |  | 3 | -3.45 | -2.16 |
|  | 48 | 1 | -3.58 | -1.91 |
|  |  | 3 | 3.22 | -2.30 |
| 60 | 24 | 1 | -3.01 | -1.15 |
|  |  | 3 | -2.65 | -1.56 |
|  | 48 | 1 | -2.70 | -1.35 |
|  |  | 3 | -2.45 | -1.65 |
| 120 | 24 | 1 | -6.88 | -3.33 |
|  |  | 3 | -6.02 | -3.96 |
|  | 48 | 1 | -6.23 | -3.79 |
|  |  | 3 | -5.75 | -4.19 |

Table 2a: L1-SRS-RSRP performance for Case 1 in TDL-C @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.68 | 0.21 |
|  |  | 3 | -1.16 | -0.20 |
|  | 48 | 1 | -1.28 | -0.02 |
|  |  | 3 | -0.97 | -0.31 |
| 30 | 24 | 1 | -2.37 | -0.38 |
|  |  | 3 | -1.87 | -0.74 |
|  | 48 | 1 | -2 | -0.56 |
|  |  | 3 | -1.68 | -0.88 |
| 60 | 24 | 1 | -4.15 | -1.54 |
|  |  | 3 | -3.47 | -1.96 |
|  | 48 | 1 | -3.65 | -1.82 |
|  |  | 3 | -3.21 | -2.16 |
| 60 | 24 | 1 | -3.30 | -1.15 |
|  |  | 3 | -2.85 | -1.50 |
|  | 48 | 1 | -3 | -1.34 |
|  |  | 3 | -2.66 | -1.65 |
| 120 | 24 | 1 | -7.34 | -3.20 |
|  |  | 3 | -6.35 | -3.98 |
|  | 48 | 1 | -6.65 | -3.71 |
|  |  | 3 | -6.1 | -4.2 |

Table 2b: L1-SRS-RSRP performance for Case 1 in TDL-C @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.39 | 0.02 |
|  |  | 3 | -1.05 | -0.27 |
|  | 48 | 1 | -1.12 | -0.18 |
|  |  | 3 | -0.93 | -0.36 |
| 30 | 24 | 1 | -2.16 | -0.49 |
|  |  | 3 | -1.82 | -0.87 |
|  | 48 | 1 | -1.95 | -0.73 |
|  |  | 3 | -1.62 | -0.96 |
| 60 | 24 | 1 | -3.90 | -1.64 |
|  |  | 3 | -3.41 | -2.09 |
|  | 48 | 1 | -3.53 | -1.93 |
|  |  | 3 | -3.16 | -2.25 |
| 60 | 24 | 1 | -3.12 | -1.29 |
|  |  | 3 | -2.77 | -1.62 |
|  | 48 | 1 | -2.88 | -1.50 |
|  |  | 3 | -2.54 | -1.75 |
| 120 | 24 | 1 | -7.07 | -3.51 |
|  |  | 3 | -6.29 | -4.01 |
|  | 48 | 1 | -6.41 | -3.91 |
|  |  | 3 | -5.89 | -4.36 |

Case 2

The measurement performance for Case 2 are shown in Table 3-4 for different fading channel conditions.

Table 3a: L1-SRS-RSRP performance for Case 2 in TDL-A @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.12 | 0.61 |
|  |  | 3 | -0.69 | -0.20 |
|  | 48 | 1 | -0.82 | 0.35 |
|  |  | 3 | -0.56 | 0.07 |
| 30 | 24 | 1 | -1.42 | 0.38 |
|  |  | 3 | -0.95 | 0.02 |
|  | 48 | 1 | -1.12 | 0.16 |
|  |  | 3 | -0.81 | -0.16 |
| 60 | 24 | 1 | -2.05 | -0.09 |
|  |  | 3 | -1.47 | -0.44 |
|  | 48 | 1 | -1.59 | -0.3 |
|  |  | 3 | -1.29 | -0.57 |
| 60 | 24 | 1 | -1.25 | 0.43 |
|  |  | 3 | -0.82 | 0.06 |
|  | 48 | 1 | -0.9 | 0.26 |
|  |  | 3 | -0.66 | 0.01 |
| 120 | 24 | 1 | -1.61 | 0.15 |
|  |  | 3 | -1.18 | -0.18 |
|  | 48 | 1 | -1.31 | -0.03 |
|  |  | 3 | -1.02 | -0.3 |

Table 3b: L1-SRS-RSRP performance for Case 2 in TDL-A @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.91 | 0.43 |
|  |  | 3 | -0.58 | 0.12 |
|  | 48 | 1 | -0.83 | 0.21 |
|  |  | 3 | -0.49 | -0.02 |
| 30 | 24 | 1 | -1.19 | 0.17 |
|  |  | 3 | -0.85 | -0.09 |
|  | 48 | 1 | -1.00 | 0.00 |
|  |  | 3 | -0.74 | -0.18 |
| 60 | 24 | 1 | -1.87 | -0.23 |
|  |  | 3 | -1.38 | -0.52 |
|  | 48 | 1 | -1.44 | -0.43 |
|  |  | 3 | -1.25 | -0.62 |
| 60 | 24 | 1 | -1.02 | 0.31 |
|  |  | 3 | -0.77 | 0.04 |
|  | 48 | 1 | -0.81 | 0.08 |
|  |  | 3 | -0.62 | -0.10 |
| 120 | 24 | 1 | -1.41 | 0.02 |
|  |  | 3 | -1.08 | -0.29 |
|  | 48 | 1 | -1.19 | -0.19 |
|  |  | 3 | -0.96 | -0.38 |

Table 4a: L1-SRS-RSRP performance for Case 2 in TDL-C @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.05 | 0.50 |
|  |  | 3 | -0.71 | 0.19 |
|  | 48 | 1 | -0.77 | 0.36 |
|  |  | 3 | -0.53 | 0.07 |
| 30 | 24 | 1 | -1.33 | 0.26 |
|  |  | 3 | -0.99 | -0.03 |
|  | 48 | 1 | -1.05 | 0.13 |
|  |  | 3 | -0.8 | -0.1 |
| 60 | 24 | 1 | -1.88 | -0.05 |
|  |  | 3 | -1.48 | -0.40 |
|  | 48 | 1 | -1.49 | -0.29 |
|  |  | 3 | -1.26 | -0.51 |
| 60 | 24 | 1 | -1.35 | 0.33 |
|  |  | 3 | -0.92 | -0.01 |
|  | 48 | 1 | -1.04 | 0.12 |
|  |  | 3 | -0.82 | -0.13 |
| 120 | 24 | 1 | -1.84 | 0.01 |
|  |  | 3 | -1.54 | -0.31 |
|  | 48 | 1 | -1.5 | -0.19 |
|  |  | 3 | -1.35 | -0.42 |

Table 4b: L1-SRS-RSRP performance for Case 2 in TDL-C @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.88 | 0.34 |
|  |  | 3 | -0.62 | 0.10 |
|  | 48 | 1 | -0.73 | 0.22 |
|  |  | 3 | -0.49 | 0.01 |
| 30 | 24 | 1 | -1.19 | 0.16 |
|  |  | 3 | -0.87 | -0.08 |
|  | 48 | 1 | -0.92 | -0.02 |
|  |  | 3 | -0.71 | -0.19 |
| 60 | 24 | 1 | -1.67 | -0.16 |
|  |  | 3 | -1.34 | -0.46 |
|  | 48 | 1 | -1.44 | -0.37 |
|  |  | 3 | -1.19 | -0.60 |
| 60 | 24 | 1 | -1.17 | 0.17 |
|  |  | 3 | -0.90 | -0.10 |
|  | 48 | 1 | -0.98 | 0.04 |
|  |  | 3 | -0.75 | -0.18 |
| 120 | 24 | 1 | -1.63 | -0.14 |
|  |  | 3 | -1.43 | -0.42 |
|  | 48 | 1 | -1.48 | -0.28 |
|  |  | 3 | -1.28 | -0.51 |

Conclusions

In this paper we provided our additional simulation results for L1-SRS-RSRP. We suggest to take our results in this paper into account for defining the accuracy requirements for L1-SRS-RSRP.

Reference

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2419148, Initial simulation results for UE CLI measurement, Huawei, HiSilicon

- R4-2501267, Additional simulation results for UE CLI measurement, Huawei, HiSilicon