3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503907

Wuhan, China, 7th – 11th April 2025

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.18.3

Document for:	Discussion

# Introduction

In previous RAN4#114 meeting, companies discussed the interference modelling, test parameters and simulation assumptions, and manufacturer declarations and applicability rules for Rel-19 NR MMSE-IRC BS requirement. The open issues on manufacturer declarations and applicability rules are captured in our other contribution [1].

In this contribution, the open issues on interference modeling and test parameters will be furtherly discussed as per the WF [2].

# Discussion

## 2.1	Interference Modelling

Open Issue:

Number of interferers for 2Rx

  - For the number of interferers for 2Rx under HetNet, further decide between the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

  - For HomNet scenario, further decide among the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

- Option 3: Not to consider 2Rx requirements for HomNet scenario.

  - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Agreements:

  - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx.

Table 2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 20 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 11.86 | 15.60 | 3.74 |
|  |  | TDLC300-100 |  |  |  | 14.00 | 16.89 | 2.89 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 15.01 | 16.15 | 1.14 |
|  |  | TDLC300-100 |  |  |  | 11.96 | 13.45 | 1.49 |

Table 2.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 20 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 14.35 | 18.13 | 3.78 |
|  |  | TDLC300-100 |  |  |  | 16.50 | 19.43 | 2.93 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 17.01 | 18.12 | 1.11 |
|  |  | TDLC300-100 |  |  |  | 13.83 | 15.41 | 1.58 |

In the previous meeting, it was agreed to introduce MMSE-IRC requirements with 2Rx for heterogenous network, while for homogenous networks, there was no consensus among the companies. It was reasonably mentioned by some companies that the gain of MMSE-IRC over MMSE is not significant to define the requirements with 2Rx. However, we observe approximately 1 to 1.5 dB gain as show in Table 2.2-1 and 2.2-2 and from our perspective and understanding of previous WI’s (for example, Rel-17 NR coverage enhancements), this could be enough to define the BS demodulation requirements.

Observation 1: The gain of BS MMSE-IRC over MMSE is around 1 to 1.5 dB with 2Rx in homogenous networks, which can be enough to define the BS IRC requirements.

## Further the gain observed in LTE is 3.1 dB for the same case [3]. But the same threshold should not be considered to define the NR requirements, as no new system level simulations were done for NR, and LTE interference profile was used for link-level simulations. LTE-based DIP profiles might not precisely reflect the interference conditions in NR and MMSE-IRC implementation in BS receivers might be optimized differently for NR.

Observation 2: The threshold gain of 3.1 dB in LTE for 2Rx case in homogenous network should not be considered to define NR requirements.

Furthermore, to align with LTE requirements and considering observation 1 and 2, 2Rx requirements should be defined for homogenous network deployment.

Proposal 1: Introduce BS MMSE-IRC requirements with 2Rx for homogenous network scenario.

From our simulation results [4] in Table 2.2-1 and 2.2-2, we do not see any difference in the gain of MMSE-IRC over MMSE, with 1 and 2 interferers for 2Rx in both homogenous and heterogenous network deployment. But considering the system level simulation in LTE, the 2nd strongest interferer DIP/INR value is much smaller than the 1st strongest interferer. Therefore, considering only 1 interferer would be enough.

Proposal 2: Feasible to consider only 1 interferer for both homogenous and heterogenous network deployments for BS MMSE-IRC requirements with 2Rx.

## 2.2	Test parameters and simulation assumptions

Open Issue:

Propagation condition for the serving cell

- Select between the following options:

  - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet

  - Option 2: TDLC300-100 for all cases

  - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2)

- Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

In the last meeting, companies discussed different channel models for homogenous and heterogeneous network deployment. Due to different opinions, option 2 and 3 were added along with option 1 for further discussion.

Heterogenous networks should prioritize lower doppler and delay spread configurations due to smaller cell radius and reduced relative mobility of users, while homogenous network should consider larger doppler and delay spread configurations to account for their wider cell coverage and potentially higher relative user mobility. Therefore, for homogenous network, TDLB100-400 and TDLC300-100 seems viable options, while for heterogenous network deployments TDLA30-10 is more promising.

Observation 3: Low doppler and delay spreads should be prioritized for heterogenous networks, while larger doppler and delay spread configurations should be considered for homogenous network scenario.

Table 2.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 11.52 | 15.26 | 3.74 |
|  |  | TDLC300-100 |  |  |  | 13.68 | 16.71 | 3.03 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 14.36 | 15.56 | 1.20 |
|  |  | TDLC300-100 |  |  |  | 11.75 | 13.35 | 1.60 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 7.55 | 13.65 | 6.1 |
|  |  | TDLC300-100 |  |  |  | 9.69 | 15.08 | 5.39 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 9.08 | 12.15 | 3.07 |
|  |  | TDLC300-100 |  |  |  | 8.18 | 11.31 | 3.13 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 3.55 | 10.13 | 6.58 |
|  |  | TDLC300-100 |  |  |  | 5.23 | 11.18 | 5.95 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 4.75 | 8.19 | 3.44 |
|  |  | TDLC300-100 |  |  |  | 4.14 | 7.51 | 3.37 |

Table 2.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 11.65 | 15.29 | 3.64 |
|  |  | TDLC300-100 |  |  |  | 14.04 | 16.99 | 2.95 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 12.07 | 13.74 | 1.67 |
|  |  | TDLC300-100 |  |  |  | 12.09 | 13.61 | 1.52 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.13 | 13.94 | 5.81 |
|  |  | TDLC300-100 |  |  |  | 9.94 | 15.27 | 5.33 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 8.45 | 11.46 | 3.01 |
|  |  | TDLC300-100 |  |  |  | 8.29 | 11.47 | 3.18 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 4.09 | 10.29 | 6.20 |
|  |  | TDLC300-100 |  |  |  | 5.78 | 11.42 | 5.64 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 4.51 | 7.97 | 3.46 |
|  |  | TDLC300-100 |  |  |  | 4.34 | 7.70 | 3.36 |

Based on various options in the open issues, we simulated TDLB100-400 and TDLC300-100 for homogenous network deployments, while TDLA30-10 and TDLC300-100 for heterogenous network scenario. From our simulation results [4] in Table 2.2-3 and 2.2-4, we observe slightly higher gain (~ 0.5 to 1 dB) of MMSE-IRC over MMSE for heterogenous networks with TDLA30-10 compared to TDLC300-100 for considered Rx configurations (2, 4 and 8 Rx). For homogenous network deployments, we observe almost similar gains (less than 0.5 dB) with both TDLB100-400 and TDLC300-100 channel model for various Rx configurations.

Observation 4: The gain of MMSE-IRC over MMSE is approximately 0.5 to 1dB higher under TDLA30-10 than TDLC300-100 channel model for heterogenous network deployment. For homogenous network, a similar gain is seen with TDLB100-400 and TDLC300-100 channel models.

Following the LTE BS IRC requirements outlined in Table 8.2.6.1-4 in TS 36.104, it would better to consider different channel model for the serving and interfering cell. Therefore, TDLB100-400 seems a suitable choice for homogenous network deployment.

Proposal 3: Use TDLA30-10 for heterogenous and TDLB100-400 for homogenous deployment scenarios for serving cell.

Table 8.2.6.1-4: Enhanced performance requirement type A for PUSCH, 10 MHz Channel Bandwidth

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-4 | 70% | -5.4 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-4 | 70% | -2.7 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-4 | 70% | -4.2 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-4 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-6 | 70% | -4.5 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-6 | 70% | -0.2 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal, interferer 1 and interferer 2. Note 2: The propagation conditions for the tested signal, interferer 1 and interferer 2 are statistically independent. Note 3: SINR corresponds to of the tested signal as defined in clause 8.1. |  |  |  |  |  |  |  |  |

# Conclusion

Based on the discussion, we made the following observations:

Observation 1: The gain of BS MMSE-IRC over MMSE is around 1 to 1.5 dB with 2Rx in homogenous networks, which can be enough to define the BS IRC requirements.

Observation 2: The threshold gain of 3.1 dB in LTE for 2Rx case in homogenous network should not be considered to define NR requirements.

Observation 3: Low doppler and delay spreads should be prioritized for heterogenous networks, while larger doppler and delay spread configurations should be considered for homogenous network scenario.

Observation 4: The gain of MMSE-IRC over MMSE is approximately 0.5 to 1dB higher under TDLA30-10 than TDLC300-100 channel model for heterogenous network deployment. For homogenous network, a similar gain is seen with TDLB100-400 and TDLC300-100 channel models.

Based on the discussion, we made the following proposals:

Proposal 1: Introduce BS MMSE-IRC requirements with 2Rx for homogenous network scenario.

Proposal 2: Feasible to consider only 1 interferer for both homogenous and heterogenous network deployments for BS MMSE-IRC requirements with 2Rx.

Proposal 3: Use TDLA30-10 for heterogenous and TDLB100-400 for homogenous deployment scenarios for serving cell.

# References

[1] R4-2503909, “On general issues for MMSE-IRC receiver under interference”, Ericsson, RAN4#114-bis, Wuhan, Hubei, China.

[2] R4-2502272, “Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#114, Athens, Greece.

[3] 3GPP TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS (Release 13)”

[4] R4-2503908, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#114-bis, Wuhan, Hubei, China.