# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503916

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.3.2

Source:	China Telecom

Title:	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, RRM impacts for SBFD operation for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM impacts for SBFD operation for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

RRM impacts of SBFD operation

Status in the WF [1]:

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

For measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, it’s preferred to consider the UE measurement capability and separate requirements needs to be specified.

Proposal 1: Consider the UE measurement capability for measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case.

For accuracy requirements, 3 cases are summarized in the WF. In Case 1, the CSI-RS BW in DL sub-band #1 and sub-band #2 are both no less than the min BW with which the existing accuracy requirements apply, thus the existing accuracy requirements apply. In Case 2, the CSI-RS BW in one of DL sub-bands is no less than the min BW, considering the performance may not be worse than the consecutive CSI-RS resource, existing accuracy requirements apply. In Case 3, the CSI-RS BW in DL sub-band #1 and sub-band #2 are both less than the min BW, existing accuracy requirements could not apply, whether to define relaxed accuracy requirements can be further discussed.

Proposal 2: For accuracy requirements,

- In Case 1, existing accuracy requirements apply.

- In Case 2, existing accuracy requirements apply.

- In Case 3, existing accuracy requirements could not apply, whether to define relaxed accuracy requirements can be further discussed.

Status in the summary [2]:

- Issue 2-1-3: Requirements for scheduling restriction - Proposals related to UL/DL transition - Proposal 1 (Nokia): - The existing RAN4 scheduling restriction requirements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions. - Proposal 2 (CMCC, E///, HW, vivo): - RAN4 scheduling restriction requirement is not needed for DL/UL transition. - Proposals related to inter-frequency SSB measurement without gap - Proposal 1 (Nokia, MTK): - There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps. - The existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation. - Proposal 2 (HW): - RAN4 to discuss whether scheduling restriction applies when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots. - Proposal 3 (vivo): - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference. - Recommended WF for UL/DL transition - Do not define new scheduling restriction requirements for DL/UL transition. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

For scheduling restriction requirements for UL/DL transition, we support RAN4 scheduling restriction requirement is not needed for DL/UL transition.

Proposal 3: RAN4 scheduling restriction requirement is not needed for DL/UL transition.

Status in the summary [2]:

- Issue 2-1-7: Requirements for UL resource muting - Proposals - Proposal 1 (LGE, CTC, vivo): - RAN4 to confirm there is no RRM impact due to UL resource muting. - Proposal 2 (MTK): - Close the issue without agreement. - Recommended WF - As baseline, there is no RRM impact due to UL resource muting. - This can be revisited if impacts are identified in the future based on further RAN1 agreements.

For UL resource muting requirements, we support there is no RRM requirement impacted by UL resource muting.

Proposal 4: There is no RRM impact due to UL resource muting.

- Conclusion

The contribution presents our views on RRM impacts for SBFD operation for evolution of NR duplex operation, with the following proposals:

Proposal 1: Consider the UE measurement capability for measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case.

Proposal 2: For accuracy requirements,

- In Case 1, existing accuracy requirements apply.

- In Case 2, existing accuracy requirements apply.

- In Case 3, existing accuracy requirements could not apply, whether to define relaxed accuracy requirements can be further discussed.

Proposal 3: RAN4 scheduling restriction requirement is not needed for DL/UL transition.

Proposal 4: There is no RRM impact due to UL resource muting.

- References

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500536, Topic summary for [114][222] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114, 17 – 21 February, 2025.