# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503917

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.25.2.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, On-demand SSB SCell operation for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR issues captured in the WF [1], summary [2] and ad-hoc minutes [3].

- Discussion

OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Status in the WF [1]:

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

When NW re-activates the OD-SSB within the T2, we understand that it’s a little frequently to re-activate the OD-SSB within the T2, thus we are ok for option 1 that UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement, and if NW re-activates the OD-SSB within the T2, UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

For the start point of T2, we prefer to be based on the time point Tend, which is not related to the point of L3 measurement reporting for FMW. The duration can be defined as 5*measCycleSCell.

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement, and UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Proposal 2: The start point of T2 is based on the time point Tend.

Proposal 3: The duration can be 5*measCycleSCell.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Status in the WF [1]:

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

For Case 2 OD-SSB based deactivated SCell L3 measurement, it’s proposed to follow the same principle in Case 1 no matter whether the serving is known or unknown.

Proposal 4: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), The same approach in Case 1 can be reused.

OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Status in the WF [1]:

Sub-issue 3-1: Lower bound for measurement in FMW - Background - The agreement in last meeting is as follow. Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.. Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW.

For lower bound for measurement in FMW, prefer not to define lower bound to measurement in FMW.

Proposal 5: No lower bound is preferred for measurement in FMW.

Status in the WF [1]:

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, UE prioritizes the measurements for SCell with activated OD-SSB in FMW, longer delay is expected for other legacy SSB carriers.

Proposal 6: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, UE prioritizes the measurements for SCell with activated OD-SSB in FMW, longer delay is expected for other legacy SSB carriers.

OD-SSB based SCell activation

Status in the ad-hoc minutes [3]:

Issue 5-2: The known cell condition – period of known condition - The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition, such as max(5*measCycleSCell, 5*DRX cycles) in FR1. ### Moderator Proposal 5-2 The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition. - max(5*measCycleSCell, 5*DRX cycles) in FR1; - 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

UE performs deactivated SCell measurement based on OD-SSB within a time window, and after the time window, UE performs deactivated SCell measurement with a slow mode, which means UE shall meet legacy deactivated SCell measurement requirement after the time window. Therefore, it’s better to reuse the legacy period of known/unknown cell condition, i.e., max(5*measCycleSCell, 5*DRX cycles) in FR1, 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2, to cover both the case within the time window and the case after the time window.

Proposal 7: The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition:

- max(5*measCycleSCell, 5*DRX cycles) in FR1;

- 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

OD-SSB SCell activation scenarios

Status in the ad-hoc minutes [3]:

Issue 6-2: OD-SSB based multiple SCells activation ### Moderator Proposal 6-2 - FFS RAN4 to define OD-SSB based multiple SCells activation requirements. - RAN4 to limit the scope at least as all SCells being activated are OD-SSBs’ SCells - FFS whether to further limit the scope - It doesn’t impact the WI’s completion.

In our views, it’s open to define OD-SSB based multiple SCells activation requirements, the limitation that the scope at least as all SCells being activated are OD-SSBs’ Scells is needed, and it’s important that it doesn’t impact the WI’s completion.

Proposal 8: It’s open to define OD-SSB based multiple SCells activation requirements, the limitation is needed and it doesn’t impact the WI’s completion.

- Conclusion

The contribution presents our views on On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement, and UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Proposal 2: The start point of T2 is based on the time point Tend.

Proposal 3: The duration can be 5*measCycleSCell.

Proposal 4: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), The same approach in Case 1 can be reused.

Proposal 5: No lower bound is preferred for measurement in FMW.

Proposal 6: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, UE prioritizes the measurements for SCell with activated OD-SSB in FMW, longer delay is expected for other legacy SSB carriers.

Proposal 7: The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition:

- max(5*measCycleSCell, 5*DRX cycles) in FR1;

- 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

Proposal 8: It’s open to define OD-SSB based multiple SCells activation requirements, the limitation is needed and it doesn’t impact the WI’s completion.

- References

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500537, Topic summary for [114][223] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2502578, Ad-hoc minutes for Netw_Energy_NR_enh WI, Ericsson, RAN WG4 Meeting #114, 17 – 21 February, 2025.