# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503918

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.25.2.2

Source:	China Telecom

Title:	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, adaptation of common signal/channel transmission for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

SSB adaptation – L1 Requirements impacts

Status in the summary [2]:

Issue 1-2-4: L1 requirements impact after/without transition - Proposals - Option 1 (Measurement priority with relation to DRX): For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode. (Samsung) - Option 2 (Measurement behaviour at transition): UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation (Samsung) - Recommended WF: - Discuss above proposals.

For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, in order to guarantee the benefits of SSB adaptation, we support UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

Proposal 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

SSB adaptation – L3 Requirements impacts

Status in the WF [1]:

Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

For deactivated SCell measurement, we support to reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements for SSB adaptation, at least for the case when the SSB periodicity is adapted to be shorter than before.

Proposal 2: For deactivated SCell measurement, it’s proposed to reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements for SSB adaptation, at least for the case when the SSB periodicity is adapted to be shorter than before.

- Conclusion

The contribution presents our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.

Proposal 2: For deactivated SCell measurement, it’s proposed to reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements for SSB adaptation, at least for the case when the SSB periodicity is adapted to be shorter than before.

- References

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500538, Topic summary for [114][224] Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #114, 17 – 21 February, 2025.