3GPP TSG-RAN WG4 Meeting #114-bis 	R4-2503922

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.28.2

Source: 	Ericsson

Title:	On RRM requirements for XR

Document for:	Approval

# Introduction

The RAN4#114 WF is in [7], RAN4#113 WF is in [6]. The RAN4#112-bis WF is in [5]. The RAN4#112 agreements and WF are in [4]. The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1].

The following agreements were made in RAN4#114 [7]:

- Deployment scenarios

  - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

- L1/L2 triggered mobility measurements and procedures

  - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions.

  - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported.

  - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Principles for measurement delay calculation

  - Follow NR-U-like approach for designing the formulas for measurement delay extension.

- General on UAI

- Agree to define UAI based on the following assumptions, and further discuss the content of UAI

- RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective)

- Not define UE behaviour and the corresponding requirements based on UAI

- Supporting of UAI is optional for UE with optional UE capability.

RAN4 also received RAN1 LS [2], where RAN4 is requested to take the below information from [2] into account and decide whether or not to introduce any UE assistance information related to measurement occasions:

RAN1 Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Below we further discuss the issues related to RAN4 requirements impact for UE operating XR.

# Discussion

## Types of MGs

In [6], it was agreed that Type 1 MGs are used as a baseline, while the relevance for cancellation of other MG types and MG purposes needs further discussion in RAN4. For example, since the configured PRS (positioning) measurements may be intended for emergency positioning, and also because positioning measurements are handled by the core network node (LMF), it is proposed that MGs associated with PRS measurements shall not be cancelled. For MUSIM MGs, the MGRP can be already very long, so applying MG cancellation would even further delay all the measurements. As of concurrent gaps, it is difficult to discuss MG cancellation for UE configured with concurrent MGs, until the single MG pattern framework is settled.

- Proposal 1: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF).

- Proposal 2: MUSIM MGs shall not be cancelled.

- Proposal 3: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 4: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

## On the impact of UL XR operation

The UL XR operation can also be scheduled during the cancelled MGs, therefore the impact of scheduling the UL XR transmissions during the cancelled MGs needs to be discussed due to the potential impact on the RAN4 requirements.

At least the following issues need RAN4 discussion and their relation to the cancelled MGs:

Correct UL timing:

- the UE needs to have the correct UL timing prior staring the UL XR operation, which may result in delayed UL operation and lost resources of the cancelled MGs.

A guard period,

- which is needed for DL-to-UL switching for UL XR operation within the target MG, to avoid UL-to-DL interference.

RF switching:

- needed for the UE to switch from DL to UL and back.

- Proposal 5: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

  - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

  - Issue 2: Guard period for DL-to-UL switching,

  - Issue 3: Impact of RF switching from DL to UL and back.

- Proposal 6: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

- Proposal 7: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

- Proposal 8: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

## On UE assistance information for XR

RAN1 has been discussing UE assistance data to suggest which and/or how RRM measurement occasions (e.g., in measurement gaps) are to be reallocated to XR. According to [2], RAN1 discussed whether the impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the max number of MGs/SMTC with restrictions that can be skipped within a time period,

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

In RAN4#114, there was agreed to not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) and to not specify any UE behaviour and the corresponding requirements based on UAI. The discussion on UAI will continue, based on these agreements.

On the information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period (mentioned in RAN1 LS): this is more related to RRM measurement performance and therefore can be pre-defined in the specification (TS 38.133). Furthermore, such number may depend on MGRP and DRX cycle length, so it can be more relevant to specify the maximum time between the remaining occasions available for RRM measurements.

- Proposal 9: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133, without the need to signal it in UAI.

On patterns in UAI (mentioned in RAN1 LS): If such patterns are defined, there can be ambiguity in patterns sent by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle.

- Proposal 10: No time-domain patterns are provided in UAI, e.g., because of scheduling impact and the potential ambiguity in patterns indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle, and the UE is not required to perform measurements more frequently than once per DRX cycle, which should also be followed for XR.

## DRX impact on MG cancellation

In addition to all the above considerations, we also note that a UE can be configured with DRX, which also has own periodicities (which can be longer or shorter than MGRP) and DRX ON duration, not to mention that DRX and XR can be configured by different network nodes (e.g., with NR-DC). When configured with DRX, the UE is not required to perform measurements more than once per DRX cycle (which may make measurements less frequent than with MGRP without DRX), and the UE is not expected to monitor control channel outside DRX ON time period. Hence, not all MG occasions are relevant for cancelling in favour of XR.

- Proposal 11: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

## Semi-static MG cancellation schemes

In the last 3GPP meeting, RAN2 has agreed to not further discuss semi-static MG cancellation schemes, and RAN1 confirmed the earlier RAN1 agreement on the DCI approach (see below).

Agreement Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.. Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

- Proposal 12: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

## RRM requirements enhancements for XR

NR-U like approach was agreed for defining measurement period for measurements impacted by MG cancellation under XR operation.

### Inter-frequency cell detection

- Proposal 13: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 14: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 15: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

### SSB time index detection

- Proposal 16: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 17: The maximum numbers of extensions of inter-frequency SSB time index detection period are for

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 18: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

### Inter-frequency measurements

- Proposal 19: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, Ceil(8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, Ceil(Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 20: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 21: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

upon exceeding Xmeas,max the UE shall restart the measurement.

### Applicability conditions

Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 22: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 23: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

### UE power class

- Proposal 24: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF).

- Proposal 2: MUSIM MGs shall not be cancelled.

- Proposal 3: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 4: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 5: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

  - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

  - Issue 2: Guard period for DL-to-UL switching,

  - Issue 3: Impact of RF switching from DL to UL and back.

- Proposal 6: For transmitting UL XR data during the cancelled MG occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled MG.

- Proposal 7: For transmitting UL XR data during the cancelled MG occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled MG.

- Proposal 8: When receiving a DCI scheduling UL XR data during a cancelled MG occasion, it is assumed that the UE will have the correct UL timing prior to the transmission of the UL XR data during the cancelled MG occasion.

- Proposal 9: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133, without the need to signal it in UAI.

- Proposal 10: No time-domain patterns are provided in UAI, e.g., because of scheduling impact and the potential ambiguity in patterns indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle, and the UE is not required to perform measurements more frequently than once per DRX cycle, which should also be followed for XR.

- Proposal 11: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

- Proposal 12: RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

- Proposal 13: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 14: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 15: UE behaviour when the maximum number of extensions for cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

- Proposal 16: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 17: The maximum numbers of extensions of inter-frequency SSB time index detection period are:

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 18: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

- Proposal 19: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, Ceil(8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, Ceil(Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 20: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 21: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

upon exceeding Xmeas,max the UE shall restart the measurement.

- Proposal 22: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Proposal 23: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

- Proposal 24: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

[6] R4-2420103, WF on RRM requirements for XR_Ph3, Qualcomm, Nov. 2024.

[7] R4-2502700, WF on RRM requirements for XR_Ph3, Nokia, Feb. 2025.

[8] R4-2503921, draftCR to 38.133, Inter-frequency measurement requirements with XR, Ericsson, Apr. 2025.