3GPP TSG-RAN WG4 Meeting #114bis	R4-2503926

Wuhan, China, Apr 7th – Apr 11th, 2025

Source:	Nokia

Title:	Simulations for UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN4#114 final agreements were made for companies to provide simulation results for all agreed test cases. In the following we will provide Nokia’s simulation results.

Simulation results

PDSCH requirements with inter cell interference

Agreed setup

The following configuration parameters are extracted from the agreements made in RAN4#113 and RAN4#114 (see [2] & [3]). For remaining parameters, we have re-used the configuration from in Rel.17 2Rx/4Rx:

| Parameter | Value | Comments |
| --- | --- | --- |
| Receivers | MMSE-MRC, MMSE-IRC (baseline), MMSE-IRC(simplified) |  |
| Interference profile | Reuse from Rel-17 2/4Rx |  |
| Network type and SCS/CBW | - FDD: 15kHz/10MHz - TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |  |
| Receiver | Baseline Simplified |  |
| Propagation conditions | TDLA30-10 |  |
| Rank + MCS | Rank 2: MCS17, 2T8R ULA Low Rank 4: MCS13, 4T8R ULA Low |  |
| Interference cell | - 16QAM - 2Tx - Frequency shift to target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells |  |

Simulation results

Table 1 shows Nokia’s PDSCH simulation results with inter-cell interference.

Table 1: PDSCH simulation results with inter-cell interference (SNR [dB] @ 70% Max TP)

| TC# | Receiver | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 6.7 |
| 2 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 8.3 |
| 3 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 10.7 |
| 4 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 14.8 |
| 5 | Baseline | TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 6.9 |
| 6 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 8.5 |
| 7 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 10.7 |
| 8 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 15.3 |

Table 1: PDSCH simulation results with inter-cell interference. Impairment results (SNR [dB] @ 70% Max TP)

| TC# | Receiver | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 9.2 |
| 2 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 10.8 |
| 3 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 13.2 |
| 4 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 17.3 |
| 5 | Baseline | TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 9.4 |
| 6 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 11.0 |
| 7 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 13.2 |
| 8 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 17.8 |

PDSCH requirements with intra cell inter user interference

Agreed setup

The following configuration parameters are extracted from the agreements made in RAN4#112bis and RAN4#113 (see [1][2]). For remaining parameters, we have re-used the configuration from in Rel.17 2Rx/4Rx:

| Parameter | Value | Comments |
| --- | --- | --- |
| Receivers | MMSE-MRC, MMSE-IRC (baseline), MMSE-IRC(simplified) |  |
| NoF CoUEs | 1 |  |
| Network type and SCS/CBW | - FDD: 15kHz/10MHz - TDD: 30 kHz/40MHz |  |
| Propagation conditions | TDLA30-10 for all configurations. |  |
| Rank + MCS | Rank 2+2: MCS17, 4T8R ULA Low, orthogonal precoder Rank 4+4: MCS13, 8T8R ULA Low, orthogonal precoder |  |
| Precoder for target UE | Target UE: Random precoder |  |
| DMRS ports | - Rank 2+2, reuse same configuration as Rel-17 - Rank 4+4, use different CDM group for 4+4 case. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007}. |  |
| Codebook | Type I Single Panel only |  |
| Other parameters | - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain: Full PRB |  |

Simulation results

Table 2 show Nokia’s PDSCH simulation results with intra-cell inter-user interference

Table 2: PDSCH simulation results with intra-cell inter-user interference (SNR [dB] @ 70% Max TP)

| TC Number | Receiver | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 10.3 |
| 2 |  |  |  | 8T8R | 4+4 | MCS 13 |  |  |  |  | 13.7 |
| 3 | Simplified |  |  | 4T8R | 2+2 | MCS 17 |  |  |  |  | 13.5 |
| 4 | Baseline | TDD 30KHz SCS | 40 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 10.5 |
| 5 |  |  |  | 8T8R | 4+4 | MCS 13 |  |  |  |  | 13.7 |
| 6 | Simplified |  |  | 4T8R | 2+2 | MCS 17 |  |  |  |  | 13.7 |

Table 2: PDSCH simulation results with intra-cell inter-user interference. Impairment results (SNR [dB] @ 70% Max TP)

| TC Number | Receiver | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 12.8 |
| 2 |  |  |  | 8T8R | 4+4 | MCS 13 |  |  |  |  | 16.2 |
| 3 | Simplified |  |  | 4T8R | 2+2 | MCS 17 |  |  |  |  | 16.0 |
| 4 | Baseline | TDD 30KHz SCS | 40 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 13.0 |
| 5 |  |  |  | 8T8R | 4+4 | MCS 13 |  |  |  |  | 16.2 |
| 6 | Simplified |  |  | 4T8R | 2+2 | MCS 17 |  |  |  |  | 16.2 |

Conclusion

In this paper we have provided Nokia’s simulation results for:

- PDSCH with Inter-cell interference.

- PDSCH with Intra-cell inter-user interference.

- References

- R4-2416555 - WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia

- R4-2419784 - WF for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- R4-2502275 - WF for [114][324] NR_demod_Ph5_Part2_UE