3GPP TSG-RAN WG4 Meeting #114bis	    R4-2503928

Wuhan, China, 07 – 11 April 2025

Agenda Item:	7.21.1

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

# Introduction

This contribution discusses the general issues related to the AI/ML aided mobility. The impact of the measurement error on the accuracy of the predicted measurements and aspects related to LCM requirements are particularly discussed in this paper.

# Discussion

## Impact of measurement error on prediction accuracy

To reduce the measurement overhead and measurement delay for network triggered L3-based mobility, prediction of RRM measurements in time domain, frequency domain, and spatial domain are being studied in Rel. 19. To predict the RRM measurement, the UE needs to perform some measurements that it can feed into its AI/ML model as input as shown in Figure 1.

Figure 1. Elaboration of prediction and predicted measurement/event.

Based on the evaluations done by RAN2, the legacy measurements (L3 filtered RSRP measurements) can be used as an input to the AI/ML model for RRM measurement prediction. The results from the evaluation highlight that the RRM measurement prediction can provide gain in terms of the measurement overhead and delay reductions in comparison to the legacy approach. These gains should however not come at a cost of degradation in the accuracy of the measurement reported to the network for the L3-based mobility when the RRM measurements reported by the UE to the network are the predicted measurements. The accuracy of measurements predicted using an AI/ML model heavily depends on the accuracy of the measurements provided as an input to the AI/ML model as acknowledged by RAN4 in the last meeting where the following agreement was made:

Agreement from RAN4#114bis [1]:

- For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

Impact of the error in the measurement(s) used as input(s) for RRM measurement prediction on the accuracy of the predicted measurement/event needs to be investigated to at least address the following two issues:

- Issue #1: Relates to the impact of error in the measurements used as input(s) to the AI/ML model on the accuracy of the predicted measurement.

- Issue #2: Relates to the accuracy of the measurement(s) used as input(s) to the AI/ML model to guarantee that the UE predicted measurement meets the requirements for L3-based mobility.

In the last meeting when this issue was discussed, some concerns on running evaluation campaign in RAN4 to conclude the impact of error in measurements used as an input to AI/ML model for prediction on the accuracy of the predicted measurements were expressed. The concerns were mainly stemming from the limited TU allocated to RAN4 during the study item phase. RAN2 is the primary working group leading the evaluation work, sending an LS highlighting the issue was also discussed as an alternative to address the situation in RAN4. Since, the details of the error model could not be concluded an agreement to send an LS to RAN2 could not be reached.

When the predicted measurements are reported by the UE to the network, it is important that the predicted measurements are accurate enough so that the predicted measurements are useful for the network to make mobility related decisions. One way to achieve this is by ensuring that the predicted measurements at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.  For example, requirements for intra-frequency measurement can be applicable to prediction in temporal domain and inter-frequency measurement requirements can be applicable to frequency domain prediction. Given the framework for pure spatial domain prediction is unclear, if or which of the existing requirements should apply to spatial domain prediction remains to be understood. From the study item point of view, this could be one of the conclusions that RAN4 could derive. The details of the issue #1 and issue #2 above can then be revisited during the normative work given that the WI for AI/ML based mobility is agreed to be part of the future release.

Proposal 1: RAN4 to conclude that the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Observation 1: If or which of the existing requirements should apply to spatial domain prediction remains to be understood.

## LCM requirements

The framework for LCM can be found in TR 38.843 [2]. The LCM framework consists of procedures for which requirements may need to be defined by RAN4. For example, activation/deactivation of the model, fallback to non-AI/ML operation, and performance monitoring are some of the procedures that will be executed by the UE for the case where the AI/ML model(s) for mobility is hosted by the UE. These procedures are being discussed in RAN4 to identify the potential requirements to be specified in the context of the Rel. 19 AI/ML for air interface work item. The agreements reached for the use cases supported in AI/ML for air interface can either be potentially applied to the AI/ML based mobility or can be used as a starting point for the discussion on aspects that are specific to AI/ML based mobility.

For AI/ML based mobility, AI/ML model can also be deployed at the network side. The network may in this setup configure UE to report the measurements for training and or inference purposes commonly addressed by data collection procedure within the LCM framework in [2]. This issue is not being discussed as a part of ongoing discussion in RAN4 for the AI/ML for air interface work item. As the performance of the AI/ML model at the network side relies on the data reported by the UE, RAN4 should discuss to identify relevant requirements for the data collection procedure for the network sided model.

Proposal 2: RAN4 to identify requirements to be defined for data collection for network sided model for AI/ML based mobility.

# Summary

In this contribution issue that in general has an impact on all prediction types and scenarios considered for AI/ML based mobility are addressed. The proposal below summarizes the discussion presented in this paper.

Proposal 1: RAN4 to conclude that the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Observation 1: If or which of the existing requirements should apply to spatial domain prediction remains to be understood.

Proposal 2: RAN4 to identify requirements to be defined for data collection for network sided model for AI/ML based mobility.

# References

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 3GPP.