3GPP TSG RAN WG4 Meeting #114bis	R4-2503933

Wuhan, China, 07 Apr. 2025 – 11 Apr. 2025

Agenda Item:	7.7.3

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. In this contribution, we analyze the potential impact to RAN4 RRM requirements based on issues identified in last meeting WF, R4-2502588.

# Discussion

In the previous meeting, RAN4 began examining the impact of RRM requirements for SDL SCell and FDD PCell. We believe the existing RRM requirements can be used as baseline. We propose that the RRM impact can be managed using two methods.

Method 1: Discussing a method to extend the delay requirement if SSB/SMTC/MG/CSI-RS/DRX ON periodicity are missed at UE due to switching to another cell. Once we agree on this delay extension principle, it can be applied to all related procedures on PCell and SCell. This approach was similarly utilized in Release 16 for defining requirements on a carrier subject to CCA.

Method 2: for some of the requirements, without extending or relaxing the particular requirement, we can specify an applicability condition under which the requirements are valid for low-band CA via switching.

We think RAN4 does not need to choose between method 1 and 2. Some of the requirement’s impact can be addressed with method 1 and some other requirements impact can be addressed with method 2.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

The rest of the discussion is organized as follows:

- RRM requirements impact for SDL SCell

- RRM requirements impact for FDD PCell

- Timing requirements

- LB CA switching delay and potential interruptions

## RRM requirements impact for SDL SCell

### When the SCell is deactivated

When the SCell is in deactivated state, the UE is not required to measure CSI; it only needs to perform L3 measurements. The periodicity for these L3 measurements is determined by the measCycleSCell parameter, as outlined in in Table 9.2.5.2.3 of TS 38.133, which is reproduced below for reference. This table specifies the measurement period for intra-frequency measurements without gaps in FR1.

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil (5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil (5 x Kp) x max (measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil (5 x Kp) x max (measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell |  |

Additional details on measCycleSCell can be found in TS 38.331 and are also included below for reference. The minimum measurement cycle for the SCell is 160 ms, while the maximum is 1280 ms.

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL    -- Need R

It is important to note that the UE is not required to measure more frequently than the specified measCycleSCell. Implementing a switch pattern with only one slot within intervals like 160 ms, 640 ms, or 1280 ms can be difficult because the switch pattern length should ideally support 1280ms. For example, when the measCycleSCell is configured as 1280ms, the UE only needs to switch to the SDL SCell once every 1280 ms and offering more frequent switching opportunities could lead to inefficient use of NW resources. When the measurement cycle is 1280 ms, the UE has flexibility regarding the exact occasion to perform the measurement and the occasion of the measurement can be up to UE implementation.

Considering the unpredictable timing of the UE's measurements on the deactivated SCell from the NW's perspective, and the fact that a switch pattern with a duration of 1280 ms would cause additional signalling overhead at the NW side, activating the switch pattern during the SCell's deactivated state is not advantageous. However, preventing the UE from performing L3 measurements could result in longer activation delays for the SCell, which may not be acceptable to all stakeholders.

One solution could be to define the measurements requirements for the deactivated SCell, without the switch pattern being active during the deactivated state. This would result in interruption on the PCell, but we think that may not be a major issue compared to switch pattern being active when the SCell is in deactivated state. If the switch pattern is active when the SCell is in deactivated state, NW cannot use that slot for the PCell transmission and causing interruption on the PCell. For a measCycleSCell of 1280ms, and SSB periodicity of 20ms, potentially NW may need to provide 64 switch locations to the SCell and resulting in 64 interruptions on the PCell. If we allow UE to perform measurements based on the UE autonomously chosen occasion, it might result in just 1 interruption on the PCell.

- The RAN1 defined switch pattern for low band CA via switching should not be applicable when the SCell is in deactivated state for the UE.

In the existing specification as well, as pointed by other companies in last meeting, the interruption requirements are specified for deactivated SCell measurements when the measCycleSCell is 640ms or larger. Some part of the interruption requirement of the existing spec is produced below for reference.

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. - If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

Unlike legacy interruption requirement, for low band CA via switching, we believe that interruption requirements should apply to all values of measCycleSCell. It's important to note that the reasons for interruption differ between legacy deactivated SCell measurements and deactivated SDL SCell measurements. In legacy, the UE does not need to remain on the SCC or keep the RF chain active if the measCycleSCell is 640 ms or longer. However, in the low band CA via switching scenario, the UE must switch to the deactivated SCell to perform measurements, requiring active transitions regardless of the measurement cycle duration.

- RAN4 to define the interruption requirement to allow SDL SCell measurement on deactivated SCell without switch pattern being applicable.

Since UE need to switch to SDL SCell for the measurements purpose, we could allow interruptions for shorter measCycleSCell but for longer measCycleSCell UE may have opportunity to select the measurement occasion which can avoid the interruption. RAN4 can further study the principle to specify the interruptions due to deactivated SDL SCell measurements.

- RAN4 to further study the principle to specify the interruptions due to deactivated SDL SCell measurements.

### When the SCell is being activated

As per the existing RAN4 SCell activation requirement for the deactivated SCell, upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3],Tactivation_time is the SCell activation delay in millisecond.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

We would like to emphasize that the UE is only required to receive SSB/CSI-RS from the SCell before sending a valid CSI report (NW may schedule the UE, but the demodulation performance is not guaranteed). During this time, the UE primarily needs to access the SCell to complete activation-related actions. These actions may include one or more of the following: coarse Automatic Gain Control (AGC), fine AGC, cell search, L1-RSRP measurement, and fine timing adjustment. For a SSB periodicity of 20 ms, in the worst-case scenario, the UE may need approximately 80-100 ms to complete these actions before commencing valid CSI acquisition. In the context of low band CA via switching this may further increase if the switch pattern cannot ensure the availability of SSB on all the SSB occasions belonging to SDL SCell. During this period, the pattern for the SDL SCell does not need to include more than the slots containing e.g., SSB, CSI-RS, as doing so would interrupt PCell transmission, and the UE cannot utilize the SDL SCell until the SCell activation is complete. Having said that, after the SCell activation, the majority of the UE's downlink data may be routed through the SDL SCell, as its primary role in the low band CA via switching context is to balance the load on the PCell.

- During the SCell activation, UE may need to receive the SCell for SSB measurement and CSI-RS for the CSI acquisition.

- After the SCell activation, the majority of the UE's downlink data may be routed through the SDL SCell and hence the pattern should consist of more slots allocated to SDL SCell

Based on the observation 1 and 2, the number of slots that needs to be allocated to SDL SCell will vary significantly and hence we believe having different pattern before the SCell activation complete and while the SCell activation is in progress seems reasonable to avoid the RRC reconfiguration after the CQI report and to avoid the wastage of resources before the CQI report is sent. there may be couple of approaches to solve the issue.

- allowing UE to switch to SDL SCell to receive the SSB and CSI-RS for CSI computation during the activation phase without the pattern being active  during the SCell activation phase. We would like to point that this is an implicit pattern than the RRC configured explicit pattern.

- Defining two different patterns and one pattern being applicable during the activation phase and other is active after the first valid CSI report is sent.  This approach needs LS to RAN1 if RAN1 is not discussing it.

- RAN4 to agree on one of the following methods and send LS to RAN1, when the SCell is being activated:

- Allow UE to switch to SDL SCell to receive the SSB for AGC, cell search, L1-RSRP/fine timing, and CSI-RS for CSI computation during the activation phase without the pattern being active during the SCell activation phase.

- Defining two different patterns and one pattern being applicable during the activation phase and other is active after the first valid CSI report is sent.

UE measurement behaviour during the SCell activation:

As per the existing requirements, UE is expected to perform AGC only during the overlapping periodicity of SSB or SMTC. However, in the low band CA context, SSB occasions are not overlapped from UE perspective as UE can receive only from one cell at a time. Hence, no need to consider SSB max or SMTC max in the SCell activation requirements for the purpose of AGC.

- RAN4 to agree on not using SSB Max or SMTC Max in the SCell activation requirements for the SDL SCell.

The next point to consider is whether to conduct the AGC measurement during the SSB occasion or the SMTC occasion. SMTC occasion-based activation was introduced to save UE power. However, since low-band carrier aggregation through switching functions in half-duplex mode between the FDD cell and the SDL cell, the number of SSB/SMTC occasions may be reduced compared to standard CA operation. Therefore, we prefer SSB-based measurement during activation over SMTC-based measurement for faster SCell activation and efficient load balancing. We would like to point that the similar enhancements (e.g., shortMeasInterval-r18) are introduced as part of the Rel-18 SCell enhancements.

- RAN4 to agree on SSB period based SCell activation instead of SMTC period based SCell activation for the LB CA operation.

### When the SCell is activated

When the SCell is activated, the UE must carry out several RRM procedures. We will examine how the switch pattern affects these procedures based on the following agreed WF in the last meeting.

For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

#### L3 measurements

While there are other type of measurement requirements that are impacted, we consider intra-frequency without gap as an example to discuss the principle for RRM requirement relaxation or extending the delay The measurement period for L3 measurements is as mentioned below.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max (200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max (200 ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

According to the measurement period outlined in the above table, the measurement period relies on the parameters SMTC, Kp, and CSSFintra. Due to the switch pattern and the half-duplex nature of the LB CA via switching, these parameters may be affected. We look at the defintion of these parameters for analyzing the RRM requirements impact.

Kp: This factor determines the number of SMTC occasions not overlapping with any MG relative to the total available SMTC occasions, including those overlapped by gaps. Existing defintion of the Kp is shown below.

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively. Kp = 1 when Navailable = 0. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap. Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Because of the switch pattern design, introducing a new scaling factor, Kp_LB, is necessary. Kp_LB will account for SMTC occasions on the relevant cell that neither overlap with gaps nor are missed due to switch patterns. This scaling factor can be defined to capture the SMTC occasions available for L3 measurement and same is highlighted in green, as illustrated in Figure 2. The precise formula for Kp_LB is contingent on RAN1 design and requires further analysis (FFS).

Figure 1: L3 measurement ocassions for LB CA

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

SMTC Period: Certain SMTC occasions may be unavailable for L3 measurement due to switching between cells, and it can result in a prolonged measurement delay. We think the non availability of the SMTC occasion for the L3 measurement can be captured in the Kp_LB design. Due to this, we think, SMTC period defintion need not change.

CSSFintra: When all the Measurement Gap (MG) and SMTC occasions overlap, the inter-frequency and intra-frequency carriers must share the SMTC occasion. Again, since this is influenced by the RAN1 pattern design, we propose to defer the final determination of the CSSFintra factor, marking it as FFS.

- RAN4 to further discuss CSSFintra for the LB CA.

#### L1 measurement requirements:

We look at the legacy L1-RSRP requirements for the impact analysis for LB CA L1 measurement requirements and later we check if we can apply same principles for other L1 measurements as well.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise For FR1, - P=1/(1-T_SSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and - P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.

From the table above, we observe that there is no need to change the value of M, as the number of samples remains consistent with legacy requirements for L1-RSRP measurements. However, switching between FDD and SDL may result in some SSBs being unavailable for L1-RSRP measurement. In legacy, when SSBs overlap with MG, a sharing factor P is used to address the non-availability of SSBs due to overlap of SSB with MG, thus specifying the remaining SSB occasions for L1-RSRP. In LB CA with switching, SSBs might be unavailable for L1-RSRP not only due to switching between FDD and SDL but also because of overlap with MG (similar to legacy). Both factors must be considered when defining SSB occasions for L1-RSRP measurement or a sharing factor for L1-RSRP. A new sharing factor, PLB, can be introduced to account for the non-availability of SSBs due to the switch pattern and overlap with MG. PLB can be defined to capture the SSB occasions available for L1-RSRP measurement and same occasions are highlighted in green, as shown in figure 3. Since PLB addresses SSB availability, we propose reusing the existing TSSB definition, TDRX and TReport.

Figure 2: L1-RSRP measurement occasions for LB CA

We observe that RAN4 followed similar approach for defining the BFD, CBD and L1-SINR requirements. We copy the relevant requirements below for reference.

BFD requirements: Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 P) TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 P) TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB. - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. CBD requirements: Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 P) TSSB). DRX cycle > 320ms; Ceil(3 P) TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB, - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. L1-SINR requirements: Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1 Configuration; TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320 ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320 ms; ceil(M*P)*TDRX. NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity..

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

#### TCI state switch delay

We summarize different TCI state switch delay supported in the existing spec. We did not consider unified TCI state switch for the analysis, but we expect similar conclusions should be applicable for unified TCI state switch as well.

|  | Known TCI state | Unknown TCI state | Comment |
| --- | --- | --- | --- |
| MAC CE based | n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length | n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length | - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| DCI based | timeDurationForQCL |  |  |
| RRC based | n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length | n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length | Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD; - Tfirst-SSB is time to first SSB transmission after RRC processing time at the UE for other QCL types; - The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| Active TCI state list update | n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. | NA | As defined in MAC CE based switch |

Among the delay components outlined in the table above regarding legacy requirements, we think THARQ is not impacted due to LB CA via switching as it is configured by the NW using the k1 value and the NW configuration already considers the switch pattern. Among the other components, we believe the parameter that is affected by the switch pattern is Tfirst-SSB. This is because Tfirst-SSB involves the UE accessing SSB signals for measurement, and the time taken to access these signals depends on the switch pattern. We consider other components to be related to UE processing delays, which we believe remain unaffected. Therefore, we propose redefining Tfirst-SSB for LB CA via switching, suggesting it should be defined as the first SSB available on the respective cell.

Due to UE operation on the SDL, and the potential HARQ delay on the FDD carrier, UE may receive the SSB on the SDL carrier before sending the HARQ on the FDD carrier. In this case, we think the delay should be max (HARQ, SSB reception time). RAN4 can further work on the delay equation.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

## RRM requirements impact for FDD PCell

We think the RRM requirements analysis of L3 measurement, L1 measurements should hold true for PCell as well.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well.

Other than the requirements discussed for the SDL SCell, we think couple of procedures such as RLM and CBD on the PCell needs special attention.

RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell.

Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected.

Based on the switch pattern, the availability of RLM-RS on the PCell may be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SDL SCell do not have UL support, HARQ feedback is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM and faster link recovery on the PCell. For example, if any of the following condition is met, UE could prioritize performing RLM and link recovery on the slots where the RLM-RS or SSB is transmitted by ignoring the pattern on those slots.

- N1 number of out of sync are triggered on PCell; or

- T310 timer is started.

Similarly, if the BFD is detected on the PCell, UE could prioritize performing CBD on the slots where the CBD-RS is transmitted by ignoring the pattern on those slots (i.e., even if the slot containing CBD-RS belonging to SCell, UE prioritize the CBD-RS reception)

- RAN4 to further discuss the switch pattern applicability when the RLM or BFD is triggered on the PCell and study potential solution for faster link recovery.

## Timing requirements

In RAN4#114 we had the following agreements:

… Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us …

When it coms to the issue of:

“requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting”

We note, as we stated also in RAN4#114, that in 3GPP rel-18 NES WI the following was added to TS 38.133, section 8.3.2. This means that 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

… For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB ....... and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, …

- 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

We can re-use the system development in rel-18 NES since the use case in LB-LB CA with switching in rel-19 when there is no SSB on SDL-carrier is similar to the case we already standardized in RRM TS 38.133 for rel-18 NES inter-band CA without SSB on SCell.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SDL-carrier for via switching in rel-19.

Regarding the issue:

“As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session.

- Option 1: 4.67us

- Option 2: 3 us”

We prefer option 1, for a couple of reasons. The first reason is that for MRTD in general we have:

RTD=TAE+ ∆RF_propagation+RF_channel_dispersion

Where TAE is BS TX Time alignment Error, ∆RF_propagationis the time difference from the BS TX to UE for the different carriers. In our case we are colocated so ∆RF_propagation=0. Finally RF_channel_dispersion is the radio channel delay profile for the inter band case. This means that for our case we get, for the inter band case:

RTD=TAE+RF_channel_dispersion

and we conclude that: RTD > TAE for inter band.

The second reason is that in existing rel-18 NES we already have MRTD = CP. This means that option 1 is consistent with our proposal to use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SDL-carrier for via switching in rel-19.

- For inter-band colocated sites we have RTD=TAE+RF_channel_dispersion → RTD> TAE and in NES rel-18 WI the MTRD = CP.

These two reasons gives us the conclusion that MRTD = CP for in LB-LB CA with switching.

- MRTD = CP for in LB-LB CA with switching

## LB CA switching delay and potential interruptions

In last meeting, we have discussed whether there is a need to define interruption requirement or not in the RRM specifications. Status of the discussion is captured in the WF below.

Interruption requirement

- Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung)

  - Capabilities for UEs with and without interruption (Samsung)

- Alt2:	RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM

- Session Chair (RRM session): check the progress in RF session and discuss if any aspects are not covered by the RF discussion.

In the last meeting, we suggested waiting for the conclusion of the RF session before agreeing on defining the interruption requirement. However, after further analysis and internal review, we are comfortable defining the interruption requirements in the RRM specification. To do so, we believe it's important to first agree on whether these interruptions should be specified in symbols or slots. To facilitate this decision, we have reproduced the exemplary pattern we considered in the RAN1 paper, as shown in the figure 3. Note that our preference is the 3rd diagram in this figure with 40 ms pattern periodicity since it offers the most opportunities for load sharing between the FDD and SDL carriers.

Figure 3: Exemplary switch patterns for LB CA via switching for load balancing.

As illustrated in Figure 3, for load balancing, the UE may need to switch between SDL and FDD multiple times within a 40 ms period. If we define interruptions in terms of slots, we lose 8 out of 40 slots, resulting in a 20% reduction in total capacity. To determine the percentage of capacity lost due to symbol-level interruptions, we calculate the total number of interrupted symbols, as shown in the table below. We have used RF Switch delay + BS TAE+ Transient period+ N_TA offset + TA for calculating the total number of interrupted symbols.

|  | RF switching delay | BS TAE | Transient period | NTA offset | TA (9kms cell radius) | Total interrupted symbols |
| --- | --- | --- | --- | --- | --- | --- |
| FDD to SDL | 35 | 3µs | 10µs | 13µs | NA | 1 |
| FDD to SDL | 140 | 3µs | 10µs | 13µs | NA | 3 |
| SDL to FDD | 35 | 3µs | 10µs | 13µs | 60µs | 2 |
| SDL to FDD | 140 | 3µs | 10µs | 13µs | 60µs | 4 |

The interruption percentage for a UE with a 35 µs switch delay is 2% (12 out of 560 symbols), while for a UE with a 140 µs switch delay, it is 5% (12 out of 560 symbols). Given the significant difference in the percentage loss of available symbols for scheduling, we propose defining interruptions in terms of the number of symbols. Based on the above analysis we make following observation.

- Based on the below table, difference between symbol level and slot level interruption is high.

|  | Symbol level interruption percentage | Slot level interruption percentage |
| --- | --- | --- |
| RF switching delay 35µs | 2 | 20 |
| RF switching delay 140µs | 5 | 20 |

- RAN4 to define interruption requirement in symbol level in RRM specification.

- The interruption length can be FFS based on the RAN1 and RF session conclusions.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

- The RAN1 defined switch pattern for low band CA via switching should not be applicable when the SCell is in deactivated state for the UE.

- RAN4 to define the interruption requirement to allow SDL SCell measurement on deactivated SCell without switch pattern being applicable.

- RAN4 to further study the principle to specify the interruptions due to deactivated SDL SCell measurements.

- RAN4 to agree on one of the following methods and send LS to RAN1, when the SCell is being activated:

- Allow UE to switch to SDL SCell to receive the SSB for AGC, cell search, L1-RSRP/fine timing, and CSI-RS for CSI computation during the activation phase without the pattern being active during the SCell activation phase.

- Defining two different patterns and one pattern being applicable during the activation phase and other is active after the first valid CSI report is sent.

- RAN4 to agree on not using SSB Max or SMTC Max in the SCell activation requirements for the SDL SCell.

- RAN4 to agree on SSB period based SCell activation instead of SMTC period based SCell activation for the LB CA operation.

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

- RAN4 to further discuss CSSFintra for the LB CA.

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well.

- RAN4 to further discuss the switch pattern applicability when the RLM or BFD is triggered on the PCell and study potential solution for faster link recovery.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SDL-carrier for via switching in rel-19.

- MRTD = CP for in LB-LB CA with switching.

- Based on the below table, difference between symbol level and slot level interruption is high.

|  | Symbol level interruption percentage | Slot level interruption percentage |
| --- | --- | --- |
| RF switching delay 35µs | 2 | 20 |
| RF switching delay 140µs | 5 | 20 |

- RAN4 to define interruption requirement in symbol level in RRM specification.

- The interruption length can be FFS based on the RAN1 and RF session conclusions.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537

[3]		R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple)