3GPP TSG RAN WG4 Meeting #114bis	R4-2503936

Wuhan, China, 07 Apr. 2025 – 11 Apr. 2025

Agenda Item:	7.27.2.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements based L1 measurement requirements.

# Discussion

## Event triggered measurement requirements

### L1-RSRP requirements for event triggered reporting

Event triggered L1-RSRP reporting delay:

Following agreements are reached for defining the event triggered measurement reporting delay.

- Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

In last meeting, it was agreed that the measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

A proposal was made to categorize delays into periodic and non-periodic event-triggered measurement reporting. However, we believe this classification is unnecessary. The primary reason is that the UE should maintain consistent measurement behavior, regardless of whether the reporting is event-triggered periodic or event triggered non-periodic. It's important to recognize that a UE can be configured for multiple events simultaneously, and altering its measurement behavior upon meeting the triggering conditions for one event could potentially disrupt the evaluation of others.

We understand that the proponent's intention was that, in non-event-triggered periodic reporting, the UE could schedule measurement occasions based on the reporting periodicity. In contrast, event-triggered periodic reporting specifies the number of reports and the interval between them. We believe that the number of reports for event-triggered periodic reporting can be limited, and there is no need to define a different measurement behavior for this type of reporting. Additionally, legacy requirements for L3 event-triggered periodic reporting have defined reporting delays in relation to the first report. Therefore, maintaining uniform measurement behavior is crucial for ensuring accurate event evaluations and avoiding unintended disruptions in UE performance.

Based on the above analysis we make the following proposal.

- RAN4 not to define different requirements for the event triggered non-periodic and periodic report. In other words, no separate requirements are needed for event triggered periodic reporting.

One more open issue in the measurement reporting delay is whether to use the maximum of the measurement period between the serving and candidate cell for the event triggered measurement report delay. We think it is not reasonable to use the maximum value because the UE should evaluate the event after obtaining measurement result of each cell. Using the maximum value would lead to the UE assessing the event only during the longest measurement intervals, potentially causing delays in measurement reporting.

For example, if the serving cell has a measurement period of 20 ms and the candidate cell has a measurement period of 160 ms, using the maximum value would result in the UE evaluating the event only every 160 ms. This could lead to delays in measurement reporting and potentially leading to late HO and there by resulting in HOF or RLF. Instead, by not using the maximum value, the UE can evaluate the event every 20 ms, ensuring more timely reporting and HO robustness.

- The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the cell which triggers the event

Impact of filtering on measurement period:

In Rel-18, the LTM L1-RSRP measurement period is based on single-shot measurements (i.e., M is set to 1). However, the primary goal of event-triggered measurement reporting is to minimize signalling overhead in the uplink (UL). To prevent false event-triggering reports caused by fast fading, RAN1 was discussing filtering methods for event-triggered measurement reporting. In the last meeting, RAN1 concluded that there is no consensus to specify a network-configurable filter, but it was acknowledged that UE implementation filtering could still be assumed.

Given that UE implementation filtering is assumed by RAN1, it is important for RAN4 to discuss the number of samples needed for the UE implementation filter. Historically, legacy requirements assumed 3 samples when the network did not configure the timeRestrictionForChannelMeasurements. Since L1-RSRP for serving cells was part of the CSI framework, this parameter was applicable for L1-RSRP measurements as well. However, L1-RSRP for LTM does not exactly fall within the CSI framework, which means this parameter was not configured in Rel-18.

Despite this, we recognize the necessity for at least UE implementation filtering to mitigate false reporting. Implementing such a filter would be crucial in ensuring the reliability and accuracy of the measurement reports, thereby reducing the potential for false event triggers due to short-term variations like fast fading.

- Measurement period of the L1-RSRP with filtering should be defined considering the multiple samples.

### Requirements for reporting criteria per measurement category

In the last meeting, a proposal was made to await the outcome of RAN1's discussion regarding the number of events that should be supported by the UE. However, we would like to emphasize that the Event Category (Ecat) for L3 measurements is defined within the scope of RAN4 (in clause 9 of 38.133), and it is unlikely that RAN1 will be addressing this specific aspect. Therefore, it is important to continue the discussion within RAN4 to ensure clarity and alignment on the requirements related to the number of events supported by the UE.

- Ecat for L3 measurements was defined within the scope of RAN4, and it is unlikely that RAN1 will be addressing this aspect

In previous meetings, there was a proposal to either share the capability of L3 events with LTM events or introduce a new capability. However, we believe that coordination between the CU and the DU presents challenges, as L3 measurements are typically configured by the CU, while L1 measurements are configured by the DU. This separation makes it difficult to manage the Ecat capability for LTM L1 events effectively.

Additionally, after consulting with our product implementation team, it appears that the existing Ecat capability is already fully utilized. Different applications may require configuration with distinct events, further complicating the potential for sharing capabilities between L3 and LTM events. Given these considerations, introducing a new capability might be necessary to accommodate the diverse requirements and ensure efficient operation.

- The existing Ecat capability is already fully utilized in product.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

We think RAN4 first needs to discuss and decide on the granularity of the LTM L1 events capability. Legacy Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. Since, LTM CSI config may contain multiple cells consisting of both intra-frequency and inter-frequency measurements, we think sharing a combined Ecat between intra and inter-frequency allows the NW flexibility to configure different cells as per the channel or load conditions.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

RAN2 agreed on introducing the following events for the purpose of event triggering measurement reporting.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

The events are designed to support the event evaluation on the PCell/serving cells and candidate cells. We think RAN4 should consider this when discussing the Ecat capability. Since agreeing on actual number requires further analysis, we suggest the actual number to be FFS.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

### Impact of event triggered to reporting to LTM procedures

Our understanding of the use case of the RAN2 introduced events is as follows:

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

Based on the above analysis we can observe that RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

- RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

TCI state known condition

The network may configure the UE with both L3-based events and L1-based events simultaneously. For example, the network may trigger an LTM cell switch based on Event LTM3 and schedule early TCI state activation and PDCCH order-based RACH based on L3-based events. In this case, the current TCI state known conditions need to be updated if the early TCI state activation is based on an L3 measurement report and the cell switch is based on an event-triggered L1 report. This is because the existing known conditions (at least for TCI state activation) are only based on the periodic L1-RSRP report.

Rel-18 TCI state known conditions and the conditions (clause 8.25 of TS 38.133) under which TCI state activation requirements are applicable (clause 6.3.1 of TS 38.133) are mentioned in the below table.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, the TCI state is still considered as unknown based on the existing known condition definition. The cell switch delay requirements are not applicable in this case, as per the Release 18 agreements. To avoid no requirement in this scenario, TCI state known condition defintion needs to be modified. Potential modification is add a sentence saying TCI state is known if the “UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 not to define different requirements for the event triggered non-periodic and periodic report. In other words, no separate requirements are needed for event triggered periodic reporting.

- The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the cell which triggers the event.

- Measurement period of the L1-RSRP with filtering should be defined considering the multiple samples.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - “The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2420101, WF on NR mobility enhancements Phase 4