3GPP TSG-RAN WG4 Meeting #114bis				             R4-2503938

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Document for:	Discussion

Introduction

In meeting RAN4#114, we made remarkable progress on application requirements and also started the discussion for details of MPR reduction approaches. The tentative agreement was captured as follows[1].

Tentative Agreement - For Topic #1, consider whether the following requirements can be applicable based on the UE extended CBW, and based on that decide the range of UE extended CBW - ACLR - SEM - Spurious emission - without violating the regulation requirements and ITU recommendation - IBE

As we already have agreement on ACLR/SEM and SE, so in this contribution, we bring our perspective on the method of converting inner RB allocation to outer RB allocation and illustrate our views about IBE.

Discussion

Width/RB numbers of extended CBW at each side of UE CBW

Firstly, the purpose of Rel-19 power boosting is applying lower MPR requirement to the UE while guarantee the transmit bandwidth by expand the inner RB allocation region. Fixed extension will lead to situations that UE is configured by lower CBW to support this feature, which is meaningless since UE could have done this to ensure the inner RB allocation without the MPR reduction. So fixed ratio extension is contrary to the original intention of this feature.

And not all scenarios can satisfy 0.5* UE CBW extension so few UE could benefit from fixed extension ratio, and NW can determine the shift value from a global perspective to ensure the co-existence problem.

On the other hand, flexible or fixed extension have the same impact on deriving IBE since fixed extension is also an alterable value that based on UE CBW.

Proposal1: Extended CBW at each side of UE CBW should be flexible.

As a compromised solution, we agree with the proposals in Huawei’s paper[2] that the length of each extended CBW should be one of the CBW in MHz that have been specified in Table 5.3.2-1 in TS 38.101-1 to reuse the map from MHz to RB. Besides, the max size of the extension at each side of UE CBW should be 0.5*CBW to reduce the optional values.

Proposal2: Support that the length of each extended CBW should be one of the CBW in MHz that have been specified in Table 5.3.2-1 in TS 38.101-1.

Proposal3: The max size of the extension at each side of UE CBW should be 0.5*CBW.

Approaches of deriving the new inner RB region with RBs in extended CBW

- Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_high - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting

Generally, we support the option 3 of this part in the last WF[1].

As defined in 38.101-1, the formula of determining RB allocation can be treated as a function that NRB and LCRB are input values and RB allocation region is the output value. Any effort about converting inner to outer will concentrate on this formula eventually since the RB allocation region only depends on it. So if we want to reflect the new inner RB allocation in spec, we can introduce new input value to this formula.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2)) where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x. RBStart,High = NRB – RBStart,Low – LCRB The RB allocation is an Inner RB allocation if the following conditions are met RBStart,Low ≤ RBStart ≤ RBStart,High, and LCRB ≤ ceil(NRB/2) where ceil(x) is the smallest integer greater than or equal to x.

The extended UE CBW actually influence the NRB in the above formula, so the RB shift values on each side should be reflected in any part of the formula contains NRB. For two side extension, the UE will have two values: RBshift_low and RBshift_high, which denote the numbers of RB extension on the left and right side of UE CBW. And the new RB number can be expressed as:

NRB_new = RBshift_low + NRB + RBshift_high

The details of formula of determining RB allocation may be different when the coordinate system origin of RB_start is different. The coordinate system origin of RB_start can base on extended NRB_new or original NRB so here we provide two ways of determining RB allocation.

- NRB based RB_start coordinate system origin

The NRB based RB_start coordinate system origin can be expressed in Fig.1.

Fig.1. NRB based RB_start coordinate system origin

The formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

The max() and min() at the outermost layer in RBStart,Low_new and RBStart,Low_new are to make sure the new inner region is the intersection of the inner MPR region of the extended UE CBW and the original MPR region based on the UE CBW.

Proposal4: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

- RBshift_low, RBshift_high ≤ NRB/2

- NRB_new based coordinate system origin

The NRB_bew based RB_start coordinate system origin can be expressed in Fig.1.

Fig.2. NRB_new based RB_start coordinate system origin

The inner RB allocation region that we expect is the intersection of red dotted line triangle and yellow triangle. So the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Proposal5: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Besides, here we offer a feasible solution to rewrite the formula and we also do not exclude other options and we are open to discuss the details.

Signalling

For the UE capability, UE should directly indicate to the NW whether it support the mechanism that CBW extension based MPR reduction. As for the applied scenarios and whether there is enough space for extension, and the CBW extended value, those questions are all up to NW and UE is not need to distinguish that.

Proposal6: UE should directly indicate to the NW whether it support the mechanism that CBW extension based MPR reduction.

For the network configuration, we need at least two IEs to convey the extension on the lower side and the higher side of UE CBW. The extension value should be selectable and indicate the bandwidth from Table 5.3.2-1 in TS 38.101-1.

Proposal7: Set two IEs to convey the extension on the lower side and the higher side of UE CBW.

Conclusion

Proposal1: Extended CBW at each side of UE CBW should be flexible.

Proposal2: Support that the length of each extended CBW should be one of the CBW in MHz that have been specified in Table 5.3.2-1 in TS 38.101-1.

Proposal3: The max size of the extension at each side of UE CBW should be 0.5*CBW.

Proposal4: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

- RBshift_low, RBshift_high ≤ NRB/2

Proposal5: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Proposal6: UE should directly indicate to the NW whether it support the mechanism that CBW extension based MPR reduction.

Proposal7: Set two IEs to convey the extension on the lower side and the higher side of UE CBW.

References

- R4-2503013, WF on power domain enhancement, RAN4#114

- R4-2501832, On power domain enhancements for single carrier, Huawei, HiSilicon