3GPP TSG-RAN WG4 Meeting #114bis				             R4-2503939

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	China Telecom

Title: 	Further study on SRS IL imbalance

Agenda Item:	7.1.1.3.4

Document for:	Discussion

Introduction

In RAN4#114[1], we drew a candidate solution that UE self-compensation was adopted and FFS on assistance information.

Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion. Candidate solution framework: - Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on RAN4 specification impacts. - No RAN1 specification impact. - FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others

So in this paper we reiterate our view and also try to provide some compromised options.

Discussion

- 2.1 General classification

Firstly, SRS IL only influences the equations on PCMAX_L,f,c in TS 38.101-1 and we can fix if in RAN4 specification without RAN1 impact. So we support the UE self-compensation of SRS IL with changes to RAN4 specification and we are open to discuss the details.

Proposal1: Support UE self-compensation of SRS IL with changes to RAN4 specification.

Based on Ericsson’s statistics during RAN4#114 email discussion we can see most companies are OK with introducing low-complexity reporting mechanism.

Reporting issue must have impact to RAN2 specification but we can reuse some reporting package to minimize the impact. So we believe we can find an as simple as possible way to introduce indication or reporting mechanism.

Proposal2: Support introducing simple indication or reporting mechanism for assistance to the network on SRS IL compensation.

Besides, considering the limited time of Rel-19, we should reach a clear agreement on SRS IL imbalance in this meeting.

Observation1: Considering the limited time of Rel-19, we should reach a clear agreement on SRS IL imbalance in this meeting.

- 2.2 Reporting trigger condition

The output power of SRS transmission is the minimum value of PCMAX,f,c and the actually calculated SRS power (denoted by PSRS_cal) as defined in 38.213.

While the insertion loss, i.e. the ∆TRxSRS can only influence the PCMAX,f,c part so the self-compensation should not be triggered unless the following equation is satisfied:

PCMAX,f,c - PSRS_cal ≤ ∆TRxSRS

Where PCMAX,f,c is not considering IL. In another word, the compensation will execute on PCMAX,f,c and only when the gap between the original PCMAX,f,c and PSRS_cal is small enough that the behavior is needed. self-compensation should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal3: The self-compensation should be triggered only if the PCMAX,f,c with IL influence is smaller than the necessary SRS power.

- 2.3 Potential solutions

In this part we provide some workable assistance information reporting solutions that impact the spec as less as possible, the reporting is on condition that UE do self-compensation as much as it can.

- Enhanced PHR on SRS

The type 3 PHR conveys the difference between PCMAX,f,c and estimated power for SRS transmission. So through monitoring the type 3 PHR the network can tell if UE is doing self-compensation and if it can compensate all the IL or not. Since PHR has the field for PCMAX,f,c and the elevation of PCMAX,f,c value means compensation is undertaking. On the other hand, if the value of PH field is not zero, the UE has enough power to compensate and the network don’t have to intervene.

There are already been type 3 PHR definition in TS 38.321 so the impact of specification would be quiet slight that we only need to care about the reporting granularity and trigger condition.

At last but not the least, type 3 PHR was only introduced in multiple entry PHR, i.e. the CA/EN-DC scenarios and single entry PHR can only report type 1 PHR as specified in TS 38.321 as below. So if this approach is adopted, we need to inform RAN2 to fix the defect.

Single entry PHR MAC CE

Multiple entry PHR MAC CE

Proposal4: Type 3 PHR for SRS transmission is a reasonable way to convey IL compensation information and does little impact on RAN2 specification.

- Direct IL imbalance value report

As we mentioned many times, SRS insertion loss value reporting is more precise and intuitive and can still provide numerical information for NW doing post-compensation even the UE runs out of all the power headroom. But this method need new allocated field and signaling that has much impact.

Observation2: Direct IL imbalance value report is precise and intuitive but has much impact.

- Necessary capability report and signaling

After all, self-compensation should be an optional feature in Rel-19, so UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

Proposal5: UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

Conclusion

Observation1: Considering the limited time of Rel-19, we should reach a clear agreement on SRS IL imbalance in this meeting.

Observation2: Direct IL imbalance value report is precise and intuitive but has much impact.

Proposal1: Support UE self-compensation of SRS IL with changes to RAN4 specification.

Proposal2: Support introducing simple indication or reporting mechanism for assistance to the network on SRS IL compensation.

Proposal3: The self-compensation should be triggered only if the PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal4: Type 3 PHR for SRS transmission is a reasonable way to convey IL compensation information and does little impact on RAN2 specification.

Proposal5: UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

References

- R4-2503014, WF on for 6Rx UE requirements, RAN4#114, AT&T