# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503943

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			NTN Ku bands SAN RF

Agenda item:		7.11.5

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised [1]) including support for 15 kHz SCS.

In last RAN4#114 meeting, RAN4 agreed on the frequency range definition of the NTN Ku-bands in both FR1-NTN and FR2-NTN and their corresponding system parameters ([2]).

In this contribution, we are discussing how to specify SAN RF requirements in both FR1-NTN and FR2-NTN in a consistent way.

# Discussion

## SAN OTA requirements, FR1-NTN and FR2-NTN

In last RAN4#114 meeting, the agreed WF ([2]) captures that SAN type 1-O and type 2-O shall be specified for SAN operating in the NTN KU-bands, while type 1-H should not be considered to save some effort.

Moreover, companies were also encouraged to further check how to best aligned requirements specified in FR1-NTN and FR2-NTN for SAN operating in the NTN Ku-bands:

During RAN4#114 discussion, it was suggested to specify FR1-NTN RF requirements for the NTN Ku-bands based on the 2-O requirements.

This approach would ensure that the FR1-NTN requirements and the FR2-NTN requirements are fully equivalent for the SAN operating in NTN Ku-bands. But, to not create any confusion with the already specified SAN type 1-O for NTN bands below 5GHz, this would require specifying a new SAN type. This new type should only be used when an NTN band shall be introduced in FR1-NTN and FR2-NTN, which is currently only the case for NTN Ku-bands n247 and n248.

Proposal1: Introduce a new SAN type (e.g. type 1-A-O) in FR1-NTN based on SAN type 2-O RF requirements for the NTN Ku-bands n247 and n248.

Our companion draft CR ([3]) captures what changes should be done to introduce this new BS type for the NTN Ku-bands. As it could be observed, those changes are relatively limited and would ensure the consistency of RF requirements for the NTN Ku-bands. This draft CR is proposed to also trigger further discussion on the missing parts of those requirements, if RAN4 agrees to introduce this new SAN type.

For convenience, we used “type 1-A-O) in this draft CR but RAN4 should further discuss and decide how to best name this new BS type.

# Conclusion

In this contribution, we analyzed how to specify SAN RF requirements in both FR1-NTN and FR2-NTN in a consistent way. We made the following observations and proposals:

Proposal1: Introduce a new SAN type (e.g. type 1-A-O) in FR1-NTN based on SAN type 2-O RF requirements for the NTN Ku-bands n247 and n248.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2503945, Draft CR to TS 38.108 - Introduction of a new BS type in FR1-NTN for the NTN Ku-bands.