# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503945

# Wuhan, Hubei, China, 07th – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.108 – Introduction of a new BS type in FR1-NTN for the NTN Ku-bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The RF requirements shall be consistent for NTN Ku-bands specified in FR1-NTN and FR2-NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To adress this, a new SAN type 1-A-O is specified based on SAN type 2-O requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements might not be consistent for NTN Ku-bands specified in FR1-NTN and FR2-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.3.2, 4.6, 9.2.1, 9.2.3, 9.3.1, 9.3.3, 9.4.3.4, 9.6.1.3, 9.6.2.3, 9.7.2.2, 9.7.3.4, 9.7.4.3, 9.7.5.3, 10.1, 10.3.4, 10.5.1.4, 10.6.4, 10.9.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.181 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note that the any other naming for this new BS type could be discussed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [2] used for the formulation of unwanted emission requirements for FR1-NTN.

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array.

NOTE:	For certain antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam.

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction.

beam peak direction: direction where the maximum EIRP is found.

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse.

Channel edge: lowest or highest frequency of the NR carrier, separated by the SAN channel bandwidth.

directional requirement: requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver.

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device.

NOTE:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA.

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

feeder link: Wireless link between satellite-gateway and satellite.

Geostationary Earth Orbit: Circular orbit at 35,786 km above the Earth's equator and following the direction of the Earth's rotation. An object in such an orbit has an orbital period equal to the Earth's rotational period and thus appears motionless, at a fixed position in the sky, to ground observers.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Highest Carrier: The carrier with the highest carrier frequency transmitted/received in a specified frequency band.

Lowest Carrier:	The carrier with the lowest carrier frequency transmitted/received in a specified frequency band.

maximum carrier output power: mean power level measured per carrier at the indicated interface, during the transmitter ON period in a specified reference condition.

maximum carrier TRP output power: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,c,TRP).

maximum total output power: mean power level measured within the operating band at the indicated interface, during the transmitter ON period in a specified reference condition.

maximum total TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated total TRP output power (Prated,t,TRP).

measurement bandwidth: RF bandwidth in which an emission level is specified.

minSENS: the lowest declared EIS value for the OSDD's declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS.

minimum elevation angle: Minimum angle under which the satellite can be seen by a UE.

necessary bandwidth: The width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or SAN.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

NOTE:	The operating band(s) for a SAN is declared by the manufacturer according to the designations in tables 5.2-1 and 5.2-2.

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met.

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range.

NOTE:     The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction assuming that for any AoA, the receiver gain is optimized for that AoA.

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beamwidth.

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with SAN channel bandwidth), and related directions over which the EIS applies.

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply.

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier.

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the SAN is declared to radiate at the associated beam peak direction during the transmitter ON period.

rated carrier output power: mean power level associated with a particular carrier the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition.

rated carrier TRP output power: mean power level declared by the manufacturer per carrier, for SAN operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period.

rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition.

rated total TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period.

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the OTA peak directions set.

receiver target: AoA in which reception is performed by SAN types 1-H or SAN type 1-O.

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD.

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA.

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

requirement set:	one of the NR SAN requirement's set as defined for SAN type 1-H, SAN type 1-O.

SAN channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink.

NOTE 1:	The SAN channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the SAN to transmit to and/or receive from one or more satellite UE bandwidth parts that are smaller than or equal to the SAN transmission bandwidth configuration, in any part of the SAN transmission bandwidth configuration.

SAN RF Bandwidth: RF bandwidth in which a SAN transmits and/or receives single or multiple carrier(s) within a supported operating band.

NOTE:	In single carrier operation, the SAN RF Bandwidth is equal to the SAN channel bandwidth.

SAN RF Bandwidth edge: frequency of one of the edges of the SAN RF Bandwidth.

SAN transmission bandwidth configuration: set of resource blocks located within the SAN channel bandwidth which may be used for transmitting or receiving by the SAN.

SAN type 1-H: 	Satellite Access Node operating at FR1-NTN with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

SAN type 1-O: 	Satellite Access Node operating at FR1-NTN with a requirement set consisting only of OTA requirements defined at the RIB.

SAN type 1-A-O: 	Satellite Access Node operating at FR1-NTN in NTN bands n247 and n248 with a requirement set consisting only of OTA requirements defined at the RIB.

SAN total assigned bandwidth: Bandwidth of the total assigned band (frequencies range) as defined in SM.1541-6 [9].

SAN transponder bandwidth: Total bandwidth of the carrier(s) in operation by one SAN transponder.

NOTE: When the SAN transponder operates one carrier only, the SAN transponder bandwidth is equal to the SAN channel bandwidth of this carrier.

SAN transponder: part of the SAN permitting to receive, channelize and transmit signals within an allocated bandwidth.

satellite: A space-borne vehicle embarking a transparent  payload, or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) or Geostationary Earth Orbit (GEO).

Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN satellite capable UE, and connected via the NG interface to the 5GC. It encompasses a transparent  payload on board a NTN platform, with satellite-gateway and gNB functions.

satellite-gateway: An earth station or gateway is located at the surface of Earth, and providing sufficient RF power and RF sensitivity for accessing to the satellite.

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific SAN direction setting.

TAB connector: transceiver array boundary connector.

total radiated power: is the total power radiated by the antenna.

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna.

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a satellite UE or SAN, measured in resource block units.

<End of the change>

<Start of the change>

### 4.3.2	SAN type 1-O, SAN type 1-A-O and SAN type 2-O

For SAN type 1-O, SAN type 1-A-O and SAN type 2-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Figure 4.3.2-1: Radiated reference points for SAN type 1-O, SAN type 1-A-O and SAN type 2-O

<End of the change>

<Start of the change>

## 4.6	Applicability of minimum requirements

In table 4.6-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O | SAN type 1-A-O | SAN type 2-O |
| SAN output power | 6.2 |  |  |  |
| Output power dynamics | 6.3 |  |  |  |
| Transmit ON/OFF power | NA |  |  |  |
| Frequency error | 6.5.1 |  |  |  |
| Modulation quality | 6.5.2 |  |  |  |
| Time alignment error | NA |  |  |  |
| Occupied bandwidth | 6.6.2 |  |  |  |
| ACLR | 6.6.3 |  |  |  |
| Out-of-band emissions | 6.6.4 |  |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  |  |
| Transmitter intermodulation | NA | NA | NA | NA |
| Reference sensitivity level | 7.2 |  |  |  |
| Dynamic range | 7.3 |  |  |  |
| ACS | 7.4.1 |  |  |  |
| In-band blocking | NA |  |  |  |
| Out-of-band blocking | 7.5 |  |  |  |
| Receiver spurious emissions | NA |  |  |  |
| Receiver intermodulation | NA |  |  |  |
| In-channel selectivity | 7.8 |  |  |  |
| Performance requirements | 8 |  |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 9.2 |
| OTA SAN output power |  | 9.3 | 9.3 | 9.3 |
| OTA output power dynamics |  | 9.4 | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  | NA | NA | NA |
| OTA frequency error |  | 9.6.1 | 9.6.1 | 9.6.1 |
| OTA modulation quality |  | 9.6.2 | 9.6.2 | 9.6.2 |
| OTA time alignment error |  | NA | NA | NA |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 | 9.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | 9.7.3 |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  | NA | NA | NA |
| OTA sensitivity | 10.2 | 10.2 | NA | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 | 10.3 |
| OTA dynamic range |  | 10.4 | NA | NA |
| OTA ACS |  | 10.5.1 | 10.5.1 | 10.5.1 |
| OTA in-band blocking |  | NA | NA | NA |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | 10.6 |
| OTA receiver spurious emission |  | NA | NA | NA |
| OTA receiver intermodulation |  | NA | NA | NA |
| OTA in-channel selectivity |  | 10.9 | 10.9 | 10.9 |
| Radiated performance requirements |  | 11 | 11 | 11 |

NOTE:	Co-location requirements are not applicable to SAN.

<End of the change>

<Start of the change>

### 9.2.1	General

SAN type 1-H, SAN type 1-O, SAN type 1-A-O and SAN type 2-O are declared to support one or more beams, as per manufacturer's declarations specified in TS 38.181 [3]. Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each beam, the requirement is based on declaration of a beam identity, reference beam direction pair, beamwidth, rated beam EIRP, OTA peak directions set, the beam direction pairs at the maximum steering directions and their associated rated beam EIRP and beamwidth(s).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the SAN is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in TS 38.181 [3].

NOTE 1:	OTA peak directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

<End of the change>

<Start of the change>

### 9.2.3	Minimum requirement for SAN type 1-A-O and SAN type 2-O

<End of the change>

<Start of the change>

### 9.3.1	General

OTA SAN output power is declared as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB. TRP does not change with beamforming settings as long as the beam peak direction is within the OTA peak directions set. Thus the TRP accuracy requirement must be met for any beamforming setting for which the beam peak direction is within the OTA peak directions set.

The SAN rated carrier TRP output power for SAN type 1-O, SAN type 1-A-O and SAN type 2-O shall be based on manufacturer declaration.

Despite the general requirements for the SAN output power described in clause 9.3.2, additional regional requirements might be applicable.

<End of the change>

<Start of the change>

### 9.3.3	Minimum requirement for SAN type 1-A-O and SAN type 2-O

In normal conditions, the SAN type 1-A-O and SAN type 2-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.181, annex B [3].

<End of the change>

<Start of the change>

#### 9.4.3.4	Minimum requirement for SAN type 1-A-O

OTA total power dynamic range minimum requirement for SAN type 1-A-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.4-1 in FR1-NTN.

Table 9.4.3.4-1: Minimum requirement for SAN type 1-A-O total power dynamic range in FR1-NTN

| SCS | OTA total power dynamic range (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| (kHz) |  |  |  |  |
| 15 | TBD | TBD | TBD | TBD |
| 30 | TBD | TBD | TBD | TBD |

<End of the change>

<Start of the change>

#### 9.6.1.3	Minimum requirement for SAN type 1-A-O and SAN type 2-O

For SAN type 1-A-O and SAN type 2-O, the modulated carrier frequency of each carrier configured by the SAN shall be accurate to within 0.05 ppm observed over 1 ms.

<End of the change>

<Start of the change>

#### 9.6.2.3	Minimum requirement for SAN type 1-A-O and SAN type 2-O

##### 9.6.2.3.1	EVM frame structure for measurement

For SAN type 1-A-O and SAN type 2-O, the EVM levels of each carrier for different modulation schemes on PDSCH outlined in table 9.6.2.3.1-1 shall be met.

Table 9.6.2.3-1: EVM requirements for SAN type 1-A-O and SAN type 2-O carrier

|  | Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- | --- |
|  | QPSK | 17.5 |
|  | 16QAM | 12.5 |
|  | 64QAM | 8 |

<End of the change>

<Start of the change>

#### 9.7.2.2	Minimum requirement for SAN type 1-O, SAN type 1-A-O and SAN type 2-O

The OTA occupied bandwidth for each carrier shall be less than the SAN channel bandwidth.

<End of the change>

<Start of the change>

#### 9.7.3.4	Minimum requirement for SAN type 1-A-O

For BS type 1-A-O, the ACLR limit specified in tables 9.7.3.4-1 for SAN GEO class and 9.7.3.4-2 for SAN LEO class shall apply.

For a RIB operating in multi-carrier, the ACLR requirements in clause 9.7.3.3 shall apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in tables 9.7.3.3-1 and 9.7.3.3-2.

Table 9.7.3.4-1: SAN type 1-A-O ACLR limit for SAN GEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 10, 15, 25, 35, 50, 70, 100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | TBD |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

Table 9.7.3.4-2: SAN type 1-A-O ACLR limit for SAN LEO class

| SAN channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | SAN adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 10, 15, 25, 35, 50, 70, 100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | TBD |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

<End of the change>

<Start of the change>

#### 9.7.4.3	Minimum requirement for SAN type 1-A-O

Out-of-band emissions in FR1-NTN are limited by OTA out-of-band emission limits. Unless otherwise stated, the out-of-band emission limits in FR1-NTN are defined from channel edge up to frequencies separated from the channel edge by 200% of the necessary bandwidth. The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier for the frequency ranges defined in clause 6.6.4.1.

The OTA out-of-band emissions requirement for SAN type 1-A-O shall not exceed each applicable limit in clause 6.6.4.2.

<End of the change>

<Start of the change>

#### 9.7.5.3	Minimum requirement for SAN type 1-A-O  and SAN type 2-O

<End of the change>

<Start of the change>

## 10.1	General

Radiated receiver characteristics are specified at RIB for SAN type 1-H or SAN type 1-O or SAN type 1-A-Oor SAN type 2-O, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for the radiated receiver characteristics requirements in clause 10:

-	Requirements shall be met for any transmitter setting.

-	The requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements defined for the radiated receiver characteristics do not assume HARQ retransmissions.

-	When SAN is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS and blocking characteristics, the negative offsets of the interfering signal apply relative to the lower SAN RF Bandwidth edge, and the positive offsets of the interfering signal apply relative to the upper SAN RF Bandwidth edge.

-	Each requirement shall be met over the RoAoA specified.

NOTE 1:	In normal operating condition the SAN in FDD operation is configured to transmit and receive at the same time.

For FR1-NTN requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) dB for the reference direction

and

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) dB for all other directions

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

For FR2-NTN requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔFR2_REFSENS = -3 dB for the reference direction

and

ΔFR2_REFSENS = 0 dB for all other directions

<End of the change>

<Start of the change>

### 10.3.4	Minimum requirement for SAN type 1-A-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_TBD, which is based on a reference measurement channel with TBD MHz SAN channel bandwidth. EISREFSENS_TBD itself is not a requirement and although it is based on a reference measurement channel with TBD MHz SAN channel bandwidth it does not imply that SAN has to support TBD MHz SAN channel bandwidth.

For GEO class SAN, EISREFSENS_TBD is an integer value in the range [TBD] to [TBD] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_TBD is an integer value in the range [TBD] to [TBD] dBm. The specific value is declared by the vendor.

Table 10.3.4-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| TBD | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD |

<End of the change>

<Start of the change>

#### 10.5.1.4	Minimum requirement for SAN type 1-A-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1-NTN, the OTA wanted and the interfering signal are specified in table 10.5.1.4-1 and table 10.5.1.4-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.4-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 10.5.1.4-1: OTA ACS requirement for SAN type 1-A-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 10, 15, 25, 35, 50, 70, 100 | EISREFSENS + 6 dB (Note 1) | SAN LEO class: TBD SAN GEO class: TBD |
| NOTE 1: EISREFSENS is given in clause 10.3.4 |  |  |

Table 10.5.1.4-2: OTA ACS interferer frequency offset for SAN type 1-A-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 10 | TBD |  |
| 15 | TBD | TBD |
| 25 | TBD |  |
| 35 | TBD |  |
| 50 | TBD |  |
| 70 | TBD |  |
| 100 | TBD |  |

<End of the change>

<Start of the change>

### 10.6.4	Minimum requirement for SAN type 1-A-O

#### 10.6.4.1	General minimum requirement

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted signal applies to each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

For SAN type 1-A-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low –ΔfOOB  and from FUL,high +ΔfOOB  up to 2nd harmonic of the upper frequency edge of the operating band. The ΔfOOB for SAN type 1-A-O is defined in table 10.6.4.1-2.

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 10.6.4.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.3 for each SAN channel bandwidth and further specified in annex A.1.

Table 10.6.4.1-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to FUL,low –ΔfOOB | EISREFSENS + 6 dB | 0.0029 | CW |
| FUL,high +ΔfOOB to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.0029 | CW |

Table 10.6.4.1-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-A-O | FUL,high – FUL,low ≤ 4000 MHz | 1500 |

<End of the change>

<Start of the change>

### 10.9.4	Minimum requirement for SAN type 1-A-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For SAN type 1-A-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.9.4-1. The characteristics of the interfering signal is further specified in annex D.

Table 10.9.4-1: OTA in-channel selectivity requirement for SAN type 1-A-O

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| TBD | TBD | TBD | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD | TBD | TBD |
| TBD | TBD | TBD | TBD | TBD | TBD |

<End of the change>