3GPP TSG-RAN4 Meeting # 114bis	R4-2503962

Wuhan, China, Apr.7th – 11th, 2025

# Title: 	View on low-low band CA via switching

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.7.2

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, a new WID was approved to introduce a called switching method to enable low-low band CA due to there exists some challenges to implement the low-low band CA band combiantion due to poor antenna implementation when sharing one common antenna, or cost/size issue when separated antennas are used.

The objectives are shown below:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In the approved WF [2], some agreements were included. In this contribution, we gives some further discussions on low-low band CA via switching (i.e. downlink carrier switching) .

# 2	Discussion

In terms of the objectives, the only switching cases are:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

For the scenarios, it was agreed that:

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

It means only the FDD-SDL low-low band inter-band CA scenario, which is CA_n5A-n29A, is considered in the current WID, the other band combinations will be handled via the basket WID approach.

The illustration for the above switching cases can be found in the motivation paper, which is extracted in the following:

Fig 1.Illustration for the switching cases between FDD and SDL band

As we discussed in [3], for low-low band CA band combination, the implementations of shared antenna and separated antennas are mentioned in TR38.872[2]. In our understanding, the purpose of this WID is to generate the carrier switching method in the case of the shared antenna for the switched-from and switched-to carriers. In other words, there is no need to consider the downlink carrier switching for separated antennas implementations since the Rx chain/antennas are totally separated for the two bands low-low CA, or the carrier switching period for separated antennas implementations is nearly 0us.

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

For downlink carrier switching, similarly we cannot restrict the implementation for UE supporting low-low CA band combination, but UE needs to report the downlink carrier switching capability to NW. This is also mentioned in the WF, which is:

Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination

The related capabilities for Tx switching are extracted from TS38.306 and shown below:

| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. ..... | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |

We think the same approach as Tx switching can be adopted, which is per band combination capability is defined.

Proposal 1. The downlink switching delay capability can be per band combination.

For the switching periods, there are some basic values agreed in the WF with the other value open:

- - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings

In our understanding, different switching delay time or switching period for the carriers switching happen in main Tx/Rx antenna port and diversity Rx antenna ports due to different implementation between main Tx/Rx antenna port and diversity Rx antenna ports. 35us and 140us values come from the Tx switching period. Besides that, some companies think the values less than 35us is feasible when there is only antenna to be switched. In other words, without other RF components like LO/PA involved into switching, the switching time is foreseeably short.

Proposal 2. Less than 35 us such as 13us could be used as candidate value for switching period.

For the time mask, there are some options listed in the WF:

Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded

In terms of the justification of the WID, an enhancement of the 3GPP specification is needed to enable the following solution:

-	Device needs to support switching: when the SCell operation is triggered, UE needs to switch to the SCell, and during the operation period there is no simultaneous Tx/Rx between the PCell and the SCell

-	Device switches back to the PCell after the SCell operation is finished

Therefore, it seems the option 2 is somehow included in the option 1 since we think the switching period cannot be located in both carriers (i.e. switching-from carrier and switching-two carrier) at the same time, and either FDD band or SDL band can be seen as switching-from carrier. In other words, switching period shall be only in one carrier in a certain TTI. We think option 1 is enough for a high level agreement on the switching time mask.

As there is no simultaneous Tx/Rx between the PCell and the SCell, as illustrated in Fig 1., Rx is only happened in either switch-from carrier or switch-to carrier. In other words, only switchedDL option (similar to switchedUL for Tx switching) can be considered for low-low CA downlink carrier switching in this WID. For the dualDL option, it could be considered in later release associated with the new band combination.

Proposal 3. Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL).

Proposal 4. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

As a part of the switching time mask, the switching period location should be discussed.

Issue 5: Switching period location - Discuss the switching period location based on the following options: - Option 1: Switching period location is entirely up to RAN1 design - Option 2: Switching period location is defined relative to the slot boundary - Other options are not precluded

Some companies think the locations should not be located in the first three symbols of any downlink slot considering the PDCCH impact. In our understanding, the similar situation is also existed in Tx switching. For Tx switching, the location can be in the boundary of a slot of either switching-from carrier or switching to carrier. For FDD-FDD combination, when UL is switched, then no UL is allowed in the switching period, in this case DL is also not allowed which cause PDCCH impact either. Therefore, we think the similar Tx switching period location can be initially applied for Rx switching. Nevertheless, we think the switching period location discussion is related to the switching pattern which will be discussed in RAN4 1 from April meeting.

Proposal 5. Switching period location discussion should wait for RAN1 consensus of switching pattern design.

In current RAN4 specification, unlike the other non-SDL single band, there are no RF requirements defined for SDL single band, instead they are defined associated with inter-band CA. For example, the reference sensitivity requirements for SDL shall be verified by inter-band CA combinations with SDL band, which are supported by UE. In addition to REFSEN requirements, MSD requirements and delta Tib/Rib requirements are defined for some inter-band CA band combination with SDL band.

In the WID, there is only one example band combination, the other band combination to be handled via the basket work item approach. Although it is not clear how to pursue it in basket WID approach at this stage, it seems RAN4 will introduce tables to specify which band combination can use downlink carrier switching method. If so, it will cause specification cumbersome if the downlink carrier switching method is defined as band combination manner. Actually basket WID approach to introduce additional band combination was also discussed in Rel-18 3/4 bands Tx switching discussion. But in the end, no basket WID approach is introduced for 3/4 bands Tx switching since it will be subjected to basket WID procedure/guidance, which needs at least 3 months delay when operator request it, and also it will increase RAN4 workload. Instead it depends on whether the switching capability is present for a certain band combination.

In addition to that, it seems the work for basket WID approach for band combination supporting downlink carrier switching would overlap/conflict with the normal PC3/HPUE basket WID since the MSD work may be needed for a band combination introduced via carrier switching basket WID, but in general, such MSD work is discussed in the normal PC3/HPUE basket WID.

Moreover, when downlink carrier switching happens, whether the UL interruption is allowed should be defined per band combination.

Therefore, we think RAN4 should try to define generic downlink carrier switching requirements for all the band combinations, rather than defining band combination specified requirements.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 6: RAN4 should try to define generic downlink carrier switching requirements, rather than band combination specified requirements.

# 3 Conclusion

In this contribution, we give some further discussion on the downlink carrier switching to enable low-low band CA band combination.

The observations, proposals and conclusions are:

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 1. The downlink switching delay capability can be per band combination.

Proposal 2. Less than 35 us such as 13us could be used as candidate value for switching period.

Proposal 3. Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL).

Proposal 4. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

Proposal 5. Switching period location discussion should wait for RAN1 consensus of switching pattern design.

Proposal 6: RAN4 should try to define generic downlink carrier switching requirements, rather than band combination specified requirements.

# 4	Reference

[1] RP-243317,New WID on low band carrier aggregation via switching, Apple, and etc.

[2] R4-2503016, WF on low-low band switching, Apple

[3] R4-2500940, Discussion on low-low band CA via switching, ZTE Corporation, Sanechips,