3GPP TSG-RAN4 Meeting # 114bis	R4-2503974

Wuhan, China, Apr.7th – 11th, 2025

# Source:	ZTE Corporation

# Title:	TP for TR38.749 UE RF requirements (Clause 8)

# Agenda Item:	6.17.2

# Document for:	Approval

- Introduction

In this contribution, a TP to TR38.749[1] is provided for UE RF requirements(Clause 8).

- Reference

- [1] TR 38.749 v0.1.0 Protection of Earth Exploration Satellite Service (EESS) Huawei, HiSilicon.

- Text Proposal

----- Start of TP -----

- 8	UE RF requirements

In terms of the EU Decision 2020/590 [4], the additional UE spurious emission requirement for EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258 are defined in table 8.1.

Table 8.1: Additional emission requirements for only EESS requirement in Rel-19

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | -5dBm | - MHz |

The NS values are agreed as follow：

- The legacy NS_202 and CA_NS_202 are updated by just replacing +1 dBm/200 MHz with -5dBm/200MHz.

- The new NS_205 and  CA_NS_205 are defined for both band n257 and band n258,

For a UE to report newly supported NS value(s) for a legacy band to the network through the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters.

- - modifiedMPRbehaviour bit to differentiate between legacy NS_202/CA_NS_202 and new NS_202/CA_NS_202 requirements, and it is mandatory to report modifiedMPRbehaviour bit from Rel-15 for new UEs.

- - NS_205 and CA_NS_205 is mandatory and the corresponding modifiedMPRbehavior bit shall be indicated for new UE(s) from Rel-15.

The description of modifiedMPRbehavior indication for NS_203 is captured in Table 8-2.

Table 8-2. modifiedMPRbehavior indication for NS_202/CA_NS_202 and NS_205/CA_NS_205

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | ... | ... |
|  | 1 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | ... | ... |
|  | 1 | ... | ... |
|  | 2 | .... | ... |
|  | 3 | - NS_202 as defined in clause 6.2.3.3 or both NS_202 and CA_NS_202 as defined in clause 6.2A.3.3 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.2.3.6 or both NS_205 and CA_NS_205 as defined in clause 6.2A.3.6 of 38.101-2 v19.0.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |

With the modifiedMPR-Behavior indication, the A-MPR requirements for NS_202/CA_NS_202 to comply with the above additional emission requirement are defined in Table 8.3.

Table 8.3. Updated A-MPR for NS_202 and CA_NS_202 with the modifiedMPR-Behavior indication

| NS values | Single band/intra-band CA | Updated A-MPR for PC1 (dB) | Updated A-MPR for PC2~PC7 (dB) |
| --- | --- | --- | --- |
| NS_202 | Single band | 11.0 | 2.0 |
| CA_NS_202 | Contiguous UL CA with contiguous allocation | 11.0 | [5.0] if offset frequency < BWintraCA, 2.0 otherwise. |
|  | Contiguous UL CA with non-contiguous allocation or NC UL CA | [13.0] if offset frequency < BWintraCA, 11.0 otherwise | [8.0] if offset frequency < BWintraCA, 2.0 otherwise. |

The A-MPR requirements for NS_205/CA_NS_205 to comply with the above additional emission requirement are defined in Table 8.4.

Table 8.4. A-MPR requirements for NS_205 and CA_NS_205

| NS values | Single band/intra-band CA | A-MPR for PC1 (dB) | A-MPR for PC2~PC7 (dB) |
| --- | --- | --- | --- |
| NS_205 | Single band | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 2.0 if offset frequency < BWchannel, 0.0 otherwise |
| CA_NS_205 | Contiguous UL CA with single contiguous RB allocation | [9.0] if offset frequency < BWintraCA, 6.0 otherwise. | [5.0] if offset frequency < BWintraCA, 0.0 otherwise. |
|  | Contiguous UL CA with non-contiguous allocation or NC UL CA | [13.0] if offset frequency < BWintraCA, 6.0 otherwise. | [8.0] if offset frequency < BWintraCA, 0.0 otherwise. |

NOTE 1: For NS_202 and NS_205, the Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the channel bandwidth.

NOTE 2: For CA_NS_202 and CA_NS_205, the Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the lowest CC among the configured UL CA.

----- End of TP -----