3GPP TSG-RAN4 Meeting # 114bis	R4-2503975

Wuhan, China, Apr.7th – 11th, 2025

# Source:	ZTE Corporation

# Title:	TP for TR38.719-02-01_CA_n40A-n41C

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

The TP to TR to introduce CA_n40A-n41C_BCS4 and 5 can be found in [1].

In current specification, there are no MSD requirements caused by UL_n41C are defined since non-simultaneous Rx/Tx operation is considered. However, n40 and n41 can support simultaneous Rx/Tx operation, in this case the MSD requirements caused by UL_n41C are studied in the TP[1], in which IMD4 and IMD6 caused by n41C would fall into n40 DL Rx.

Due to company think the IMD MSD test point for 1 band 2UL CC should consider practical deployment aspect, which means the traditional IMD MSD test point  for 1 band 2UL CC, a.k.a 1RB+1RB allocation, can not reflect the practical deployment, instead full RB+full RB allocation should be considered.

After a long discussion in RAN4, companies achieved the agreements to define IMD MSD requirements for both 1RB+1RB allocation and full RB+full RB allocation. More details can be found in the WF[2].

Due to the MSD requirements have already done, this contribution provides a text proposal to introduce CA_n40A-n41C in TR38.719-02-01[3].

- Reference

- [1] R4-2408380, R4-2408380 TP for TR38.718-02-01_CA_n40A-n41C, ZTE Corporation, Skyworks

- [2] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

- [3] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- Text Proposal

----- Start of TP -----

  - 5.x		CA_n40-n41

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n40-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n40-n41 | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

      - 5.x.1.2	Channel bandwidths per operating band for CA

Same configurations in the specification.

      - 5.x.1.3	UE Co-existence studies

For the UL/DL harmonic, harmonic mixing and cross band isolation caused by single band n40 or single band n41, it was already analyzed in the fallback configurations of DL CA_n40A-n41A with UL single carrier or UL CA_n40A-n41A, and it can be applied for DL CA_n40A-n41C with UL single carrier or UL CA_n40A-n41C.

Due to simultaneous Rx/Tx operation is supported for TDD bands n40 and n41, thus the co-existence studies caused by UL CA_n41C should be analyzed additionally for CA_n40A-n41C, respectively, which can be found in table Table 5.x.1.3.1-1:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA of n41

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n40 fDL | 2300 | 2400 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 160 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2336 | 2850 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2176 | 3010 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2016 | 3170 |  | 4832 | 5540 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1856 | 3330 |  | 4672 | 5700 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1696 | 3490 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1536 | 3650 |  | 7328 | 8230 |
| Analysis | 3rd order, 5th order, 7th order IMD , 9th order, 11th order and 13th order caused by CA_n41C impact band n40 DL |  |  |  |  |

      - 5.x.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n40-n41 are already defined in the spec TS38.101-1.

      - 5.x.1.5	REFSENs requirements

According to the co-existence study, additional IMD MSD requirements caused by UL CA_n41C should be defined for CA_n40A-n41C.

In terms of the WF R4-2502955, it was agreed that the mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1.

The MSD value given in the WF R4-2502955 are shown in table 5.x.1.5-1:

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314,16,X |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 35.5 | TDD | IMD314,16,X,Y |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. NOTE 16: This band is subject to IMD7 also which MSD is not specified. NOTE X: This band is subject to IMD5, IMD9, IMD11 and IMD13 also which MSD are not specified. NOTE Y: This second test point is optional. |  |  |  |  |  |  |  |  |

      - 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

----- End of TP -----