3GPP TSG-RAN4 Meeting # 114bis	R4-2503976

Wuhan, China, Apr.7th – 11th, 2025

# Source:	ZTE Corporation, CMCC

# Title:	TP for TR38.719-02-01_CA_n41C-n79A

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

The TP to TR to introduce CA_n41C-n79A_BCS4 and 5 can be found in [1].

In current specification, there are no MSD requirements caused by UL_n41C are defined since non-simultaneous Rx/Tx operation is considered. However, n41 and n79 can support simultaneous Rx/Tx operation, in this case the MSD requirements caused by UL_n41C are studied in the TP[1], in which IMD4 and IMD6 caused by n41C would fall into n79 DL Rx.

Due to company think the IMD MSD test point for 1 band 2UL CC should consider practical deployment aspect, which means the traditional IMD MSD test point  for 1 band 2UL CC, a.k.a 1RB+1RB allocation, can not reflect the practical deployment, instead full RB+full RB allocation should be considered.

After a long discussion in RAN4, companies achieved the agreements to define IMD MSD requirements for both 1RB+1RB allocation and full RB+full RB allocation. More details can be found in the WF[2].

Due to the MSD requirements have already done, this contribution provides a text proposal to introduce CA_n41C-n79A in TR38.719-02-01[3].

- Reference

- [1] R4-2408381,TP for TR38.718-02-01_CA_n41A-n79C and CA_n41C-n79A, ZTE Corporation, Mediatek, Sanechips

- [2] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

- [3] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- Text Proposal

----- Start of TP -----

  - 5.x		CA_n41-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n41-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n79 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n41-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n79A | CA_n41C-n79A | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	UE Co-existence studies

For the UL/DL harmonic, harmonic mixing and cross band isolation caused by single band n41 or single band n79, it was already analyzed in the fallback configurations of DL CA_n41A-n79A with UL single carrier, and it can be applied for DL CA_n41C-n79A with UL single carrier.

Due to simultaneous Rx/Tx operation is supported for TDD bands n41 and n79, thus the co-existence studies caused by UL CA_n41C should be analyzed additionally for CA_n41C-n79A, respectively, which can be found in table Table 5.x.1.3.1-1:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA of n41

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n79 fDL | 4400 | 5000 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 160 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2336 | 2850 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2176 | 3010 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2016 | 3170 |  | 4832 | 5540 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1856 | 3330 |  | 4672 | 5700 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1696 | 3490 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1536 | 3650 |  | 7328 | 8230 |
| Analysis | IMD4 and IMD6 caused by n41C would fall into n79 DL Rx. |  |  |  |  |

      - 5.x.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n41-n79 are already defined in the spec TS38.101-1.

      - 5.x.1.5	REFSENs requirements

According to the co-existence study, additional IMD MSD requirements caused by UL CA_n41C should be defined for CA_n41C-n79A.

In terms of the WF R4-2502955, it was agreed that the mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1.

The MSD value given in the WF R4-2502955 are shown in table 5.x.1.5-1:

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=93) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414,15 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 6 | TDD | IMD414,15,X |
| NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. NOTE 15: This band is subject to IMD6 also which MSD is not specified Note X: This second test point is optional. |  |  |  |  |  |  |  |  |

      - 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.x.2		Specific for 2 bands UL CA

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41C-n79A | 23 | +2/-3 |

      - 5.x.2.2		UE co-existence studies

Due to both band n41 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n41A-n79C since the Tx and Rx for either n41 or n79 will not be operation at the same time.

      - 5.x.2.3		REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

----- End of TP -----