3GPP TSG-RAN4 Meeting # 114bis	R4-2503979

Wuhan, China, Apr.7th – 11th, 2025

Source:	ZTE Corporation, Sanechips

Title:	TP for TR38.719-03-01_3DL_xUL CA_n3A-n34A-n79A

Agenda Item:	6.3.4

Document for:	Approval

- Introduction

3DL/2UL CA_n3A-n34A-n79A_BCS 4 and 5 have already been included in the revised basket WID, the fallbacks were already completed.

This contribution provides a text proposal to introduce 3DL/2UL CA_n3A-n34A-n79A_BCS 4 and 5 in TR38.719-03-01.

- Reference

- [1] TR38.719-03-01,Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2),v0.4.0

- Text Proposal

----- Start of TP -----

  - 5.x			n3-n40-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1		Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n34+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n34A-n79A | CA_n3A-n34A CA_n3A-n79A CA_n34A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n34-n79, the TIB,c and RIB,c values are proposed in the following tables:

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n34-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n34-n79 | - | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n3 the band order from left to right is n1, n3 and n3. |  |  |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n34A, CA_n3A-n79A and CA_n34A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD5 issue caused by n3+n34, i.e.  4*fn34-fn3, fall into the its own band n79 Rx;

No IMD issue caused by n34+n79 falls into the its own band n3 Rx;

IMD5 issue caused by n3+n79, i.e.  4*fn3-fn79,  fallx into the its own band n34 Rx;

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined. The MSD values are proposed in table 5.x.2.2-1..

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n34-n79 | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4900 | 3.2 | IMD5 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |
|  | n34 | N/A | 5 | N/A | 2020 | 9.7 | IMD5 |

----- End of TP -----