3GPP TSG-RAN WG4 Meeting #114bis							        	R4-2503987

Wuhan, CN, 7th-11st April 2025

Title:	RRM Core requirements on event triggered L1 measurement reporting

Source:	China Telecom

Agenda item:			7.27.2.1

Document for:	Discussion

- Introduction

In the RAN4 #114 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

In legacy, requirements for reporting criteria per measurement category for L3 measurement and reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the requirements for reporting criteria per measurement category shall be defined too.

Both L3 measurement reporting and L1 measurement reporting could trigger LTM, no need for UE to be able to support different events of L3 and L1 in parallel per category. So we think a separate capability is better.

Proposal 1: Introduce requirements for reporting criteria per measurement category.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

Issue 2-7: whether and how to coordinate with R19 MIMO

Candidate options: - Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC)

Although event triggered L1 measurement reporting are considered for both Rel-19 MIMO and Rel-19 Mobility, Rel-19 MIMO and Rel-19 Mobility are still separate features. And they are two parallel on-going work items, there may be some potential troubles with mixed requirements. So, we think it is better to define separate capabilities for MIMO and mobility.

Proposal 3: Define separate capabilities for MIMO and mobility.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Introduce requirements for reporting criteria per measurement category.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: Define separate capabilities for MIMO and mobility.

- References

[1] R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.