3GPP TSG-RAN WG4 Meeting #114bis	R4-2503990
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.14.2.3

Source:	CAICT

Title:	On Rel-19 MIMO OTA

Document for:	Approval

1	Introduction

This paper provides our views on some remaining open issues on Rel-19 MIMO OTA, including both core part and perf. part.

2	Discussion

2.1 Remaining open issues on Rel-19 dynamic MIMO OTA

The core part WI is expected to be completed at RAN#109 (Sep. 2025). According to the WID [1] and the latest SR [2], the following issues on Rel-19 FR1 dynamic MIMO OTA are still open:

- Dynamic channel model validation target values

- Dynamic channel model validation pass/fail limits

- UE performance metric

- MU assessment for FR1 dynamic MIMO OTA (RAN5)

Regarding dynamic channel model validation target values, good progress on the alignment among different companies has been observed during the past few RAN4 meetings. As captured in the WF of the last RAN4 meeting [3], the channel model target values are expected to be frozen no later than RAN4#115 (May 2025).

Issue 1-1-2: Dynamic channel model PDP target values Agreement: - Alignment among different CE vendors is encouraged. It is also recommended to freeze the channel model values no later than RAN4#115 (May 2025), to make sure the core part WI can be completed on time at RAN#109 (Sep. 2025). - Include original PDP value without any processing related to the delay resolution into TR next meeting. - Use Chebyshev window with 120dB sidelobe factor as a starting point to produce PDP target values. The window applied should be indicated in the TR.

Regarding dynamic channel model validation pass/fail limits, since the dynamic MIMO OTA and Rel-17 FR1 static MIMO OTA share the same test frequencies, the same system setup (16-probe MPAC), the same type of channel models (CDL-C UMa and UMi), part of the same validation parameters (PDP, TCF, SCF, and XPO), and the same validation measurement setups, it should be technically reasonable to reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation.

Proposal 1: Considering the dynamic MIMO OTA and Rel-17 static MIMO OTA share the same system setup (16-probe MPAC), the same type of channel models (CDL-C UMa and UMi), and the same validation measurement setups, it is proposed to reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, and XPO.

The agreements on UE performance metric are captured in the WF of the last meeting [3] as below:

Issue 1-2: Dynamic MIMO OTA test and UE performance metric Agreement: - All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test. FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF. - Use the following as a starting point: - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

The OTA performance metrics are always expected to be able to reflect an overall performance of a UE in different positions/orientations/scenarios, taking average of metrics of different states is a customary method for defining OTA performance metrics including TRP, TRS, TRMS, MASC. Thus, it is recommended to take the average of {CDF10%TMTUMi, CDF50%TMTUMi, CDF90%TMTUMi, CDF10%TMTUMa, CDF50%TMTUMa, CDF90%(TMTUMa)} as the UE performance metric for dynamic MIMO OTA.

Proposal 2: Consider taking the average of {CDF10%TMTUMi, CDF50%TMTUMi, CDF90%TMTUMi, CDF10%TMTUMa, CDF50%TMTUMa, CDF90%(TMTUMa)}  as the UE performance metric for dynamic MIMO OTA to reflect an overall performance of a UE.

2.2 Target frequency bands for Rel-19 MIMO OTA requirements

At the last meeting, it was agreed to choose either n8 or n20 to present low band, and encouraged companies to provide feedback on available number of devices and data [3]:

Agreement: - Down select the following four bands as a starting point: n3, n7, n77, (n8 or n20) and choose either n8 or n20 to represent low band. - Encourage companies to provide feedback on available number of devices each band and provide devices to test labs for data collecting.

According to GSA 5G Device Ecosystem Report (Sep. 2024), the number of devices that support band n8 is about 1.3 times that of the devices supporting band n20 (1463 vs. 1094).

Fig. 1.  Announced 5G spectrum support, number of models by band, for bands supported by 500 devices or more (data not available for all devices). Quoted from GSA 5G Device Ecosystem Report (Sep. 2024)

In addition, band n8 has been listed in the Rel-18 MIMO OTA WID [4] as a target band, but it has been deprioritized at a late stage of Rel-18 due to time limitation. It is recommended to prioritise the Rel-18 leftover, i.e, band n8.

Proposal 3: From the perspective of both available number of devices and completing the Rel-18 leftover issue, it is recommended to choose n8 rather than n20 as the target band, that is, select n3, n7, n8, n77 as four target bands of MIMO OTA requirements in Rel-19.

3	Conclusion

This paper provides our views on some remaining open issues on Rel-19 MIMO OTA.

Proposal 1: Considering the dynamic MIMO OTA and Rel-17 static MIMO OTA share the same system setup (16-probe MPAC), the same type of channel models (CDL-C UMa and UMi), and the same validation measurement setups, it is proposed to reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, and XPO.

Proposal 2: Consider taking the average of {CDF10%TMTUMi, CDF50%TMTUMi, CDF90%TMTUMi, CDF10%TMTUMa, CDF50%TMTUMa, CDF90%(TMTUMa)}  as the UE performance metric for dynamic MIMO OTA to reflect an overall performance of a UE.

Proposal 3: From the perspective of both available number of devices and completing the Rel-18 leftover issue, it is recommended to choose n8 rather than n20 as the target band, that is, select n3, n7, n8, n77 as four target bands of MIMO OTA requirements in Rel-19.

References

- RP-250197, Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #107, Mar 2025.

- RP-250196, SR for TRP_TRS_MIMO_OTA_Ph3, vivo, CAICT, RAN meeting #107, Mar 2025.

- R4-2502296, WF for [114][328] MIMO_OTA_Ph3, CAICT, RAN4#114, Feb 2025.

- R4-223509, Revised WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs, CAICT, RAN#98, Dec 2022.