3GPP TSG-RAN4 Meeting #114bis	R4-2503992

Wuhan, Hubei,  China, 07th – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-2 for FR2 MPR improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RP-240828, it is noted that a large margin exists between the current MPR requiremant and measured power back-off for intra-band CA. MPR for FR2 single carrier UL or intra band UL CA with DL intra band CA can be defined based on only UL CBW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MPR based on activated cumulative aggregated channel BW (CABW) or configured/activated UL BWchannel_CA applies instead that based on configured CABW with UE capabilities. - For the case of single carrier UL with DL intra band CA is configured/activated, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 CA MPR based on configured CABW still prevent the NW from fully utilizing the UE performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2, 6.2A.2.3, 6.2A.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draft CR is divided into three parts: - Configured UL BWchannel_CA based MPR improvements (marked as yellow) - Activated CABW based MPR improvements (marked as green) - Activated UL BWchannel_CA based MPR improvements (marked as light blue) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

6.2A.2.2	Maximum output power reduction for power class 1

6.2A.2.2.1	Maximum output power reduction for power class 1 intra-band contiguous UL CA

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA+∆MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Tables 6.2A.2.2-1 and 6.2A.2.2-2. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and the intra-band contiguous CA is activated with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2.

∆MPR for FR2-1 256 QAM as specified in Table 6.2.2.1-5 applies.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | bandwidth basis3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, activated Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, activated UL BWchannel_CA is used as bandwidth basis. Otherwise, configured Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall instead be MPR2 if [fr2-MPR-ImprovementUplinkDependent-r19] is supported, and be MPR3 if [fr2-MPR-ImprovementActivationDependent-r19] is supported, and be MPR4 if [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, and be derived as MAX(MPR1, MPR2) otherwise, where:

MPR1 shall be determined from Table 6.2.2.1-1 if configured CABW  200 MHz, from Table 6.2.2.1-2 if configured CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.1-1 if configured UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if configured UL BWchannel_CA > 200 MHz.

MPR3 shall be determined from Table 6.2.2.1-1 if activated CABW  200 MHz, from Table 6.2.2.1-2 if activated CABW > 200 MHz.

MPR4 shall be determined from Table 6.2.2.1-1 if activated UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if activated UL CCs bandwidth > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.1-1 and Table 6.2.2.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL configured BWchannel_CA assuming lowest SCS among all configured CCs. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth assuming lowest SCS among all activated CCs. If [fr2-MPR-ImprovementUplinkDependent-r19]  and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL activated BWchannel_CA bandwidth assuming lowest SCS among all activated CCs

<Unchanged sections are omitted>

6.2A.2.3	Maximum output power reduction for power class 2

For power class 2, MPR (except 256 QAM) specified in sub-clause 6.2A.2.4.1 applies for intra-band contiguous UL CA and sub-clause 6.2A.2.4.2 applies for intra-band non-contiguous UL CA.

Table 6.2A.2.3-1: (Void)

For FR2-1 256 QAM, for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth, the following rule for MPR applies:

MPR = MPRC_CA+∆MPR

MPRC_CA is defined in Table 6.2A.2.3-2. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and the intra-band contiguous CA is activated with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3.

△MPR as specified in Table 6.2.2.1-5 applies.

Table 6.2A.2.3-2: Maximum power reduction (MPRC_CA) for UE power class 2 in FR2-1

|  |  | bandwidth basis2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256QAM applicability. NOTE 2: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, activated Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, activated UL BWchannel_CA is used as bandwidth basis. Otherwise, configured Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

For FR2-1 256 QAM, for intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA+∆MPR, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL configured BWchannel_CA assuming lowest SCS among all configured CCs. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth assuming lowest SCS among all activated CCs. If [fr2-MPR-ImprovementUplinkDependent-r19]  and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL activated BWchannel_CA bandwidth assuming lowest SCS among all activated CCs

<Unchanged sections are omitted>

6.2A.2.4	Maximum output power reduction for power class 3

6.2A.2.4.1	Maximum output power reduction for power class 3 intra-band contiguous CA

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Tables 6.2A.2.4-1 and 6.2A.2.4-2. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and the intra-band contiguous CA is activated with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2.

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1

|  |  | bandwidth basis2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). NOTE 2: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, activated Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. If [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, activated UL BWchannel_CA is used as bandwidth basis. Otherwise, configured Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 |
| QPSK | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 |
| 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRC_CA shall instead be MPR2 if [fr2-MPR-ImprovementUplinkDependent-r19] is supported, and be MPR3 if [fr2-MPR-ImprovementActivationDependent-r19] is supported, and be MPR4 if [fr2-MPR-ImprovementUplinkDependent-r19] and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, and be derived as MAX(MPR1, MPR2) otherwise, where:

MPR1 shall be determined from Table 6.2.2.3-1 if configured CABW  200 MHz, from Table 6.2.2.3-2 if configured CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3-1 if configured UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if configured UL BWchannel_CA > 200 MHz.

MPR3 shall be determined from Table 6.2.2.3-1 if activated CABW  200 MHz, from Table 6.2.2.3-2 if activated CABW > 200 MHz.

MPR4 shall be determined from Table 6.2.2.3-1 if activated UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if activated UL CCs bandwidth > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3-1 and Table 6.2.2.3-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL configured BWchannel_CA assuming lowest SCS among all configured CCs. If [fr2-MPR-ImprovementActivationDependent-r19] is supported, NRB_agg_C is the number of the aggregated RBs within the fully allocated configured cumulative aggregated channel bandwidth assuming lowest SCS among all activated CCs. If [fr2-MPR-ImprovementUplinkDependent-r19]  and [fr2-MPR-ImprovementActivationDependent-r19] are supported and all activated CCs form a contiguous block, NRB_agg_C is the number of the aggregated RBs within the fully allocated UL activated BWchannel_CA bandwidth assuming lowest SCS among all activated CCs

<Unchanged sections are omitted>