3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503994

Wuhan, China, 7th – 11st April 2025

Title: 	NR_LBCA_Sw Considerations

Agenda Item:	7.7.2

Source: 	Nokia

Document for:	Approval

# Introduction

In RAN#106 meeting work item for NR_LBCA_Sw was approved. This contribution discusses about UE requirements and related capabilities. Work item objectives as listed [1] are as following.

During RAN4#114 meeting following issues was recorded [2]

Issue 1: Scope clarification

-	Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI

-	It is not precluded that such two low bands are further combined with the additional band

-	The combination of those two low bands with other band can be further discussed in the future release

Issue 2: Switching period values

Summary of company proposals (for information):

5us: 3 (Spreadtrum, Huawei, Ericsson) 13us, assuming no LO retuning: 2 (Ericsson, Huawei) 35us: 6 (Huawei, Qualcomm, ZTE, CATT, Spreadtrum, Xiaomi) 70us: 1 (Samsung) 100us: 2 (Apple, Murata) 140us: 9 (Apple, Qualcomm, ZTE, CATT, MediaTek, Spreadtrum, Samsung, Xiaomi, Murata) 140 + FFS us: 1 (Skyworks) 200us: 1 (Google) 210us: 4 (Qualcomm, ZTE, Samsung, Xiaomi)

-	Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations

-	35us, 140us

-	RAN4 to futher discuss 13us without LO switching

-	LS will capture the currently confirmed values

-	It is not precluded to send another LS with more values in future meetings

Issue 3: Switching period capability

-	Discuss whether the switching delay capability can be per band combination

Issue 4: Time mask for switching

-	Discuss the time mask for switching based on the following options:

-	Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)

-	Option 2: Switching period is always within the switch-from carrier

-	Option 3: Switching period is contained within the switching gap configured by the network

-	Other options are not precluded

Issue 5: Switching period location

-	Discuss the switching period location based on the following options:

-	Option 1: Switching period location is entirely up to RAN1 design

-	Option 2: Switching period location is defined relative to the slot boundary

-	Other options are not precluded

Issue 6: UE reference architecture

-	Companies are encouraged to contribute proposals on the UE reference architecture as a basis for developing RF requirements for the feature

Issue 7: Test methodology

-	Companies are encouraged to contribute proposals related to the testability of the time mask requirement

Issue 8: Optional advanced UE features

-	Further consider optional advanced UE features (other proposals are not precluded):

-	In the case of the carrier of band n5 support 4Rx antenna ports but n29 support 2Rx antenna ports, then there are 2Rx antenna ports might not be involved in the carrier switching, UE can optionally report the advanced UE capability to allow the carrier to receive in the receiver antenna port which is not involved in the carrier switching.

# Discussion

Operators possess 10 to 20 times more spectrum in mid bands compared to low bands. The former is advantageous for use in proximity to cell sites, whereas the latter is more beneficial at greater distances. Low-band spectrum supports substantial traffic volumes in both urban (indoor) and rural areas, resulting in congestion that significantly degrades customer experience [3]. Low-band SDL bands are underutilized due to their close frequency proximity to other low bands, complicating their aggregation for CA. Conversely, SDL low bands may be located far from another low-band carrier, rendering the UE antenna design impractical. To address these practical challenges in UE implementation, a switched Low-Band to Low-Band Carrier Aggregation operation is proposed.

WID and WF are recorded in a way that the switching framework UL switching is applicable as such for DL switching. However, this is not the case as UL switching is contributed mainly by static LO settling time, but DL switching is more related to dynamic timing advance in live network as we elaborate more about factors contributing to the switching time.

Observation 1. Using uplink switching framework for DL CA switching is not well justified as the factors contributing to the switching time are not same.

## Switched LB CA feature description

The switched LB CA operation, as stipulated by the WID, is based on a semi-static switching pattern configured via RRC. The switching pattern alternates between two modes:

- FDD

  - UL and DL

- SDL

  - DL only

At UE these two modes are enabled in following domains:

- Antenna

  - Antenna is tuned from one carrier frequency to another. Assumption is a switch-based aperture tuning

- RFFE

  - RF path selection from antenna to TRX

- TRX

  - RF Rx selection and/or

  - RF Rx chain configuration for given carrier frequency

Figure 1. Concept of switched LB CA feature [3]

To support the semi-static switching pattern configured via RRC it needs to be included to the specification. A suggestion is to include it under Clause 5.2A.2 as shown below highlighted in yellow.

*** Start of TP to TS 38.101-1 Clause 5.2A.2 ***

If the non-concurrent CA capability is applied to the inter-band carrier aggregation, the operation may be restricted to non-concurrent activity, which must follow semi-static switching pattern configured via RRC, allowing the Tx operation to be periodically deactivated to facilitate Scell activation.

*** End of TP ***

Proposal 1. RAN4 to consider updating the specification as suggest in the TP for TS 38.101-1 Clause 5.2A.2.

## Potential implementations

Figure 2. Possible implementation options for switched LB CA. A) Rx chains are shared in RFIC for two carriers, B) Dedicated Rx chains for each carrier in TRX but switched in RFFE

For simplicity, a 2+2 DL MIMO operation is considered. One Rx chain, either primary or diversity in the TRX, comprises a LNA, mixer, analog channel filter, ADC. The Local Oscillator is shared between the primary and diversity receivers. RFFE domain may or may not include an LNA for each Rx chain. Detailed antenna interface, including antenna tuning, is not depicted here. Antenna tuner is placed between RFFE and antenna and not active for conformance tests. As RAN4 defines specification only for conducted performance operation of an antenna tuner is out the scope of RAN4 performance requirements.

Option A presumes that the Rx RF chain is shared between the FDD carrier and the SDL carrier; therefore, the configuration for each carrier, as well as the frequency and time synchronization, need to be established for each cycle. This will result in a period during which the receiver cannot properly demodulate the received signal.

Option B presumes that the Rx RF chains in the TRX are separate, and routing to the antenna is switched in the RFFE domain. In this way, any configuration for the Rx RF chain can be pre-programmed before carrier switching occasion, allowing reception to start at the desired point in time.

Time and frequency synchronization for the SDL carrier may be supported by an FDD carrier or may use its own synchronization signals.

Observation 2. UE HW implementation may have an impact to UE capability to receive DL signal.

Observation 3. The tuning functionality of an antenna tuner is not a concern within the scope of RAN4 performance requirements; however, it may be considered to assess any potential impact on network performance.

## UE timing considerations

Depending on the implementation, RF solution may have various DL switching time capabilities, alternating DL reception between two carriers in a manner which resembles TDD operation on different carriers. Pcell will set the timing reference and contains essential signals for maintaining the radio link such as HARQ, consequently the FDD-SDL switching pattern must be sufficiently short.

### DL Timing

Considering for example downlink reference measurement channel for TDD which has:

- TDD slot configuration DDDSU

- Special slot configuration 10D+2G+2U

A typical DL interruption length for UL reconfigurations with 15kHz SCS is one slot in 38.133. However, this long interruption with a short switching pattern will degrade UE ability receive on SDL carrier.

Observation 4. A long gap or delay due to DL carrier switching will degrade UE ability to receive on SDL carrier when carrier switching pattern is short.

There are shorter than one slot DL interruptions allowed in 38.133 at UE switching between two uplink carriers.

“When dynamic switching between two uplink carriers is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [38.331]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [38.331].”

Example of allowed interruption lengths can be found from 38.133, Table 8.2.2.2.10-1.

Figure 3. 38.133, Table 8.2.2.2.10-1.

Reflecting figure 3 to this work item, an analogous requirement would be 2 symbol interruption on DL with shortest UL switching time of 35us. However, there are CA combinations listed in 38.101-1 that do not allow a DL interruption for DL at UL switching as HW capability of a UE is assumed to enable this.

Observation 5. Current specification do not allow interruption or it allows DL interruption from two symbols up to one slot.

### UL timing

General on/off time mask is defined in 38.101-1 as following.

Figure 5. General UE on/off time mask

On power requirement start at the start slot boundary and end also at end slot boundary. Off power shall be met at start slot boundary -10us and end slot boundary +10us.

Transient period from off to on consist of the transceiver configuration, power ramp up and phase error settling of LO due to ramp up. Transient period from on to off is a time taken by RF ramp down only. Considering a typical implementation current requirement in TS 38.101-1 provides plenty of margin for UE RF ramp down, however 10us transition period is consistent with off to on transient time in TS 38.104. One example of transient period requirements can be found from [4]

### Timing at UE

#### Two separate receivers

Considering a UE implementation like in figure 2 option B in which there are two receivers available for CA_n5-n29. According to WID both carriers are co-located, synchronized and are in single TAG. Timing advance for uplink is signalled by gNB by:

TTA = NTA + NTA,offset., where NTA,offset is fixed, 25600xTc = 13us and 0 for DSS operation.

Considering the operations required on both sides of a CA switching period at the slot boundaries, an extreme case is depicted in figure 6, where the TTA and TAE are 0us.

Figure 6. Timing at the UE with two receivers considering semi-static switching pattern and phase aligned downlink frames when TTA = 0.

UE transitioning from FDD to SDL can pre-program SDL RF Rx chain prior to the slot boundary preceding carrier switch. Switching from the FDD RF Rx chain to the SDL RF Rx chain inside TRX is seamless and can occur during the CP. DL RF switching time denoted as X includes the switch time for changing the RFFE routing from antenna to TRX input. Optionally the switching time for antenna tuner can be considered. All the needed RFFE routing changes can be done in parallel. The state change of the antenna tuner can be done in parallel with RFFE state changes thus not contributing the total switching time. The total time required for switching is no more than worst case AGC settling time between slots during normal operation. Existing implementations for programming the antenna tuner state change may not be optimal for the proposed concept but should not be used as a basis to question its feasibility.

Regarding UL, transmitter turns off after last transmitted symbol at the slot boundary. UL power ramp down time is fraction of the first CP and in general much faster than the transition time required for ramp up. For the band combination considered in this WID ramp down is not an issue as the frequency bands are apart not to cause desensitization.

UE transitioning from SDL to FDD, UL can be pre-programmed to start transmission at the slot boundary allowing some time for PLL to settle after ramp up. It is possible to position the UL transmission start, RFFE routing change and antenna tuner state change to happen at the same time.

Figure 7. Timing at the UE with two receivers when TTA ≥ 0

In reality TTA is > 0 and dynamic in live network which is illustrated in figure 7. Dynamic TTA changes the timing considerations due to change of UL transmission with respect to DL. Transitioning from FDD to SDL UL timing configuration can be the same as for TTA=0us. However, the UL and SDL now overlap in time which is possible only if FDD and SDL can be multiplexed into same antenna in RFFE but still the UE may have non-optimal OTA performance if both carriers are not covered by same antenna tuner setting. It is up to the UE implementation to decide whether the antenna tuner state change after carrier switching will occur at UL start symbol or at FDD DL start symbol.

In future switched LBCA scenarios, it may become necessary to support band combinations where the UL and SDL simultaneous operation is not feasible. To ensure a future-proof switched DLCA solution, it is imperative that for TTA > 0 conditions there is no time overlap of UL and SDL.

Figure 8. illustrates a method to address issues arising from overlapping UL transmission with SDL.

Figure 8. Timing at the UE with two receivers with mute symbol

Gray area at the end of a SDL slot is representing mute symbol scheduled by gNB. It is possible to consider one symbol to be muted by default, and total number of muted symbols can be scheduled dynamically for different TTA by network signalling. Here the last symbol of SDL is chosen to be muted to preserve UL.

Observation 6. By assuming a UE is having two dedicated receiver chains it is possible to switch from FDD carrier to   SDL carrier without an interruption or gap on downlink or uplink.

Observation 7. Switching from SDL to FDD carrier, UL transmission may overlap with SDL unless the first symbols of UL or last symbols of SDL are muted.

Observation 8. It can be decided that the last symbol of SDL is muted by default.

Observation 9. An interruption or gap for both carriers at both sides of CA switching period would be a suboptimal solution for a UE with two receivers given the dynamic nature of TTA.

Observation 10. Network may schedule dynamically more than one mute symbols at the end of SDL slot to cover different values of TTA.

#### Shared receiver for FDD and SDL

Considering shared receivers there is no possibility to configure receivers before slot boundary so there is a need to allow more time for switching by muting last symbols of DL for both FDD and SDL. It possible to consider one symbol to be muted by default but number of muted symbols may be also dynamic for different TTA by network signalling.

Figure 9. Timing at the UE with shared receiver with mute symbols at the end of FDD DL and SDL

Observation 11. For a shared receivers in TRX there is a need to mute one or more symbols at the end of FDD DL and SDL slots prior to carrier switching to allow RF Rx chains to be reconfigured.

Observation 12. UE may signal minimum number of mute DL symbols at the end of a slot it requires based on its capability

Proposal 2. RF switching time period can differ between transitions from FDD to SDL and from SDL to FDD.

Proposal 3. RF switching period shall degrade only FDD DL and/or SDL at the end of a slot prior carrier switching.

Proposal 4. Last symbol of SDL slot is muted by default.

Proposal 5. The network shall dynamically schedule resources in time such that zero or more symbols at the end of a FDD DL slot preceding carrier switching to a SDL carrier are not allocated to a UE to cover different UE capabilities.

Proposal 6. The network shall dynamically schedule resources in time so that more than one symbols at the end of a SDL slot preceding carrier switching to a FDD carrier are not allocated to a UE to cover different values of TTA.

Proposal 7. UE capabilities for FDD DL-SDL switching are denoted as {x,y}, where x represents the minimum number of symbols not allocated FDD DL prior to carrier switching and y represents minimum number of symbols not allocated SDL DL prior to carrier switching and 0≤x≤2, 1≤y≤2.

# Conclusion

Considering the motivation and objective of the work item addressing low band congestion and improving user experience we propose:

Proposal 1. RAN4 to consider updating the specification as suggest in the TP for TS 38.101-1 Clause 5.2A.2.

Proposal 2. RF switching time period can differ between transitions from FDD to SDL and from SDL to FDD.

Proposal 3. RF switching period shall degrade only FDD DL and/or SDL at the end of a slot prior carrier switching.

Proposal 4. Last symbol of SDL slot is muted by default.

Proposal 5. The network shall dynamically schedule resources in time such that zero or more symbols at the end of a FDD DL slot preceding carrier switching to a SDL carrier are not allocated to a UE to cover different UE capabilities.

Proposal 6. The network shall dynamically schedule resources in time so that more than one symbols at the end of a SDL slot preceding carrier switching to a FDD carrier are not allocated to a UE to cover different values of TTA.

Proposal 7. UE capabilities for FDD DL-SDL switching are denoted as {x,y}, where x represents the minimum number of symbols not allocated FDD DL prior to carrier switching and y represents minimum number of symbols not allocated SDL DL prior to carrier switching and 0≤x≤2, 1≤y≤2.

# 4	References

[1] RP-243317 NR_LBCA_Sw WID_cl, RAN#106

[2] R4-2503016 NR_LBCA_Sw WF, RAN4#114

[3] RP-24253 Motivation for a new work item on low band carrier aggregation via switching in Rel-19, RAN#106

[4] R4-1703046 Transient time – impact ON OFF time mask, Qualcomm, 3GPP TSG-RAN WG4#82Bis