3GPP TSG-RAN WG4 Meeting #114bis	R4-2504008

Wuhan, China, April 7th – 11th 2025

Agenda Item:	7.23.2.1

Source:	Ericsson

Title:	New SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of new BS RF requirements for SBFD slots. Discussion on impact on existing BS RF TX and RX requirements for SBFD slots and SBFD general aspects are presented in separate submissions [1-3].

# 2	Discussion

In-channel adjacent sub-band leakage ratio

##### Issue 3-2-1: Necessity of in-channel adjacent subband leakage ratio requirements Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

In RAN4#113 meeting, it was agreed to define a new requirement of in-channel adjacent leakage ratio requirement for SBFD capable BS, and further study the requirement level.

Our proposal is to reuse the concept of ACLR. Considering that there are two types of ACLR values defined in specifications, i.e., ratio and absolute PSD value, the following text analyse which one is more proper to be used and how the value should be adjusted for SBFD.

The concept of ACLR in ratio was defined when there was only one bandwidth and base stations with similar power levels. For in-channel adjacent subband leakage ratio for SBFD, the ratio’s relevance is questionable. For each different SBFD subband configuration, the DL subband size varies, but what matters for the UL subband is the amount of interference received. If the DL subband size differs but the UL subband size remains the same, the amount of interference should be consistent. Therefore, with varying subband bandwidths and DL subband powers, specifying an absolute PSD value makes more sense. The proposal is to take the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 [4] as a starting point.

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Reuse the concept of the existing ACLR (absolute PSD value) when specifying SBFD in-channel adjacent sub-band leakage requirement.

Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

### Sub-topic 3-3: In-channel adjacent subband selectivity/blocking requirements ##### Issue 3-3-2: Necessity of in-channel adjacent subband blocking requirement Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration. ##### Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements Agreement: - No necessity to introduce the new requirement for in-channel adjacent subband selectivity.

In RAN4#113 meeting, it was agreed to define a new requirement of in-channel adjacent subband blocking requirement for SBFD capable BS, and further study the requirement level.

One point requiring further study is whether in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. In our view, ICS cannot cover in-channel adjacent subband blocking. ICS need to place the interfering signal on the UL subband while in-channel adjacent subband blocking need to place the interfering signal on the DL subband. Besides, ICS comes from when receiving two UEs and one is closer than the other (all coming from UE), while in-channel adjacent subband blocking comes from the other sectors.

When defining the requirement level of in-channel adjacent subband blocking requirement, spatial isolation is an important factor to consider. In SI, spatial isolation was widely discussed and was summarized in TR 38.858, where the spatial isolation values evaluated by different companies were captured in Table 9.2.2.1-1 (partly copied below for reading purpose). Considering the values provided from all companies comprehensively, a compromise value of 80 dB spatial isolation between sectors is selected.

For a SBFD capable BS with 53 dBm TRP, -27 dBm is expected at the UL subband, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

During the discussion at the RAN4#114 meeting, some companies propose that CLI should be considered when defining in-channel adjacent subband blocking. In our view, this new requirement measures the inter-sector interference. It is not about avoiding the beam direction from one site to another. It is about avoiding beam directions that cause unintended radiation, which radiates backward from one sector to another in the near field, such as sidelobes radiating backward across the near field. The CLI handling schemes for interaction in the near field at the same site differ from what is currently being studied in RAN1 and has not been studied so far. Therefore, CLI handling scheme should not be considered when defining in-channel adjacent subband blocking requirement.

- The interfering signals of In-channel selectivity (ICS) for SBFD come from UE and are placed on the UL subband, while the interfering signal of in-channel adjacent subband blocking comes from other sectors and is placed on the DL subband.

- In-channel selectivity (ICS) cannot cover in-channel adjacent subband blocking.

- In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

Table 9.2.2.1-1: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary (Partial copy)

| FR1 |  |  | Samsung | Samsung | Ericsson | Huawei | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (subband filter-1) | Wide Area BS (subband filter-2 and EM conjugated structure) | Wide Area BS | Wide Area BS | Wide Area BS |  |
| BS TX Power dBPTX = ① dBm |  |  | 49 dBm | 49 dBm | 53 dBm | 53 dBm | 54 dBm |  |
| Number of co-site co-channel sectors considered |  |  | 1 | 1 | 2 | 2 | 2 (plus 1 sector for self-interference) |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used | DPD utilized | DPD utilized | DPD, CFR | DPD | DPD, digital filtering |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc | 75 dBc | 100 dBc | 75-90 dBc (Varies depending on scheduled beam directions) | 85 dBc | 60-80 dBc |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used | Based on 75dB for typical spatial isolation | Based on 75dB for typical spatial isolation and additional 25dB by installing EM conjugated structure between sectors | Typical site layout with around 400mm between sectors | Spatial separation between TX panel with absorbing material and choke structure. | Spatial separation between TX/RX panel; cross polarization |  |

Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Location of transient period between SBFD and non-SBFD Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

During the offline discussion at RAN4#114 meeting, the topic of location of transient period between SBFD and non-SBFD was discussed intensively, and the agreement as shown above was reached in the end, though several open issues remain that require further investigation. The following text and figure show our opinion on the location of transient periods:

Below is a summary of our view on the location of transient periods, along with a figure to clarify our views:

- Case-A: as part of regular NTA_offset in the UL part(s) of SBFD symbol(s).

- Case-B: in guard symbol(s) reserved in the DL part(s) of SBFD symbol(s).

- Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission.

- Case-D: as part of regular NTA_offset in non-SBFD UL symbol(s).

It is important to emphasize that the figure provided below illustrates these cases at the symbol level, rather than at the slot level.

Figure 1: Illustration of location of transient period between SBFD and non-SBFD symbols

The location of transient period between SBFD and non-SBFD symbols are as follows:

- Case-A: as part of regular NTA_offset in the UL part(s) of SBFD symbol(s).

- Case-B: in guard symbol(s) reserved in the DL part(s) of SBFD symbol(s).

- Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission.

- Case-D: as part of regular NTA_offset in non-SBFD UL symbol(s).

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	The interfering signals of In-channel selectivity (ICS) for SBFD come from UE and are placed on the UL subband, while the interfering signal of in-channel adjacent subband blocking comes from other sectors and is placed on the DL subband.

Observation 2	In-channel selectivity (ICS) cannot cover in-channel adjacent subband blocking.

Observation 3	In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

Proposal 1	Reuse the concept of the existing ACLR (absolute PSD value) when specifying SBFD in-channel adjacent sub-band leakage requirement.

Proposal 2	Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage.

Proposal 3	Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

Proposal 4	For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

Proposal 5	The location of transient period between SBFD and non-SBFD symbols are as follows:

Case-A: as part of regular NTA_offset in the UL part(s) of SBFD symbol(s).

Case-B: in guard symbol(s) reserved in the DL part(s) of SBFD symbol(s).

Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission.

Case-D: as part of regular NTA_offset in non-SBFD UL symbol(s).

# References

R4-2504007, SBFD general aspects, Ericsson

R4-2504009, Impact on SBFD BS RF TX requirements, Ericsson

R4-25040010, Impact on SBFD BS RF RX requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP