3GPP TSG-RAN WG4 Meeting #114bis		R4-2504015

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.3.3

Source:	Intel Corporation

Title:	Discussion on Performance part of less than 5MHz RRM requirements in Rel-19

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in December 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The performance part of the work item is supposed to be complete in June 2025, and in this paper, we propose to stick to the test cases list for the RRM performance requirements for less than 5MHz in Rel-19.

In the last WG meeting, initial version of test cases list is agreed and captured in the WF [2].

# Discussion

## General view on the Rel-19 RRM requirements

RRM core requirements in Rel-19

RAN4 agreed the core requirements BigCR in [3], and in the below table we summarize the RRM core requirements that are defined in Rel-19 for less than 5MHz work item.

| Item | Core requirement | New test case needed? |
| --- | --- | --- |
| IDLE mode CA/DC measurements | N2 = [6] for 12 PRB target SSB | Yes |
| (Conditional) HO delay with PSCell change | T∆_PSCell = 3 Trs for 12 PRB SSB target PSCell | Yes |
| SCell activation delay | T∆ = 2 Trs is added when SCell is with 12 PRB SSB | Yes |
| ((Subsequent) Conditional) PSCell addition delay | T∆ = 3 Trs for 12 PRB SSB PSCell | Yes |
| ((Subsequent) Conditional) PSCell change delay | T∆ = 3 Trs for 12 PRB SSB PSCell | No Can be verified by PSCell addition delay tests |
| SCG activation delay | T∆ = 3 Trs for 12 PRB SSB PSCell | No Can be verified by PSCell addition delay tests |
| Deactivated SCell measurements | New time period requirements | Yes |
| Deactivated PSCell measurements | New time period requirements | Yes |

In the above table we can see that not all the core requirements need new test cases since that, as long as the UE behaviour is verified in one of the test cases there is no need for the UE to be tested in other test cases which verify the same UE behaviour. For example, regarding the T∆_PSCell requirements introduced in Rel-19, as long as the UE is verified in one set of the test cases among {PSCell addition, PSCell change, CHO with PSCell change, SCG activation delay} on its behaviours related to T∆_PSCell , it is verified that the UE is implemented correctly.

Another way is to define all the test cases in the specification, and meanwhile define the test applicability for the UE supporting less than 5MHz DC operations that the UE is required to only pass one set of the test cases which verify the same PSCell requirements.

## Performance part

Test cases list

One of the general principles to define performance requirements is to specify only test cases that verify new requirements and new functionality under the scope of mobility or RRM performance for UE.

In Rel-18 less than 5MHz work item performance part discussions, RAN4 introduced intra-frequency Cell Reselection, intra-frequency handover, RLM, BFD/LR and L3 measurements.

In the last WG meeting, initial test cases list is agreed in the below table.

Table 1 Test cases list for less than 5MHz CA/DC

| No. | Item of core requirements | Type of test cases | FR | Subclause | Note | Company |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | SCell activation delay | SCell activation and deactivation of SCell is with 12 PRB SSB bandwidth | FR1 | A.6.5.3.X | Discuss test case setup based on draftCR in the next meeting. ((un)known, DRX cycle, measCycle) | Ericsson |
| 2-1 | ((Subsequent) Conditional) PSCell addition/change delay | Addition and release delay of PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.11.X | The UE is required to pass only one set of test cases among 2-1, 2-2, 2-3 and 2-4. | Ericsson |
| 2-2 |  | Inter-frequency subsequent CPA to PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.12.X1 |  | ZTE |
| 2-3 |  | Intra-frequency subsequent CPC to PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.12.X2 |  | ZTE |
| 2-4 |  | Handover with PSCell delay | FR1 | A.6.3.1.X3 |  | Intel |
| 3-1 | Deactivated SCell/PSCell measurements | SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB | FR1 | Modify A.6.6.1.9 | Modify existing test cases. Tested together with neighbour cell measurements. | Nokia |
| 3-2 |  | SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used | FR1 | Modify A.6.6.2.13 |  | Huawei |
| 3-3 |  | Deactivated PSCell measurements with 12 PRB SSB on the target PSCell | FR1 | A.6.6.1.X | New test case is introduced. | Intel |
| 4-1 | IDLE mode CA/DC measurements | SA IDLE mode CA/DC measurements on target SSB with 12 PRB bandwidth | FR1 | A.6.6.9.X |  | Huawei |

Proposal: Stick to the agreed list to introduce the test cases in April and May WG meeting.

We are supposed to complete the performance part of the item before June 2025, according to the TU allocation. So, the plan is to stick to the test cases list and work split in the coming meetings and companies should bring draftCR-s in April and May meetings.

# Conclusion

We are supposed to complete the performance part of the item before June 2025, according to the TU allocation. So, the plan is to stick to the test cases list and work split in the coming meetings and companies should bring draftCR-s in April and May meetings.

Proposal: Stick to the agreed list to introduce the test cases in April and May WG meeting.

# References

- RP-243309, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2502586, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel Corporation

- R4-2502608, Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation, ZTE Corporation, Sanechips