3GPP TSG-RAN WG4 Meeting #114bis		R4-2504019

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.25.2.1

Source:	Intel Corporation

Title:	Discussion on RRM requirements for On-demand SSB SCell operations

Document for:	Discussion

# Introduction

The Rel-19 NR network energy saving enhancements work item was approved in [1]. The work item Core part objectives for on-demand SSB are provided below.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

# On-demand SSB RRM requirements

## Deactivated SCell measurements based on on-demand SSB

Deactivated SCell measurements based on On-demand SSB

It is important to specify correct UE behaviours, and deactivated SCell measurements requirements based on the OD-SSB to guarantee fair performance enhancement compared to legacy system.

One of the principles we have had for measurements between PCell and SCells are that we assume the UE to have separate searchers dedicated to either PCell or SCells so that measurements are not shared between PCell and SCell measurements. This can be told by looking up to the specification TS 38.133 clause 9.1.5.1.2 on the CSSF definitions. The outside gap CSSF of the intra-frequency without gap deactivated SCell measurements is the number of configured SCells.

In Rel-19, we introduced deactivated SCell measurements based on OD-SSB and when there is no always-on SSB on the target SCell, we need to make sure that the UE puts all its SCell measurement resources on the OD-SSB when transmitted.

Proposal 1: Prioritize UE deactivated SCell measurements based on OD-SSB over other measurements on the SCells, when OD-SSB is configured and activated.

This is to say that when there is OD-SSB configured and activated, the CSSF outside gap value is altered to prioritize OD-SSB measurements over others. For example, for FR1 only CA in SA mode measurements in Table 9.1.5.1.2-1, CSSF equalled NSSC_SSB + Y + Z + 2 x NSSC_CSIRS + NSSC_CCA_RSSI/CO should equal NSSC_ODSSB, which is the number of SCell that has activated OD-SSB being measured by the UE. Obviously if this is the case, we don’t allow the UE to measure on any other SCell which is not with activated OD-SSB, during the fast measurement windows. There is impact on the requirements for legacy measurements that the measurement reports are delayed considering that the measurements are not carried out during fast measurement windows.

Or if we allow sharing between activated OD-SSB and always-on SSB measurements among different SCell carriers, it is not fair to allow equal sharing among all carriers during the fast measurement windows for OD-SSB. One of the ways is to define a ratio to make more use of the fast measurement windows for OD-SSB while still allow the UE to have a decent measurement report on other SCell-s which are not with OD-SSB.

Proposal 2: Introduce measurement sharing factor among OD-SSB SCell and other SCell carriers in the CSSF for SCell measurements.

Going along this direction, when there are multiple activated OD-SSB for the UE to measure on, we can also consider sharing schemes among the carriers, though equal sharing seems to be the best in terms of trade-off between performance and complexity. The network in this way easily understands how to configure the UE to achieve optimal systematic mobility performance on the SCells.

Another consideration can be when the activated OD-SSB are not overlapped at all at time domain we can further enhance the sharing scheme to allow the UE to retune between activated OD-SSB frequencies to make faster measurement reports. This imposes on the other hand, the network to consider scattered time domain allocation of OD-SSB for different SCell carriers. However, this implies complex configurations at the deployment side and does not make excessively perfect sense over a plain simple way to just make cascade/linear activations of OD-SSB among target SCell carriers.

Fast measurement windows

In the previous WG meetings, fast measurement windows are agreed that the UE uses fast measurement windows which starts from the activation of OD-SSB for quick measurement reporting based on OD-SSB. The idea is to let the UE reports by one time but really quick during the fast measurement window.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

After the fast measurement and report, the UE is required to measure on the target SCell following legacy measurement requirements even in Case #1. However, since it is not guaranteed that the OD-SSB is always activated to the UE, we don’t think there is need to specify measurements after the fast window before the OD-SSB is activated again by network signalling. At least after the OD-SSB deactivation, the UE is not required to measure on the target SCell under Case #1.

Proposal 3: After the fast measurement window, the UE is not required to measure on the target SCell under Case #1, until the next OD-SSB activation command.

The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Proposal 4: The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Case #1 measurement requirements

Then when it comes to the measurement requirements implementation, we can specify for Case #1 the below requirements.

For FR1, measurement period requirements and SS detection requirements for deactivated SCell are both 5 x Kp x measurement cycle x CSSF while for FR2 it is ceil(M x Kp x Klayer1_measurement) x measurement cycle x CSSF.

Case #2 where always-on SSB is transmitted

The difference between deactivated SCell measurements for Case #2 and Case #1 is major from the fact that after the fast measurements, the UE in Case #2 falls back to legacy measurements based on the always-on SSB on the same target SCell, while UE in Case #1 is not required to measure on anything on the SCell.

Proposal 5: UE falls back to legacy deactivated SCell measurements after the fast measurement window under Case #2.

During the fast measurement window, the UE follows the same requirements as in the Case #1. Same CSSF will apply between Case #1 and Case#2 deactivated SCell measurements in the fast measurement windows. This facilitates unified UE baseband implementation in terms of fast measurements on the target deactivated SCell.

Proposal 6: For Case #2 during the fast measurement window, the UE follows the same requirements as in the Case #1.

Considering that during the fast measurement window, the UE follows the measurement cycle of OD-SSB periodicity configurations -- when the OD-SSB is with the same frequency of always-on SSB, there is no problem for the UE to carry out the measurements based on the measurement cycle of OD-SSB periodicity.

Proposal 7: Same measurement cycle applies in the fast measurement window for both Case #1 and Case #2 UE deactivated SCell measurements.

## SCell activation based on on-demand SSB

Known cell conditions based on On-demand SSB during SCell activation

In the previous meeting RAN4 agreed the principles for known SCell criteria as in the below box.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Regarding Case #1 where there is no always-on SSB transmitted on the target SCell, the known cell conditions are based on on-demand SSB only.

We propose that RAN4 specifies the known cell conditions based on OD-SSB in a similar way as in the legacy requirements:

Proposal 8: In Case #1, SCell in FR1 with OD-SSB is known if it has been meeting the following conditions:

- During the period equals max(5 * [measCycle], 5 * DRX cycle) for FR1 before the reception of the SCell activation command:

  - The UE has sent a valid measurement report for the SCell being activated and

  - The OD-SSB measured remains detectable according to the cell identification conditions.

- The OD-SSB measured during the period equals max(5 * measCycle, 5 * DRX cycle) also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise the OD-SSB SCell in FR1 is unknown.

For Case #2 when there are both always-on SSB and OD-SSB transmissions, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycle, 5 * DRX cycle) period. The corresponding side conditions should also be fulfilled that the target SSB of the measurement report should remain detectable all the time during the period. Note that mostly measCycleSCell is probably larger than measCycle, and since measCycleSCell is the legacy/existing implementation, we would still like to propose this method.

Proposal 9: For Case #2, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycle, 5 * DRX cycle) period before the activation command is received at the UE.

L1 and L3 measurements based on On-demand SSB during SCell activation

During a typical activation procedure, the UE needs to get the L3 results on the target SCell first and L1 measurements are carried out on the SCell for better beam information. One of the typical applications of On-demand SSB is to trigger transmissions together with SCell activation command and to seize transmissions after receiving UE activation CSI report. RAN1 also discusses about other deactivation procedures in the meantime. It is possible that during the whole process we could allow the UE to measure on On-demand SSB-s for either L3 or L1 measurements or both.

It is likely that RAN1 confirms reusing physically layer designs for the On-demand SSB-s for activation procedure. RAN4 specifies the L3 and L1 measurements requirements during the activation based on the On-demand SSB configurations.

In the previous meetings, RAN4 agreed on which type of SSB is used during SCell activation:

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

Issue 1-5-1: The type of OD-SSB based SCell activation FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios. - Option 1: Direct SCell activation at SCell addition - Option 2: Multiple SCells activation - Option 3: Deactivated PUCCH SCell activation - Option 4: Rel-17 Fast SCell Activation - Option 5: Rel-18 SCell Activation with the L3 reporting during activation

So we propose:

Proposal 10: Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot (n + THARQ + T activation_time + T CSI_Reporting ) / NR slot length.

If the SCell is known and belongs to FR1, T activation_time is based on OD-SSB: T FirstODSSB + Trs + 5ms.

If the SCell is unknown and belongs to FR1, L3 measurements are based on OD-SSB for both Case #1 and Case #2, where T activation_time is:

- 6ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + T HARQ + max(T uncertainty_MAC + T FineTiming + 2ms, T uncertainty_RRC + T RRC_delay) if SP CSI-RS is used,

- 3ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + max(T HARQ + T uncertainty_MAC + 5ms + T FineTiming, T uncertainty_RRC + T RRC_delay) if periodic CSI-RS is used.

In the above functions, T L1-RSRP, measure is based on the OD-SSB for both case #1 and case #2.

In the previous release we specified fast SCell activation delay requirements for UE capable of fast SCell activations on intra-band contiguous CA and inter-band CA where the target SCell has similar spatial characteristics as the PCell. The below requirements can be referred to.

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting [scellWithoutSSB-interband], if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The [EPRE] difference at the UE is smaller than or equal to [9] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter [SSB-less-Referencecell]. If UE is not indicated with [SSB-less-Referencecell], the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Thus, the mentioned Rel-19 SCell activation requirements which are based on OD-SSB apply only when the UE indicates support of Rel-19 NES UE features but do not support the fast SCell activation features or the conditions of the target SCell do not meet the specified conditions. When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

Proposal 11: When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

# Conclusion

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

Proposal 1: Prioritize UE deactivated SCell measurements based on OD-SSB over other measurements on the SCells, when OD-SSB is configured and activated.

Proposal 2: Introduce measurement sharing factor among OD-SSB SCell and other SCell carriers in the CSSF for SCell measurements.

Proposal 3: After the fast measurement window, the UE is not required to measure on the target SCell under Case #1, until the next OD-SSB activation command.

Proposal 4: The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Proposal 5: UE falls back to legacy deactivated SCell measurements after the fast measurement window under Case #2.

Proposal 6: For Case #2 during the fast measurement window, the UE follows the same requirements as in the Case #1.

Proposal 7: Same measurement cycle applies in the fast measurement window for both Case #1 and Case #2 UE deactivated SCell measurements.

Proposal 8: In Case #1, SCell in FR1 with OD-SSB is known if it has been meeting the following conditions:

- During the period equals max(5 * [measCycle], 5 * DRX cycle) for FR1 before the reception of the SCell activation command:

  - The UE has sent a valid measurement report for the SCell being activated and

  - The OD-SSB measured remains detectable according to the cell identification conditions.

- The OD-SSB measured during the period equals max(5 * measCycle, 5 * DRX cycle) also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise the OD-SSB SCell in FR1 is unknown.

Proposal 9: For Case #2, the SCell is considered a known SCell when measurement report is sent either based on always-on SSB or OD-SSB during max(5 * measCycleSCell, 5 * measCycle, 5 * DRX cycle) period before the activation command is received at the UE.

Proposal 10: Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot (n + THARQ + T activation_time + T CSI_Reporting ) / NR slot length.

If the SCell is known and belongs to FR1, T activation_time is based on OD-SSB: T FirstODSSB + Trs + 5ms.

If the SCell is unknown and belongs to FR1, L3 measurements are based on OD-SSB for both Case #1 and Case #2, where T activation_time is:

- 6ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + T HARQ + max(T uncertainty_MAC + T FineTiming + 2ms, T uncertainty_RRC + T RRC_delay) if SP CSI-RS is used,

- 3ms + T FirstODSSB + T SMTC_MAX + Trs + T L1-RSRP, measure + T L1-RSRP, report + max(T HARQ + T uncertainty_MAC + 5ms + T FineTiming, T uncertainty_RRC + T RRC_delay) if periodic CSI-RS is used.

In the above functions, T L1-RSRP, measure is based on the OD-SSB for both case #1 and case #2.

Proposal 11: When the UE supports fast SCell activation for intra-band contiguous CA or inter-band CA and indicating support of scellWithoutSSB or scellWithoutSSB-InterBandCA-r18 and the conditions meet fast SCell activation conditions, the Rel-19 requirements do not apply.

# References

- RP-234065, New WID: Enhancements of network energy savings for NR, Moderator (Ericsson)