3GPP TSG-RAN WG4 Meeting #114bis		R4-2504021

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.17.2.2

Source:	Intel Corporation

Title:	Discussion on delay reduction by optimizing CSSF outside gap

Document for:	Discussion

# Introduction

The Rel-19 NR RRM enhancements work item was approved in March 2024 and an updated WID was approved in RANP #105 in [1]. The work item descriptions of objectives of the item are shown in the below box.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last WG meeting, a WF was approved in [2] to capture all the discussions and agreements.

In this paper we provide our analysis on delay reduction by optimizing CSSF outside gap.

# Discussion

## CSSF outside gap in CA/DC operations

Additional searcher for CSSF outside gap

One of the directions of enhancements to CSSF outside gap within the scope of this item is to allow UE 3 searchers assumption. This means that the UE with three searchers support 3 parallel measurements, as compared to the legacy assumption which is 2, that facilitate a much faster measurement in general and it is beneficial to the system mobility/measurement performance. Different from single CC measurements, this assumption applies to both FR1 and FR2 measurements.

When measurement gap is not configured to the UE, the UE uses the additional searcher on measurements without gap to speed up the measurement reporting of any interested carrier for the system.

In the previous WG meetings, agreement was

Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

In the Rel-19 specifications, we define the additional searcher enhancements so that the UE can measure outside measurement gap on a target carrier either from network indication or from the UE’s choice.

  - The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

  - The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

  - The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

  - The CSSF outside gap definition for the additional searcher measurement requirements is 1 when there is only one target carrier, while it is N when there is N carriers configured/decided by UE among the targets.

  - This requirement applies only when there is no measurement gap configured in Rel-19.

  - This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

Proposal 1: The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

Proposal 2: The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

Proposal 3: The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

Proposal 4: This requirement applies only when there is no measurement gap configured in Rel-19.

Proposal 5: This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

# Conclusion

In this paper we provide our analysis on delay reduction by optimizing CSSF outside gap.

Proposal 1: The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

Proposal 2: The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

Proposal 3: The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

Proposal 4: This requirement applies only when there is no measurement gap configured in Rel-19.

Proposal 5: This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple