3GPP TSG RAN WG4 Meeting#114bis                                 R4- 2504022

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.2

Source:	vivo

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: Postpone this discussion until the requirement and test definition is clear

In this contribution, we further provide our views on CSI prediction.

- Discussion on CSI prediction

In the previous meetings, RAN4 have agreed to use relative throughput as requirement metric for CSI prediction, and companies are invited to bring contribution on baseline throughput.

In last meeting, the following simulation assumptions have been agreed for further study.

Simulation assumptions for AIML based CSI prediction are defined in Table 1. Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. Subcarrier spacing; 15kHz for 2GHz, 30kHz for 4GHz. gNB TX antennas; Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. UE RX antennas; 4. Channel model; TDLA30 as baseline with XP medium correlation. Doppler spread; 20Hz, 50Hz and 100Hz. Delay spread; 30ns. Channel estimation; Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). Rank per UE; Rank 1 and 2. CSI feedback assumption; Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements. CSI feedback overhead; Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later. Baseline; Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements). KPI; Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms.

Based on the above simulation assumptions, the intial evaluation results for FDD and 32gNB Tx are provided in the following table.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Doppler spread: 20Hz | Doppler spread: 50Hz | Doppler spread: 100Hz |
| --- | --- | --- | --- |
| SGCS | 0.993 | 0.902 | 0.298 |

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

During the simulation, there would be so many simulation assumptions which are not aligned but have large impact on the performance, such as the input RB number, input Rx number of the AI model. Also, different model structures and model parameters would have large impact on the performance. Then, if it is difficult to align the results corresponding to test metric, reference AI model for CSI prediction would be needed.

Proposal 1: Reference AI model for CSI prediction would be needed, if it is difficult to align the results corresponding to test metric

In the study of R18 and R19, AI CSI prediction is supposed to achieve some performance gain compared to non-AI CSI prediction, and then it may be not suitable for AI CSI prediction to directly reuse the the test setup and simulation assumptions of non-AI CSI prediction. RAN4 could work on the test setup and simulation assumptions of AI CSI prediction, by the future evaluations. For this one-sided use case, reference model should be defined to align results from companies, and then the requirements could be derived based on the aligned results.

Proposal 2: AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Then in the coming meeting, RAN4 could further discuss the simulation assumptions for Tput, e.g., CSI-RS configuration, codebook configuration of Rel-18 eTypeII-Doppler, and SNR. Current RAN1 agreements and non-AI prediction configuration may be used as starting point.

Proposal 3: RAN4 to discuss further simulation assumptions for Tput in the RAN4#114bis, e.g., CSI-RS configuration, codebook configuration of Rel-18 eTypeII-Doppler, and SNR.

In 2024 Q1 to Q3, it is SI of AI prediction and RAN1 focused on the simulation of AI prediction, to obtain acceptable performance gain compared to non-AI prediction. AI prediction has been agreed for WI in 2024 Q3. In 2024 Q4, RAN1 has been limited to the study on consistency of training/inference. Then, in fact, RAN1 has not start the work of spec impact of AI prediction. LCM aspects, such as monitoring, have not been discussed yet. Then, RAN4 could not discuss LCM requirements (e.g., monitoring) and should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

Proposal 4: LCM requirements (e.g., monitoring) should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: Initial evaluation results for CSI prediction AI/ML model is shown in Table 2-1, for FDD and 32gNB Tx.

Table 2-1. Initial evaluation results for CSI prediction AI/ML model, for FDD and 32gNB Tx.

| FDD, 32 gNB Tx | Doppler spread: 20Hz | Doppler spread: 50Hz | Doppler spread: 100Hz |
| --- | --- | --- | --- |
| SGCS | 0.993 | 0.902 | 0.298 |

Proposal 1: Reference AI model for CSI prediction would be needed, if it is difficult to align the results corresponding to test metric

Proposal 2: AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Proposal 3: RAN4 to discuss further simulation assumptions for Tput in the RAN4#114bis, e.g., CSI-RS configuration, codebook configuration of Rel-18 eTypeII-Doppler, and SNR.

Proposal 4: LCM requirements (e.g., monitoring) should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.