3GPP TSG RAN WG4 Meeting #114bis	R4-2504023

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.19.3

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN4#114 meeting, there were some further discussions on key issues including KPIs related, the update on simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1].

In this contribution, we will further analyze the impacts on requirements for beam prediction and update some simulation results on proposed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [2].

- Requirements for beam predictions

In the previous meetings, the RRM impacts on beam management was discussed. The package of requirements for beam predictions was initially identlified as follows:

- Measurement period for prediction

- The KPIs on prediction accuracy

- TCI state known/unknown

- Performance monitoring

Based on the latest progress of each bullet, we will further provide our analysis on the potential requirements.

  - Prediction delay requirements

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

For the latency on prediction, the above agreement was reached in the RAN4#113 meeting. It was agreed that RAN4 would define the requirement for an overall delay. Next, we will analyze these two parts of delay in more detail.

- Measurement for prediction

This process refers to UE performing the L1-RSRP measurements for set B and obtaining the corresponding results as model inputs. Basically, the legacy measurement period on L1-RSRP can be as the reference for defining requirements on measurements of set B.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

To be specific, the related parameters to be considered at least include:

- The number of samples for set B measurements

The number of samples will affect the prediction accuracy. For spatial prediction, RAN4 needs to further discuss whether the measurement of set B is based on one-shot measurement or measurement of multiple sample numbers. For time domain prediction, more discussion is needed on how to define the corresponding delay with consideration of characteristics of time dimension prediction, e.g., the number of samples corresponding to the latest measurement instances and the interval between time instances. The corresponding values depend on the network configuration. Configurable values need to be further discussed by RAN1/2.

- Beam sweeping factor

For DL Tx beam prediction evaluations, RAN1 had some preliminary discussions and simulations of the hypothesis on the Rx beam as AI/ML model input for training and/or inference during the SI phase, including:

- Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample.

- Option 2: Measurements of specific Rx beam(s).

- Option 3: Measurements of random Rx beam(s) per model input sample.

- Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping.

Among them, option 1 is implemented in the same way as the existing requirement definition, which requires beam sweeping to find the best Rx beam. The value of the existing beam sweeping factor can be reused. However, since RAN1 did not further discuss the Rx beam assumption in the SI and WI phases. It is possible to implement it as any of the above four options. Therefore, RAN4 may need to further determine the Rx beam assumption.

- sharing factor N

The value of this parameter may be related to factors such as measurement gap and SSB/SMTC period. It can wait until other factors are more clearly defined.

- inference delay

For model inference delay, the inference process is mainly from the data input to the model to the corresponding inference result is derived. Additionally, there may be some data processing time on top of that, which part of the delay may be related to the specific inference scenario (the number of Set B and Set A, time domain prediction or spatial prediction, etc.) and the processing capability of the model. It needs to be further discussed how to determine the value by RAN4.

Based on the above considerations, we have the following proposal:

Proposal 1: On the definition of prediction delay requirements for BM, RAN4 to consider:

- For Measurement for prediction, the current measurement period for L1-RSRP measurement can be the baseline and further discuss the details by considering the parameters, which at least include:

  - The number of samples for set B measurements (for spatial domain prediction); The number of the latest measurement instances and the time interval between two adjacent time instances (for time domain prediction)

  - beam sweeping factor

  - sharing factor N

- For inference delay: the reference latency and possible processing time

2.2	TCI state known/unknown

Agreement in RAN4#113 meeting Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions Agreement in RAN4#114 meeting Issue 2-4: QCL Source RS for TCI states Agreement: - wait for RAN1 discussion to conclude without sending any LS

On whether the TCI state associated with a predicted Tx beam is known or unknown, the main divergence lies in whether the Rx beam is implicitly considered as known information for DL Tx beam prediction. In RAN4#113 meeting, RAN4 further discussed the issue of UE Rx beam knowledge and reached the further agreement as above. And in RAN4#114 meeting, RAN4 briefly discussed the QCL Source RS for TCI states.

On how to know Rx beams information, it is somehow implementation dependent. In some implementations, UE first predicts the Tx beam and further determines the best Rx beam paired with the Tx beam by beam sweeping. For some UEs with specific model implementation, the information of Rx beam may be implicitly indicated or included as the model input, and then UE can directly know Rx beam corresponding to the best TX beam. Basically, it is not preferred to use the latter implementation as the default assumption for the DL Tx beam prediction.

To move forward, we suggest firstly identifying the case that no beam sweeping is needed after Tx beam prediction Regarding the relationship between Set A and Set B. There are two cases in RAN1 evaluation, including:

Case i): Set A and Set B are different (Set B is NOT a subset of Set A).

Case ii): Set B is a subset of Set A.

At the stage of data collection for inference, UE performs measurements on set B with a specific Rx beam assumption. As we mentioned in clause 2.1, there are several options of hypothesis on the Rx beam as AI/ML model input for training and/or inference. Based on RAN1 simulation, Option 1 with the measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample, AI/ML provides better performance than other methods. If we take Option 1 as the base assumption, when Set B is a subset of Set A and the best predicted Tx beam is in Set B, the Rx information should be known because the best Rx beam corresponding to Set B has been found by beam sweeping before the prediction. In this case, no need to perform beam sweeping again.

For other cases, e.g., the case that Set A and Set B are different or no beam sweeping is performed during data collection for training/inference, etc., RAN4 can further discuss the feasible methods to obtain Rx beam knowledge, e.g., introducing UE capability or making the reduced beam sweeping factor assumption.

Proposal 2: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information, e.g., introducing UE capability or making the reduced beam sweeping factor assumption

2.3	Performance monitoring

The framework for performance monitoring is beginning to take shape after RAN1/2 discussion. For information, we copy the latest RAN1 conclusions on performance monitoring for BM as follows.

Agreements in RAN1#118bis - For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreements in RAN1#119 - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, the method of UE-assisted performance monitoring is supported, which is achieved by comparing the prediction results with the actual measurements results, calculating the final performance metric(s) and then reporting it to NW for assisting further decision-making on model management.

From RAN4 perspective, the requirements on performance monitoring shall be considered. However, before discussing the definition of requirements in detail, we believe that it needs to first figure out the testing goal for performance monitoring.

As we mentioned above, for UE-assisted performance monitoring, the set of monitoring actions, including the comparison between prediction results and actual measurement results and the evaluation by calculating performance metrics, are performed on UE side. From UE perspective, it is expected to correctly perform the whole monitoring process. As alternatives, the corresponding UE behavior can be verified from different perspectives as follows:

-	Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE

-	Option 2: The testing goal is to verify whether UE can report the performance metrics within a certain latency upon performance monitoring is required.

-	Option 3: both of the testing goals in Option 1 and 2 need to be verified

For Option 1, from NW perspective, the decision-making for subsequent actions (e.g., model deactivation, fallback) highly depends on the monitoring results of prediction performance. Therefore, the correctness of the performance metric reported by UE is a key point for verifying.

In terms of Option 2, it focuses on the performance metric that can be reported to the network within the specific latency and further ensures NW can make corresponding model management according to the results in time. Further details on reporting, e.g., when and how to report are still under discussion in RAN1.

For Option 3, both Option 1 and 2 are required.

Based on the above analysis, we propose to first figure out what is to be verified for performance monitoring and further work on the requirements design based on the agreed testing goal.

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

  - KPIs related

KPIs on beam management were further discussed in the previous meeting. The alternatives being discussed by companies are summarized as the agreement in RAN4#113 meeting. Based on it, RAN4 made further agreement in RAN4#114 meeting shown as follows:

Agreement in RAN4#113 meeting Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Issue 3-4: Metrics/KPIs for beam prediction Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams Recommended WF - To be discussed Agreement in RAN4#114 meeting Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Next, we will conduct separate analyses for two key performance indicators (KPIs): RSRP prediction accuracy and beam prediction accuracy.

2.4.1	RSRP prediction accuracy

2.4.1.1	Definition alignment

According to the RAN4#114 meeting conclusions, when RSRP prediction results are supported as part of the reporting content, RSRP accuracy will be defined as one of the KPIs in requirements. In the latest RAN1#120 meeting, RAN1 reached several agreements regarding RSRP prediction, including: explicit support for predicted RSRP as reportable content and its associated quantization. Therefore, from RAN4 perspective, corresponding RSRP accuracy requirements should be defined.

Agreement in RAN1#120 meeting - For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value. - For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. - For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Specifically, regarding how Option 1 is applied, preliminary conclusions were already reached during the RAN4#112bis meeting, though with an open item remaining: [top-K beams selection based on ground-truth L1-RSRP]. RAN4 can use this as the basis for further discussion.

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

For the part in bracket [in top-K beams based on ground-truth L1-RSRP], we would like to put several possibilities on the table for further analysis.

- Definition 1: The index i may be any beam index

- Definition 2: The index i may be any beam index in top-K beams based on ground-truth L1-RSRP

- Definition 3: The index i may be any beam index in top-K beams based on predicted L1-RSRP

For definition 1, if we assume that the value of index i can range from 1 to N, where N is the total number of beams in set A, it means that the predicted RSRPs corresponding to all Beams of the predicted set A need to meet RSRP accuracy requirements. Given that only Top-K predicted beams and corresponding predicted RSRP (if needed) are required to report, for the remaining beams, whether the corresponding RSRP is predicted accurately may not be so critical. From this point of view, the definition 1 is not preferred.

For definition 2, if the predicted beams selected to meet accuracy requirement are based on ground-truth L1-RSRP, there may be a case to be noticeable as follows.

Case: The Top-K beam based on predicted L1-RSRP (e.g., beam X) is not any of the Top-K beams based on ground-truth L1-RSRP

To explore the probability of this case occurring in actual predictions, we have some simulations and the results provided below. Table 1 shows the observation that there is a certain possibility that the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP. For example, for the Top-index(index=2) beam based on predicted L1-RSRP, the probability that this beam is one of the Top-2 beams based on ground-truth L1-RSRP is 72.15%. Further, if the measurement error is considered in model input, the probability of this happening will be lower.

Table 1 the probability that Top-index predicted beam is one of the Top-K(K=2/5) genie-aided beams

| Simulation results |  | Probability (%) |  |  |
| --- | --- | --- | --- | --- |
| Inference SNR |  | -3dB |  |  |
| Predicted Top-index Beam | the Top-index predicted beam is one of the Top-2 genie-aided beams | No error | Case 1NOTE | Case 2 NOTE |
| Index=1 |  | 92.18 | 86.02 | 91.57 |
| Index=2 |  | 72.15 | 53.41 | 69.64 |
| Predicted Top-K Beam | the Top-index predicted beam is one of the Top-5 genie-aided beams | No error | Case 1 | Case 2 |
| Index=1 |  | 99.60 | 99.12 | 99.52 |
| Index=2 |  | 99.36 | 98.07 | 99.28 |
| Index=3 |  | 97.21 | 93.33 | 96.79 |
| Index=4 |  | 88.91 | 83.29 | 88.59 |
| Index=5 |  | 71.43 | 63.29 | 70.12 |
| NOTE: The case 1 and case 2 refer to the measurement error modelling described in Clause 3 |  |  |  |  |

Observation 1: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth of RSRP.

In this case, according to definition 2, the predicted L1-RSRP of beam X is not required to satisfy accuracy requirement as this beam is not any of the Top-K beams based on ground-truth L1-RSRP. However, the truth is that the corresponding predicted L1-RSRP of beam X still needs to be reported (if needed). For DL Tx beam prediction, UE is responsible for predicting and reporting the strongest beams and corresponding predicted RSRP since any reported beam may be selected by the network as the Tx beam to be used, so the corresponding RSRP for each reported beam needs to have a certain accuracy. Therefore, if we go with definition 2, in the case of the Top-K beam based on predicted L1-RSRP is not any of the Top-K beams based on ground-truth L1-RSRP, the corresponding predicted RSRP cannot be guaranteed.

Observation 2: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Based on the above analysis, we propose definition 3 that the index i refers to any beam index in top-K beams based on predicted L1-RSRP. For this definition, it aims to ensure the accuracy of all predicted RSRPs of reported beams.

Proposal 4: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

2.4.1.2	Simulation results with consideration of measurement error

In this section, we present simulation results based on the latest assumptions [2] to evaluate the defined absolute RSRP accuracy requirements. We also consider the performance impact of the following three error modellings on beam prediction accuracy in each KPI:

- Case 1: Only BB error is considered

- Case 2: Only RF error is considered

- Case 3: Both BB error and RF error are considered

Here, baseband (BB) error is modeled using LLS-based measurement error modeling with an assigned SNR of -3 dB. For RF erro, Truncated Gaussian distribution under ±4dB RF error is used

Table 2 show the simulation results of RSRP difference between the predicted L1-RSRP of the Top-K (K=1, 3, 5) predicted beams and the ideal L1-RSRP of the same beams. The measurement errors are only added in the model input.

From Table 2, it can be observed that, for all cases, Top-1 beams show the highest RSRP difference compared with corresponding Top-3/Top-5 case. Top-3/Top-5 beams reduces sensitivity to errors. Besides, with the consideration of measurement error, the RSRP difference have moderate increase in Case 1(only BB error) and severe degradation in Case 2(only RF error). For Case 3 (both BB error and RF error), it has additive effect, with Top-1 beams suffering the worst performance (9.06 dB, ~120% increase vs. no error)..

Table 2 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1/Top-3/ Top-5 predicted beams and the ideal L1-RSRP of the same beams

| Simulation results | L1-RSRP difference (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| Top K | No error | Case 1 | Case 2 | Case 3 |
| Top -1 | 4.12 | 4.30 | 8.56 | 9.06 |
| Top -3 | 3.85 | 4.11 | 7.40 | 8.16 |
| Top -5 | 3.16 | 3.47 | 6.20 | 6.67 |

Table 3 show the simulation results of RSRP difference between the predicted RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal RSRP of the corresponding Top-1/ Top-3/ Top-5 genie-aided beam. The measurement errors are only added in the model input.

Table 3 90%-tile L1-RSRP difference between the predicted RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal RSRP of the corresponding Top-1/ Top-3/ Top-5 genie-aided beam

| Simulation results | L1-RSRP difference (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| Top K | No error | Case 1 | Case 2 | Case 3 |
| Top -1 | 1.68 | 1.94 | 3.34 | 3.66 |
| Top -3 | 1.30 | 1.69 | 3.06 | 3.26 |
| Top -5 | 2.14 | 2.54 | 4.18 | 4.56 |

For this metric, compared with no error case, there is only small degradation (~0.2–0.4 dB across K) when considering BB error. And for RF error, larger impact (e.g., +1.66 dB for Top-1) is observed for RSRP prediction. But it is less severe than in Table 2. When both BB and RF errors are considered, it is the near-linear summation of individual errors (e.g., Top-5: 2.54 dB + 4.18 dB → 4.56 dB).

Based on the simulation results in Table 2 and Table 3, we have the following observations:

Observation 3: RF errors are more detrimental than BB errors, especially for beam-specific predictions (comparision between predicted beam and acutal beam with the same ID ).

Observation 4: Larger K improves resilience but may not compensate for RF-dominated errors (e.g., Case 3 in Top-5 still shows >6 dB difference).

2.4.2	beam prediction accuracy

2.4.2.1	Definition alignment

Based on the conclusions from the previous meeting, in terms of beam precidtion accuracy, the current candidates consist of three components:

-	Option A: Beam prediction accuracy

-Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option B: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

-	Option C: a combination of Option A and/or Option B or modified Option A/B

For Option A, according to our submitted simulation results of specific metrics (i.e., Top-1, Top-K/1, and Top-1/K) from previous meetings, we have made the following observations:

- With the increasing number of K (i.e., Top K/1 and Top 1/K), the prediction accuracy increases accordingly

- When measurement error is considered, there is significant performance degradation on Top-1 beam prediction (even lower than 50% accuracy in some cases).

- The performance on prediction accuracy defined as Top-K/1and Top-1/K are closed.

From RRM requirement perspective, RAN4 needs to determine one metric on beam prediction accuracy among these candidate metrics. Firstly, as per the observations above, the metric of Top-1 lacks certain robustness. And for the metrics of Top-K/1 and Top-1/K, the performance is approximate so that RAN4 can only choose one of them as the KPI. In our thinking, considering there is a case in UE that just reports the beam ID of Top-K without any ordering information. The Top-1 predicted beam may not be known, Top-K/1 is more appropriate as the KPI to figure out even without any rankings. Another consideration is that, Top-K/1 has been currently considered as the one of KPIs in the current simulation evaluation. In the following, we will take Top-K/1 as one of the KPIs, provide corresponding simulation results and analyze them

On the top of Top-K/1, we recognize that both Option B and Option C essentially introduce certain margins based on the Top-K/1 metric. This aspect has been considered in simulation assumption discussions. Accordingly, the latest approved simulation assumptions incorporate the following set of candidate KPIs:

KPI 1: - Top K/1 without considering some margin - the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", K=1,2,3,4,5 - Top K/1 with considering some margin; The margin can be implemented as: - Alt 1: Extend Top K/1 to Top K/N - the percentage of "the Top-N genie-aided beam containing ‘m’ of the Top-K predicted beams", where m = 1, 2, K/N= 2/2, 2/4, 3/2, 3/4, 4/4, 5/2; - Alt 2: The successful rate for the correct prediction which is considered as maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – x dB, where K=2,3,4,5, X= 1dB

For Alt 1, the rationale for proposing Top-K/N primarily stems from our previously submitted simulation results, which yielded the following key observations:

- The RSRP difference between the ideal L1-RSRP of Top 1 genie-aid beam and Top 2 genie-aid beam is quite close and about 0.5dB. To some extent, this shows that even if there is no error, for the Top two beams, it is difficult for UE to say which beam is the real Top 1 when performing RSRP measurement. From this point of view, it is also reasonable to use the KPI of Top K/N to measure the accuracy of beam prediction.

- Except for the difference between the ideal L1-RSRP of the Top-5 predicted beams and the ideal L1-RSRP of Top N (when N=1,2) genie-aid beam reaches more than 10dB, the RSRP between the ideal L1-RSRP of the other (Top 1,2,3) predicted beams and the ideal L1-RSRP of Top N (when N=1,2) genie-aid beam are close.

By summarizing the above two observations, we can get: the KPI of Top K/N mitigates the effects that the RSRPs between Top 1 and Top 2 genie-aid beams are too close (within 0.5dB), which may cause the actual Top-1 beam may be indistinct.

Observation 5: The KPI of Top K/N mitigates the effects that the RSRPs between Top 1 and Top 2 genie-aid beams are too close (within 0.5dB), which may cause the actual Top-1 beam may be indistinct

2.4.2.2	Simulation result with consideration of measurement error

For further comparison, here, we also conducted simulation verification of Top K/1 without/with considering some margin. The simulation results are shown below.

Similar to the simulation of Top K/1, we also consider the performance impact of the following three error modellings on beam prediction accuracy in each KPI:

- Case 1: Only BB error is considered

- Case 2: Only RF error is considered

- Case 3: Both BB error and RF error are considered

<simulation results on Top K/1 without considering some margin >

The corresponding simulation results of the three cases are as follows:

Table 4 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Prediction accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Top K/1 | No error | Case 1 | Case 2 | Case 3 |
| Top 1 | 68.72 | 66.35 | 40.00 | 37.11 |
| Top 2/1 | 87.95 | 86.59 | 67.39 | 65.62 |
| Top 3/1 | 91.38 | 90.36 | 79.36 | 79.68 |
| Top 4/1 | 93.38 | 92.77 | 84.10 | 82.81 |
| Top 5/1 | 96.41 | 95.50 | 89.24 | 86.02 |

For BB error, there is a minor degradation (~2–3% drop across K) compared with no error case. And for RF error, the severe impact is observed (e.g., Top-1 drops to 40%, ~28.7% absolute reduction). When both BB error and RF error are considered, it is slightly worse than RF error-only, suggesting RF dominates prediction performance. For the K value, higher K improves resilience (e.g., Top-5/1 retains 86.02% accuracy under combined errors).

<simulation results on Top-K/N >

In Table 5, we show the corresponding results on the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams (Alt 1,where m=1). As can be seen from the results of simulation#1.

Table 5 Evaluation results of prediction accuracy on Top K/N for BM-Case1 DL Tx beam prediction by considering measurement error (m=1)

| Simulation results | Prediction accuracy (%) |  |  |  |
| --- | --- | --- | --- | --- |
| Simulation#1: the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams | No error | case 1 | case 2 | case 3 |
| Top 2/2 | 92.18 | 91.57 | 79.36 | 77.67 |
| Top 2/4 | 98.24 | 97.51 | 90.12 | 89.00 |
| Top 3/2 | 94.17 | 93.33 | 85.22 | 83.69 |
| Top 3/4 | 98.48 | 98.07 | 93.09 | 91.81 |
| Top 4/4 | 98.80 | 98.47 | 95.26 | 93.17 |
| Top 5/2 | 98.00 | 97.83 | 93.82 | 90.76 |

It can be observed that, when the Top-N genie-aided beam is one of the Top-K predicted beams, the prediction becomes easy (most of the prediction accuracy is above 95%) even if measurement error is considered. This may make the requirement too relaxed, so that even if this KPI is met, it does not fully represent the true quality of these beams is good.

Besides, to explore the suitable value of m, we further simulated the case of m=2, corresponding to simulation#2 on the percentage of the Top-N genie-aided beam is two of the Top-K predicted beams.

Table 6 Evaluation results of prediction accuracy on Top K/N for BM-Case1 DL Tx beam prediction by considering measurement error (m=2)

| Simulation results | Prediction accuracy (%) |  |  |  |
| --- | --- | --- | --- | --- |
| Simulation#2: the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams | No error | case 1 | case 2 | case 3 |
| Top 2/2 | 72.15 | 69.56 | 43.29 | 43.69 |
| Top 2/4 | 92.42 | 91.65 | 76.71 | 76.39 |
| Top 3/2 | 86.59 | 85.54 | 69.40 | 69.48 |
| Top 3/4 | 95.29 | 94.78 | 85.38 | 84.90 |
| Top 4/4 | 96.33 | 95.98 | 79.52 | 87.47 |
| Top 5/2 | 92.74 | 91.73 | 83.61 | 81.45 |

It can be seen that when m=2, the accuracy drops of ~20–30% vs. m=1. For RF error, in some case, it makes the halve accuracy (e.g., Top-2/2: 72.15% → 43.29%).

Observation 6: Compared with the KPI of “the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams”, the KPI of “the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams” may better reflects beam reliability of the reported beams

<simulation results on the successful rate of maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – 1 dB >

In Table 7, we show simulation results on the successful rate of maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – 1 dB. From the simulation result, it can be observed that compared with RF error, there is minimal degradation (~0.5–1% drop across K) with consideration of BB errors and it have negligible impact on this KPI. When both RF and BB errors are considered, the performance is Similar to RF-only, suggesting RF error dominates the impact on the prediction accuracy.

Table 7 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results | Prediction accuracy (%) |  |  |  |
| --- | --- | --- | --- | --- |
| maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – 1 dB | No error | Case 1 | Case 2 | Case 3 |
| K=1 | 90.42 | 89.96 | 73.17 | 74.22 |
| K=2 | 91.94 | 91.24 | 82.97 | 82.25 |
| K=3 | 92.82 | 92.13 | 85.62 | 84.98 |
| K=4 | 94.65 | 94.22 | 88.84 | 88.03 |
| K=5 | 97.61 | 97.27 | 93.57 | 93.17 |

Observation 7: For the metric- maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – 1 dB, there is minimal degradation when only BB error is considered. When both RF and BB errors are considered, the performance is similar to RF error-only, suggesting RF error dominates the impact on this KPI for prediction accuracy

- Testability issues for beam management

3.1 Channel model

In the RAN4#114 meeting, the following agreements about the channel models are captured

Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

As we discussed in the previous meeting, a flat channel (e.g., TDL channel model) could not reflect the real performance of AI model and channel models with spatial characteristic need to be emulated in the test system. Thus, RAN4 should at least focus on the CDL channel as the test environment. However, CDL channel with multi-clusters are too complicated in the whole sphere space to emulate. Fig. 1. shows the power angular spectrum (PAS) of AoA and ZoA of CDL-A, CDL-B, CDL-C channel model with NLOS clusters in a sphere space.

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Fig. 1 Illustration of PAS of CDL-A, CDL-B and CDL-C channels

Based on the theory of prefaded signal (PFS) method to emulate the channel in OTA chamber, a great number of probes and high-number ports channel emulator will be implemented, which is not feasible for vendors. Thus, RAN4 needs to find a way to simplified the test system. For option 5, it is a feasible way to simplified the CDL channel model with a small number of clusters, but the test dependability will degrade. Based on the AI BM characteristic, we could have other feasible way and here is the analysis.

Observation 8: The simplified CDL channel with small number clusters will degrade the test dependability.

3.1.1 Feasible way to simplify the test system

UE always recognize the AoA direction of the strongest power path as the reference path for beamforming. When some certain different Tx beams are sweeping and transmitted in the channel environment, the AoA of the strongest power path will not change while the power is still changing. Fig. 2 shows an example of the barlett power spectrum for AoA estimation with different Tx beams in fading channel environment, where the UE is assumed as a 4×4 antenna array. It can be observed that at least for beam sets 2-3, 5-10, 13-14-… the AoAs of the strongest path in UE side are the same.

Fig. 2. Bartlett power spectrum for AoA estimation with different Tx beams

Based on the above understanding, some Tx beams can be grouped together (e.g., beam 5-10) if the strongest receive power AoA are the same. Besides, there are different ways to group the Tx beams and needs to be studied. For example, setting a range for AoA of the strongest path and using this range to group Tx beams. Strongest path AoA based grouping is just an example for introducing this theory. An illustration of the Tx beam grouping is shown in Fig. 3.

Fig. 3. Tx beams grouping

Since only AoA of the signal needs to be emulated in the anechoic chamber while different Tx beams in one group are corresponding to only one AoA, which means that only one probe with one AoA to emulate multiple Tx beams in one group is feasible, and the number of groups equals to the number of probes. TE only needs to change the power of the specific probe in the specific time to emulate the power of the different Tx beams transmitted through fading channel, which makes the test system much easier. Fig. 4. shows an illustration of multi-probe anechoic chamber test system with the Tx beam grouping method. In this MPAC system, only power and direction need to be emulated which is applicable for AI/ML based BM case.

In Fig. 4., the probe 1, 2, …, L is for emulating the Tx beams in group 1, 2, …, L, respectively. Only one probe is allowed to be active at the same time for transmitting the beams in the corresponding group. The Tx beams are realized in BS emulator and the fading channel is realized in channel emulator (CE). When the probe 1 is active and probe 2 to L are deactive, probe 1 could emulate the same AoA and different power of different beams in group 1, which are the basic elements for AI/ML based BM testing (L1-RSRPs and spatial characteristics).

Fig. 4. Illustration of MPAC test system with Tx beam grouping for emulating AoA and power

Besides, if other channel characteristics needs to be reconstructed in quiet zoom, the test system in Fig. 5 can be referred to reconstruct the channel environment, e.g., spatial correlation, PAS, etc. However, more probes will be needed in this MPAC system.

In Fig. 5, the probe set [1, 2, …,K] is for emulating the Tx beams in group 1, and another set […, i, j,…] is for emulating the Tx beams in group 2, and so on. Only one set of probes is allowed to be active at the same time for transmitting the beams in the corresponding group. When the probes in probe set [1, 2, …,K] are active and other probe sets are deactive, probe set [1, 2, …,K] could emulate the spatial correlation, PAS. This test system could emulate a more accurate environment that the test system in Fig. 5., but more probes are needed.

Fig. 5. Illustration of MPAC test system with Tx beam grouping for reconstructing channel environment

Therefore, following procedures can be used for simplification:

Step 1. Generate the BS beamforming power pattern of every Tx beam

Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

Step 5. Using one or small number of probes to emulate one beam group.

Step 6. Delete the unnecessary probes and merge some adjacent probes.

Observation 9: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 10: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 5: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 6: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 7: RAN4 to consider the following two approaches for design the test system

approach a

approach b

3.2 Test setup

In RAN4#114 meeting, the following agreements have been achieved.

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

For this issue, we provide some simulation results for the OTA probe setup under the NLOS channel CDL-A, B, C based on the method, where the 128 Tx beams set A beamforming characteristics are given as:

Azimuth angle in degree: [-84.375 -73.125 -61.875 -50.625 -39.375 -28.125 -16.875 -5.625 5.625 16.875 28.125 39.375 50.625 61.875 73.125 84.375]

Zenith angle in degree: [81 84 87 90 93 96 99 102]

The power patterns of the 128 Tx beams in azimuth cut and elevation cut (elevation angle in degree = zenith angle in degree – 90degree) are shown in Fig. 6.

(a)

(b)

Fig. 6 BS beamforming power pattern of 128 Tx beams

CDL-A

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 7 Probe allocation for CDL-A

CDL-B

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 8 Probe allocation for CDL-B

CDL-C

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 9 Probe allocation for CDL-C

Based on the probe allocation in Fig 7-9, it can be seen that some allocated probes are really close in angle-domain and can be merged and replaced by only one probe. Figure 10 shows an example for how to merging the probes in angle domain.

(a) original probe allocation for CDL-A, B, C (red: CDL-A, blue: CDL-B, green: CDL-C)

(b) new probe allocation after UE rotation in zenith angle domain

(c) 5 probe allocation after merge considering UE Rx resolution

Fig. 10 probe allocation after merge and rotation

In Fig. 10 (a), the allocated probes for CDL-A, B, C are illustrated and the total number is 31, which is not feasible. However, test should be performed under different channel models and no need to consider channel model switch (e.g., CDL-A to CDL-B) in one test. Thus, we can rotate the channel (rotate DUT in 3D-turnable) to narrow the channel model range in angular domain, which is shown in Fig. 10 (b). In Fig. 10 (b), all the channels are rotated in zenith angular domain and probes are symmetry about zenith angle = 90°. Besides, UE Rx beamforming has its resolution in angular domain, e.g., 30 deg. Fig. 10 (c) shows an example of probe allocation when considering UE Rx 30 degrees resolution, which eventually provided 5 candidates probe locations and greatly decreases the cost of the test system.

Proposal 8: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Proposal 8: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- Summary

Proposal 1: On the definition of prediction delay requirements for BM, RAN4 to consider:

- For Measurement for prediction, the current measurement period for L1-RSRP measurement can be the baseline and further discuss the details by considering the parameters, which at least include:

  - The number of samples for set B measurements (for spatial domain prediction); The number of the latest measurement instances and the time interval between two adjacent time instances (for time domain prediction)

  - beam sweeping factor

  - sharing factor N

- For inference delay: the reference latency and possible processing time

Proposal 2: For the UE Rx beam knowledge,

- at least for the case that the Top-K beams to be reported are in set B and set B is a subset of set A, if beam sweeping has been done during measurements on set B, UE Rx beam information is known by UE and no further beam sweeping is needed.

- for other cases, e.g., set B is not a subset of set A, RAN4 to further discuss how to obtain Rx beam information, e.g., introducing UE capability or making the reduced beam sweeping factor assumption

Proposal 3: For performance monitoring for BM, RAN4 to first determine the testing goal. The following alternatives can be considered:

- Option 1: The testing goal is to verify whether the performance metrics can be correctly calculated by UE;

- Option 2: The testing goal is to verify whether UE can perform monitoring and report the performance metrics within a certain latency upon performance monitoring is required;

- Option 3: The combination of Option 1 and Option 2.

Observation 1: There is a certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth of RSRP.

Observation 2: For RSRP Absolute accuracy, if the index i can be any beam index in top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for those beams which belong to Top-K predicted beam but not any of the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Proposal 4: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i can be any beam index in top-K beams based on predicted L1-RSRP.

Observation 3: RF errors are more detrimental than BB errors, especially for beam-specific predictions (comparision between predicted beam and acutal beam with the same ID ).

Observation 4: Larger K improves resilience but may not compensate for RF-dominated errors (e.g., Case 3 in Top-5 still shows >6 dB difference).

Observation 5: The KPI of Top K/N mitigates the effects that the RSRPs between Top 1 and Top 2 genie-aid beams are too close (within 0.5dB), which may cause the actual Top-1 beam may be indistinct

Observation 6: Compared with the KPI of “the percentage of the Top-N genie-aided beam is ONE of the Top-K predicted beams”, the KPI of “the percentage of the Top-N genie-aided beam is TWO of the Top-K predicted beams” may better reflects beam reliability of the reported beams

Observation 7: For the metric- maximum ideal RSRP among top-K predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – 1 dB, there is minimal degradation when only BB error is considered. When both RF and BB errors are considered, the performance is similar to RF error-only, suggesting RF error dominates the impact on this KPI for prediction accuracy

Observation 8: The simplified CDL channel with small number clusters will degrade the test dependability.

Observation 9: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 10: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 5: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 6: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 7: RAN4 to consider the following two approaches for design the test system

Proposal 8: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- References

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm

- R4-2503021, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT