3GPP TSG RAN WG4 Meeting #114-bis	R4-2504027

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.28.2

Source:	vivo

Title:	Further discussion on RRM requirements for XR

Document for:	Discussion

# Introduction

In the RAN4#114 meeting, there were further discussions on RRM requirements impact for XR. Following agreements were made and captured in the WF [1].

< Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported. <Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension. <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [2] was sent to RAN4 to trigger further discussions in RAN4.

Later on, RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the last RAN4 meeting mentioning that RAN4 is discussing other solutions.

In [5, 6, 7], we provided our views on the RRM requirements impact and issues raised in the RAN1 LSs. In this contribution, we further provide our views on RRM requirements impact for XR in Rel-19 and questions raised in the RAN1 LSs.

# RRM requirements impact

## 2.1 Scope

Deployment scenario

In the last RAN4 meeting, it was agreed that EN-DC and NE-DC for DCI-based solution is not supported for measurement gap skipping. It is FFS for NR-DC.

If a gap/restriction occasion is cancelled by MN via DCI, it may not be able to inform SN timely. At least, There are no RAN2 discussions on this so far. It means the SN cannot schedule the UE during the gap occasion being cancelled. Thus, benefit to support the feature is compromised in DC scenarios.

From RAN4 perspective, we may not define requirements for NR-DC scenarios, or we send an LS to RAN2/RAN1 to confirm whether it is supported or not.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

Measurement gap type

In the RAN4#113 meeting, following agreement was made.

<Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

In the last meeting, it was agreed that only gap-based measurement skipping is supported. In any case corresponding requirements for gap skipping with Type 1 measurement gap will be defined. It may need a bit discussion whether additional gap type is to be considered.

Following type gap types are under discussion.

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

Type 1 MG is the baseline from Rel-15 and it should be supported. It includes both per-UE gap and per-FR gap configured via GapConfig.

The Type-2 MG, however, is not so clear. Whether it is a single gap configured via GapConfig-r17 or it can be multiple gaps. If it is the latter one, then what would be the difference between type-2 MGs and concurrent gaps. If it is the former one, then it should be supported as there is no difference compared to Type-1 MG in terms of gap skipping.

Concurrent gaps are quite complicated. Moreover, whether concurrent gaps with NCSG/pre-configured gaps are included or not should be clarified. It may not be necessary that NW configures concurrent gaps when XR service is enabled. It is preferred not to consider concurrent gaps in Rel-19.

For NCSG, in general there would be no difference compare to Type 1 MG or Type 2 MG from scheme of triggering gap skipping perspective. However, the interruption ratio due to VIL is relatively small, which is 2ms in FR1 and 1.5ms in FR2 within one visible interruption repetition period. In typical measurement gap configuration, 6ms MGL with 40/80ms MGRP would be configured for the UE. The interruption rate due to measurement gap is up to 15%. If 20ms MGRP is considered in FR2, the interruption rate is up to 30%. However, for NSCG the interruption rate can only be up to 7.5% for 20ms MGRP in FR2. Considering the possibility of collision between XR traffic and NCSG, the impact to XR service is very small. Thus, there is no need to consider NCSG when gaps are to be skipped.

Pre-configured measurement gaps are similar to Type-2 MG when it is activated which can be triggered by UE autonomous control or network configuration. When it is not activated, it is the same as no measurement gaps are configured. However, the timeline relationship between pre-MG activation/deactivation and gap skipping indication should be carefully considered.

Positioning measurement gaps are not clear by itself. In Rel-16, legacy per-UE gap is used for PRS measurements. When RRM measurements also need gaps, then gap sharing between RRM and PRS measurements is used. In Rel-17, per-FR gap can also be used for PRS measurements. Additionally, pre-configured MG by PosGapConfig-r17 is also supported for PRS measurement which can be activated via DL MAC CE or UL MAC CE depends on UE capability. If positioning measurements are based on legacy measurement gap, then it would be naturally be considered. For Rel-17 pre-configured MG for positioning, it is proposed not to consider it.

MUSIM gaps is for operations in a different network which can minimize impact under current network operation. If network believes XR service is more import it can reject to allocate MUSIM gaps when it is requested by UE. Similarly, if a device believes XR service is more import, it may not request MUSIM gaps at all. There is no need to specify complicated solutions to skip MUSIM gaps.

Proposal 2: Similar to type 1 MG, Type 2 MG can be considered if only per-UE gap or per-FR gap are configured.

## 2.2 RRM requirements

In the last meeting, it was agreed to use NR-U-like approach to define requirements for gap skipping when extended measurement delay is needed.

- Follow NR-U-like approach for designing the formulas for measurement delay extension.

How the delay is extended has not been decided yet.

<Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases.

The difference between option 1 and option 2 is whether the number of skipped occasions should be multiplied by CSSF. In principle, the measurement delay is specified for each inter-frequency layer measured with gaps. With option 1, the same number of gap occasions in which the frequency layer can be measured are given. Thus, regardless of which frequency layer is supposed to be measured in the skipped occasions, same number of additional gap occasions can be used for the measurements so that UE can complete the measurements without compromising accuracy.

Proposal 3: Option 1 is used to define requirements for gap skipping.

Regrading conditions for measurement skipping, following was discussed.

<Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Since NR-U like requirements is defined for measurement gap skipping, it is necessary to limit the max number of gaps skipped during the measurement period. Otherwise, the mobility performance cannot be ensured.

Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

Impacted clauses

In the last meeting, the impacted clauses for requirements due to gap skipping were identified and captured in the WF[1].

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |

However, it seems the requirements for LTM measurement are missing. In the last meeting, it was agreed that gap skipping are for both L3 measurements and L1 measurements.

- During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions.

Therefore, requirements in clause 9.15	NR inter-frequency L1 measurement should also be updated for gap skipping.

Proposal 5: Requirements in clause 9.15 NR inter-frequency L1 measurement are updated for gap skipping.

A companion draft CR [8] is provided.

# UE assistance information

In the last RAN4 meeting, it was agreed to define UAI and proposals on UAI were also captured in the WF [1].

<Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

There are also serval aspects about UE assistance information that had been discussed in RAN1 as captured in the LS [2].

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

For the L3 measurements within gaps, following examples is used for illustration of what UAI is needed.

For the example of measurement configuration depicted in Fig 1, measurement gap is configured together with one inter-frequency layer. For this configuration, CCSFwithin_gap,i is 1 for the frequency layer. The UE needs all of the measurement gaps to meet the requirements.

Fig 1. Configuration of measurements on one inter-frequency layer with gaps

In this case, when some of the gaps are used for data scheduling the UE cannot meeting existing measurement requirements. UE needs additional gap occasions to get enough samples to meet the accuracy requirements. Since this is a very simple case, measurement delay can just be extended by number of skipped gap occasions.

For the example of measurement configuration depicted in Fig 2, measurement gap is configured together with three inter-frequency layers where different SMTC periodicity is used. For this configuration, CCSFwithin_gap,i is 3 for all the frequency layers.

Fig 2. Configuration of measurements on multiple inter-frequency layers with gaps

In this case, some of the gaps may not always be used to meet minimum measurement period requirements. The requirements are assumed that there are always 3 candidates of MOs to be measured within a gap. For MO#1 and MO#2, it is not the case. So, some of the gaps could be left unused as long as measurement period requirements are satisfied.

Suppose 2 samples are needed for one measurement period. one of the possible measurement implementations is depicted in Fig 3. Another possible measurement implementation is depicted in Fig 4.

Fig 3. Measurement implementation example 1

Fig 4. Measurement implementation example 2

Based on above analysis, it is observed that some of the gaps may not be used for measurements and it is up to UE implementation which gaps are not used for measurements.

Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

In the case as depicted in Fig 2, Fig 3 and Fig 4, the unused gaps are about 37.5% (9/24). It is significant gain if these gaps can be used for data transmission/reception. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. Therefore, there is obvious benefit if UE can provide assistance information about which gaps are not used for measurements.

Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Moreover, for some advanced UE, it may not use all the gaps during the measurement period specified by minimum requirements. For example, measurement period is based on 8 samples for FR1 inter-frequency measurements. The advance UE may just use 5 of them. So, the UE either not to use some of the gaps or use all the gaps with shorter measurement period. If the former case, then the gaps not used for measurement can be reported to network to schedule the UE during the gaps. Even for the example in Fig 1, there could be unused gaps due to advanced UE implementation.

If there are enough unused gaps and it is used for data scheduling there is no impact to measurements. For this case, UE assistance information is better introduced. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided.

Proposal 6: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Then it comes the question that the reported gaps not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no gaps not used for measurement depending on measurement configuration. However, when enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, it needs to keep impact to mobility performance limited. In order to achieve this goal, it needs UE to provide information of gaps not used for measurement. If it is totally up to NW indication, the mobility performance could highly be degraded as gaps may have different priority for measurements, e.g., the most loaded gap has highest priority and it may be used to measure sparse reference signals and should not be used for data transmission/reception, or if gap sharing scheme is not equal sharing, some gaps may have high priority to meet requirements.

One possible solution is that NW indicates the percentage of gaps not used for measurement during a period of time. It is up to UE to decide which gaps are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 7: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement so that UE can provide corresponding gap patterns that meet the percentage.

In addition to gap patterns, other UAIs can also be considered.

Similar to NR-U requirements, the max number of gaps skipped during the measurement period is better captured in the requirements as discussed in section 2.2. Besides, the maximum interval between two measurement occasions available for RRM measurements can be defined as UAI. If the interval is too long, the measurement results may not reflect the actual signal quality.

Proposal 8: The maximum interval between two measurement occasions available for RRM measurements can be defined as UAI.

When DRX is configured, the UE may measure once within gaps in one DRX cycle. Only the gaps occasions overlapped with DRX on duration can be used data transmission/reception. There could be different reporting scheme for UE assistance information, e.g., only reporting gaps overlapped with DRX on duration within a time period.

Proposal 9: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Regardless of the gaps not used for measurements is due to measurement configuration or advanced UE implementation, it is more like UE capability rather than UE assistance information. It is better that RAN4 to discuss which scheme should be used. The difference is that NW will follow UE reporting if it is UE capability, but the NW may not follow UE reporting if it is UE assistance information depending on current specification. At least for the case where measurement is not impacted, it is better to report as UE capability similar to “NeedForGaps”.

Proposal 10: The information about gaps not used for measurement are reported as UE capability rather than UE assistance information.

After RAN4 makes agreement on the UAI, an LS should be sent to RAN2 on the details of the UAI so that RAN2 can design the signaling.

Proposal 11: Send an LS to RAN2 and RAN1 on RAN4 agreements about UE assistance information.

# Solutions for enabling Tx/Rx in gaps/restrictions

Indication solution

RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the last RAN4 meeting mentioning that RAN4 is discussing other solutions.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

In the RAN1#118 meeting, RAN1 concluded details of DCI-based explicit indication for gap skipping.

Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. Agreement A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

For explicit indication by DCI, there are two options how UE would interpret different values for a gap skipping indication. UE only needs to support one option. As the example below, it could mean completely different UE behavior even if the same value is indicated by the DCIs.

Fig 5. Example of explicit DCI indication options

For option 2, UE needs to decide if a gap can be skipped by the latest DCI indication at least 5ms ahead of the start of the gap. For option 1, UE needs to decide if a gap can be skipped by the DCI indication during the gap period and 5ms ahead of the start of the gap. If there is DCI indication with value ‘1’, then the gap shall be skipped. For the example in Fig 5, the two gaps shall be skipped if it is option 1 but it shall not be skipped if it is option 2.

For both options, UE needs to decide if a gap can be skipped by monitoring DCIs contiguously. This complicates UE implementation as the UE needs to re-schedule measurements after a gap skipping. More importantly, it leaves UE less possibility to optimize measurements based on measurement configurations which will degrade mobility performance.

Observation 4: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

However, if gap skipping is explicit indicated by RRC it could give UE flexibility to optimize UE measurement. For example, if UE reports gap patterns that can be skipped via UE capability and NW just to indicate those gaps then it is the best to indicate via RRC with the reported gap patterns. With this, there is no impact to measurement requirements and UE doesn’t need to re-schedule the measurements when a gap is skipped. So, at least RRC based signaling should be supported for such cases.

Multiple options for semi-static schemes were proposed in the last RAN4 meeting.

<Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

Given the complexity of DCI triggering scheme and benefit of RRC based signaling, semi-static solutions should be introduced.

Proposal 12: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

Moreover, for DCI-based triggering scheme, it is expected that some restrictions should be put on the dynamic indication. For example, the gaps to be skipped during a time period is indicated by RRC in advance. The DCI indicated gaps are only selected from those configured by RRC. This will largely simplify UE measurement scheduler which only needs to be conducted after RRC reconfiguration and ensure mobility performance is not degraded too much.

Proposal 13: RAN4 agrees that for DCI-based triggering scheme, the gaps to be skipped during a time period is indicated by RRC in advance.

RAN1 designed a uniform triggering mechanism for both gap skipping and restriction skipping. However, RAN4 agreed in the last meeting that only gap skipping is considered in the WI. It also needs to inform RAN1 of the conclusion on this.

Proposal 14: RAN4 LS to RAN1 and RAN2 the agreements on introduction of RRC-based triggering scheme and early indication for DCI-based triggering scheme, and additionally on only gap skipping is considered in the WI.

Per-FR gap

Regarding skipping indication on frequency domain, following proposals were presented in the last meeting.

- Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

RAN1 made following agreements on DCI indication to skip a gap occasion.

Agreement When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and - if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field. - if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

Based on whether PDSCH is scheduled or not, and whether CIF filed is present or not, the first gap occasion to be skipped is for what serving cells are clear. Eventually, frequency range is also clear based on the determined serving cells.

Proposal 15: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

# Summary

In this contribution, we further provided views on RRM requirements impact for XR in Rel-19, UAI and triggering schemes. Following proposals and observations are present.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

Proposal 2: Option 1 is used to define requirements for gap skipping.

Proposal 3: Similar to type 1 MG, Type 2 MG can be considered if only per-UE gap or per-FR gap are configured.

Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

Proposal 5: Requirements in clause 9.15 NR inter-frequency L1 measurement are updated for gap skipping.

Proposal 6: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Proposal 7: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement so that UE can provide corresponding gap patterns that meet the percentage.

Proposal 8: The maximum interval between two measurement occasions available for RRM measurements can be defined as UAI.

Proposal 9: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Proposal 10: The information about gaps not used for measurement are reported as UE capability rather than UE assistance information.

Proposal 11: Send an LS to RAN2 and RAN1 on RAN4 agreements about UE assistance information.

Proposal 12: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

Proposal 13: RAN4 agrees that for DCI-based triggering scheme, the gaps to be skipped during a time period is indicated by RRC in advance.

Proposal 14: RAN4 LS to RAN1 and RAN2 the agreements on introduction of RRC-based triggering scheme and early indication for DCI-based triggering scheme, and additionally on only gap skipping is considered in the WI.

Proposal 15: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 4: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

- References

- R4-2502700	WF on RRM requirements for NR_XR_Ph3, Nokia

- R1-2405736	LS on UE assistance information, RAN1

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

- R4-2420198	Response LS on gaps/restrictions that are caused by RRM measurements, RAN4

- R4-2415198	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2418759	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2500827	Further discussion on RRM requirements for XR, vivo

- R4-2504029	draft CR on NR inter-frequency L1 measurement for XR, vivo

- Appendix: Draft reply LS on UE assistance information

3GPP TSG RAN WG4 Meeting #114-bis	R4-25x

Wuhan, China, 7th – 11th April, 2025

Title:	Reply LS on UE assistance information

Response to:	R1-2405736 (R4-2411007)

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information.

RAN4 discussed the UE assistance information for reporting not used measurement gas/SMTC occasions to network. Following conclusions are made.

RAN4 agreed to introduce following [UE assistance information / UE capability]:

- The patterns of gaps occasions not used for measurement within a time period.

- The maximum interval between two measurement occasions available for RRM measurements

RAN4 will further discuss and determine the details of the UAI/UE capability and provide update to RAN1 and RAN2.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above RAN4 agreements into account in the future work and design the singalling.

3. Reference:

- R1-2405736	LS on UE assistance information, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta

- Appendix: Draft LS on indication solution for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #114-bis	R4-25x

Wuhan, China, 7th – 11th April, 2025

Title:	LS on indication solution for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 sent a reply LS R4-2420198 in the last RAN4 meeting with following statement.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

RAN4 further discussed semi-static indication solutions and made following agreement.

- RRC based indication solution is supported in addition to DCI-based indication solution. It is up to UE which solution is supported.

- Gap patterns during a period for gap skipping is indicated in RRC for DCI-based indication solution.

RAN4 would like to kindly ask RAN2 to introduce necessary RRC signaling. RAN4 will further study how the gap patterns are specified.

Additionally, RAN4 agreed to only support gap skipping in the WI.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to introduce necessary RRC signalling for gap skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta

- Appendix: Draft LS on scenarios for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #114-bis	R4-25x

Wuhan, China, 7th – 11th April, 2025

Title:	LS on scenarios for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 discussed deployment scenarios for gap/restriction skipping. RAN4 made agreements that NR SA and NR carrier aggregation is supported from RAN4 requirements perspective. EN-DC and NE-DC are not supported. It is FFS for NR-DC

If a gap occasion is skipped by MN via DCI indication, there may not be information exchange between MN and SN. It means the SN cannot schedule the UE during the gap occasion cancelled. Thus, benefit to support the feature in DC scenarios is compromised.

RAN4 would like to ask RAN2 and RAN1 if NR-DC scenario is well supported for gap/restriction skipping.

2. Actions:

To: RAN WG2, WG1

ACTION: 	RAN4 kindly asks RAN2 and RAN1 to provide feedback on supporting of NR-DC scenarios for gap/restriction skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta