3GPP TSG RAN WG4 Meeting #114-bis	R4-2504028

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.7.3

Source:	vivo

Title:	Further discussion on RRM requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the RAN#106 meeting, a new WI [1] on low band carrier aggregation via switching was approved. There are deployment constraints that could impact RRM requirements.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In the RAN4#114 meeting, RAN4 discussed RRM requirements for low band CA via switching. Following agreements were made and captured in the WF [2].

Issue 1-1-1: RRM work scope - Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion. - Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

In this contribution, we further provide our views on RRM requirements impact due to low band carrier aggregation with switching.

# Discussion

Carrier aggregation is the important feature to utilizing spectrum owned by operators. However, for some FDD band combinations, it is challenging to implement by the UE considering complexity, cost and feasibility of RF front architectures. To address operators’ concern, a new WID [1] was approved to specify switching scheme.

According to the WID [1], the following two cases are considered.

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

When UE works on FDD carrier 1, which should be PCell, the UE doesn’t need to receive on SDL carrier 2. If UE switches to SDL carrier 2 to received, then the UE doesn’t need to receive/transmit on FDD carrier 1. However, when UE switches between the two carriers, there would be switch period during which the UE cannot receive/transmit on either one of the carriers.

Based on the WID objective, only a semi-static switching pattern based on RRC configuration will be introduced. It is not clear for now how this semi-static switching pattern will be specified. RAN4 may discuss the impact of switching pattern based on certain assumptions.

Interruption requirements

Similar to UL Tx switching, switching period for RF retuning is also needed for DL switching. The switching period will be decided in RF session. Additionally, from RRM perspective receive timing difference should also be considered when defining RRM requirements. Since both carriers are in a single TAG and co-located, there is no need to consider impact of TA. Then only TAE of BS synchronization needs to be considered, which may be 4.7us or 3us based on agreements in the last meeting.

Then based on different timing difference between the two cells. The switching between the two carriers can be illustrated in Fig 1 and Fig 2, respectively. It is assumed that switching is performed on SCell carrier, which will be determined in RF session.

Fig 1. Switching between PCell and SDL Cell

Fig 2. Switching between PCell and SDL Cell

In the last RAN4 meeting, RF session agreed switching periods between FDD carrier and SDL carrier.

- Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us

During the switching period, the UE should not be scheduled on PCell (assuming the switching is conducted on SDL carrier) which would be captured in TS 38.101-1 similar to Tx switching. Since band combination in this WI is CA_n5A-n29A, there is no other carriers can be scheduled during the switching period. Furthermore, FDD carrier and SDL carrier will not operate at the same time. Therefore, there is no necessity to define interruption requirements in RRM spec.

Proposal 1: No need to define interruption requirements for carrier switching between FDD CC and SDL CC.

SDL carrier operation related requirements

For SDL carrier operation, L3 measurement for activated/deactivated SCell, SCell activation/deactivation delay, L1 related procedures, e.g., BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement need to be discussed from RRM requirements perspective.

Before discussing details of these requirements, some assumptions regarding the overall operation of LB CA via switching should be made. First of all, the switching pattern plays a critical role. As depicted in Fig 1. and Fig 2., symmetric and asymmetric switching patterns may be considered. The length of time period (X and Y) is critical which can have different impact on RRM requirements.

There are serval aspects that can impact the switching pattern design. One of the aspects is HARQ process. According to existing HARQ design, the maximum of K0 is 32ms and maximum of K1/K2 is 15ms. If HARQ process is enabled, the dwell time Y on SDL carrier cannot be longer than 15ms. The dwell time X on PCell carrier can be longer.

Based on dwell time X and Y, there may not be always SSB being available on SDL carrier. Thus, the requirements for activated and deactivated SCell measurement could be very complicated due to unavailability of SSBs on the measurement occasions. Furthermore, there is no mobility on SDL carrier. The FDD PCell and SDL SCell is collocated so that the SCell change would be together with PCell. Thus, the measurement would be only for activation of SCell on SDL carrier, i.e., the SCell can be known for activation if the measurement is performed not long ago.

If the SDL SCell is known, depending on how long ago the SCell is measured, e.g., <2400ms or > 2400ms, the UE needs one or two SSBs for SCell activation. If the SDL SCell is unknown, then very long time is needed for the SCell activation where L1-RSRP measurement need to be conducted for acquisition of QCL and TCI state information.

Thus, to make the SCell activation more efficient, it would be better to assume certain QCL relationship between SSBs on the PCell carrier and SDL SCell carrier. The SSBs on SDL carrier is supposed to be QCLed with SSBs on PCell carrier. It can also assume that timing information of PCell is directly used for SCell where fine timing tracking is not needed during SCell activation. With the assumptions, UE still need one or two SSBs for AGC depending on whether the SCell is known or not.

Furthermore, it is also beneficial to support inter-band SSB-less SCell activation. This will largely reduce SCell activation delay when NW transmits A-TRS after sending SCell activation command.

Proposal 2: Define L3 measurement requirements for deactivated SDL SCell only, where switching pattern should be taken into account additionally.

Proposal 3: No need to define L3 measurement requirements for activated SDL SCell and neighbor cell on the carrier.

Proposal 4: Requirements for SDL SCell activation are defined for known/unknown cases and inter-band SSB-less operation.

Proposal 5: Requirements, at least for SDL SCell activation, are defined for both SSB-based and SSB-less SCell operation.

For detailed requirements for deactivated SCell measurement, it would be better to wait for RAN1 progress. If SDL SCell is deactivated, is it still necessary to have the pattern for switching between PCell and SCell? Maybe not as UE only performs measurements on the deactivated SCell. There could be long overhead, e.g., 15ms every 40ms or so, depending on pattern design. NW may disable the pattern if SCell is deactivated. This needs further RAN1 discussion.

Proposal 6: Wait for RAN1 progress on pattern design for defining deactivated SCell measurement requirements.

For L1 related measurement and procedures, e.g., BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR measurement, the requirements may not be needed in our view. These procedures mainly related to beam management. If QCL relationship can be assumed between FDD PCell and SDL SCell, then these procedures may not be essential to the CA via switching. The QCL information can be obtained based on the same procedures performed on the PCell. Since the FDD PCell and SDL SCell are collocated and dwell time on SDL SCell is small, e.g., < 15ms, it could be possible not to rely the L1 measurement/procedure on SDL SCell.

Proposal 7: RAN4 to further study whether it is feasible to assume SSBs on FDD PCell and SDL SCell are QCL-ed.

Proposal 8: If QCL assumption can be made, there is no need to define requirements for L1 measurement (e.g., L1-RSRP/L1-SINR) / procedures (e.g., TCI state switch, PL-RS switch delay etc.) on SDL SCell.

FDD PCell related requirements

For FDD PCell carrier related operation, all the requirements related to availability of SSB on the carrier need to be discussed. For example, intra-frequency measurement delay may need to be extended due to some measurement occasions are not available because the UE dwells on SDL SCell during the supposed measurement occasion. There are also many L1 measurements (e.g., RLM/BFD/L1-RSRP/L1-SINR)/L1 procedures ((e.g., TCI state switch, PL-RS switch delay etc.) that would be impact due to the switching pattern.

Proposal 9: RAN4 to discuss requirements for FDD PCell operation at least for intra-frequency measurement, RLM/BFD/L1-RSRP, TCI state switch.

Assumptions for R19 LB CA via switching

In the last meeting, it was agreed that the receive timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session, where option 1 is 4.67us and option 2 is 3 us. In existing specifications, the TAE for inter-band CA is 3us. Since FDD PCell and SDL SCell is collocated, there will be no additional timing difference observed at UE. Thus, the receive time difference should be assumed as 3us.

Proposal 10: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

# Summary

In this contribution, we further provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: No need to define interruption requirements for carrier switching between FDD CC and SDL CC.

Proposal 2: Define L3 measurement requirements for deactivated SDL SCell only, where switching pattern should be taken into account additionally.

Proposal 3: No need to define L3 measurement requirements for activated SDL SCell and neighbor cell on the carrier.

Proposal 4: Requirements for SDL SCell activation are defined for known/unknown cases and inter-band SSB-less operation.

Proposal 5: Requirements for SDL SCell operation, at least for SDL SCell activation, are defined for both SSB-based and SSB-less SCell operation.

Proposal 6: Wait for RAN1 progress on pattern design for defining deactivated SCell measurement requirements.

Proposal 7: RAN4 to further study whether it is feasible to assume SSBs on FDD PCell and SDL SCell are QCL-ed.

Proposal 8: If QCL assumption can be made, there is no need to define requirements for L1 measurement (e.g., L1-RSRP/L1-SINR) / procedures (e.g., TCI state activation, PL-RS switch delay etc.) on SDL SCell.

Proposal 9: RAN4 to discuss requirements for FDD PCell operation at least for intra-frequency measurement, RLM/BFD/L1-RSRP, TCI state switch.

Proposal 10: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

- References

- RP-243317 	New WID on low band carrier aggregation via switching

- R4-2502588	WF on RRM requirements for NR_LBCA_Sw, Apple