3GPP TSG RAN WG4 Meeting #114-bis	R4-2504031

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.27.2.1

Source:	vivo

Title:	On RRM requirements for LTM event-triggered L1-RSRP reporting

Document for:	Discussion

# Introduction

In the RAN4#114 meeting, RAN4 discussed RRM requirements for low band CA via switching. Following agreements were made and captured in the WF [2].

#### Issue 2-3: impact of time-to-trigger (TTT) on RRM Agreement: - No need to explicitly capture TTT in core requirements. #### Issue 2-8: L1 filtering assumption in event evaluation Agreement: - RAN4 should start work assuming no L1 filtering. FFS whether to define requirement with L1 filtering assumed, taking into account the progress in RAN1/2. #### Issue 2-9: Impact of UL resource waiting time. Agreement: - For event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. #### Issue 2-10: Impact on other LTM requirements Tentative agreement: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable. - Further discuss how to capture this in RRM spec.

In this contribution, we further provide our views on RRM requirements for event-triggered L1-RSRP reporting in R19 LTM.

# Discussion

Event triggered reporting delay for L1 reporting

RAN4 has been discussed event triggered reporting delay for L1 reporting for several meetings. Following options were agreed in the last meeting.

#### Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

Regarding the how the report shall be triggered, RAN1 and RAN2 have made the following conclusions.

RAN2 Agreements - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Event LTM1 is not defined. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met. - MAC layer handles the event evaluation and measurement report triggering. - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition.

RAN1 Agreements - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS - Note: This does not imply the support of mTRP scenarios

For periodic measurement reporting, if an event is triggered then the UE starts to send measurement reporting periodically. For the first report after condition is met, the UE needs to conduct measurement to get results for both of the cells corresponding to the event. Therefore, maximum L1-RSRP measurement period of both the cells should be used to define reporting delay.

For other measurement reporting, it is similar to first report for periodic measurement reporting. Thus, similar principle can be used to define the measurement reporting delay requirements.

Proposal 1: For the first report after condition is met for periodic measurement reporting, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

Proposal 2: For aperiodic measurement reporting, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

#### Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: (vivo) - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions: - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Per discussions in the last meeting, companies may think this is too much restriction if it is captured in the core requirements. It may be better to consider these conditions in the test. In general, this could be another approach to clarify the conditions for L1-RSRP reporting delay requirements.

Proposal 3: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements:

- No indicate TCI change during the evaluation period.

- No configuration change w.r.t measurement RS during the evaluation period

- After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Reporting criteria

#### Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) #### Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) #### Issue 2-6: how to define requirements for reporting criteria per measurement category Candidate options: - Proposal 1: Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM. (vivo)

In R18, LTM-CSI-ReportConfig is configured under csi-MeasConfig of ServingCellConfig. The max number of report configs is limited by UE capability supportedMaxAperiodic-LTM-CSI-ReportConfig-r18, supportedMaxPeriodic-LTM-CSI-ReportConfig-r18, and supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18, which are reported per-BC. In each LTM-CSI-ReportConfig, only one report type, either periodic, semi-persistent or aperiodic, is allowed. For event-triggered L1 reporting, how to configure this new kind of reporting is subject to RAN1/2 discussion. But in our view, it is highly possible that this new kind of reporting is treated as a new report type, while the max number of report config supported by UE shall be discussed and determined by RAN1/2.

So far, RAN1/2 has made the following agreements about the resource type that can be configured in an LTM event.

RAN2 #126 Agreements - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. RAN2 #127 Agreements - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. RAN2 #127bis Agreements - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). RAN2 #128 Agreements - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

RAN2 #129 Agreements - Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". - Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). - Do not create a new resource configuration just for the event triggered measurements. - Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". - Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. RAN1 #118bis Agreements - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS - Note: This does not imply the support of mTRP scenarios RAN1 #119 Agreements For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s).

RAN1/2 have already agreed to use same RS type for event evaluation. For each event, which type of RS is reported shall be explicitly configured in the report quantity, i.e. either SSBRI or CRI. SSBRI is the only supported report quantity in R18. For R18 gNB triggered reporting, the number of layers can be identified by number of SSB frequency configured for measurement. Hence, even multiple candidate cells with different SSB frequencies is configured in a same resourceSet, UE may perform SSB-based L1 measurement at different time occasions.

The issue would be similar for the event-triggered case. UE may identify the SSB frequency layer based on SSB frequency, and perform L1 measurement according to R18 rules. For LTM3 and LTM5, there is no differentiation between frequency layers, i.e., any L1 measurement results that meet the event-triggered condition may trigger L1 measurement reporting.

Firstly, event triggering and measurement reporting for L1 measurement are introduced. Similar to that for L3 measurement, it is better to define corresponding capabilities so that NW and UE can understand each other how many events triggering and reporting can be configured and supported.

For L3 measurement, capabilities for event triggering and reporting are defined as below.

#### 9.1.4.1 Introduction This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 9.1.4.2, the UE shall meet all other performance requirements defined in clause 9 and clause 10. The requirements in this clause also apply for a UE in EN-DC with PSCell on a carrier frequency with CCA or SA NR with PCell on a carrier frequency with CCA. The UE can be requested to make measurements under different measurement identities defined in TS 38.331 [2]. Each measurement identity corresponds to either event-based reporting, periodic reporting, or no reporting. In case of event-based reporting, each measurement identity is associated with an event triggering criterion. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion. The purpose of this clause is to set some limits on the number of different event triggering, periodic, and no reporting criteria the UE may be requested to track in parallel. #### 9.1.4.2 Requirements In this clause a reporting criterion corresponds to either one event (in the case of event-based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1..

The requirements count criteria for event-based reporting, periodic reporting and no reporting. All the reporting is related to measurement identities. For L1 measurement, there is no measurement identities. Thus, it is okay to define a new UE capability for event triggering and reporting for LTM. However, it needs further discussion whether UE needs to share the capabilities between L3 measurements and L1 measurements.

Regarding how the UE capabilities are defined, RAN4 starts to work on SSB-based L1 measurements for LTM firstly. In general, intra-frequency measurement and inter-frequency measurement capabilities can be defined similar to L3 measurements. For CSI-RS based L1 measurements for LTM, there are unresolved issues that could impact the UE capabilities for reporting. It can be discussed after the relevant issues being resolved.

Proposal 4: RAN4 to introduce requirements for reporting criteria for LTM and define a separate capability for supported number of L1 measurement reporting.

Proposal 5: Requirements for sharing the capabilities for reporting between L3 measurements and L1 measurements should also be considered.

Proposal 6: RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM firstly. For CSI-RS based L1 measurements for LTM, the reporting criteria can be discussed after critical issues are resolved.

Others

#### Issue 2-7: whether and how to coordinate with R19 MIMO Candidate options: - Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (Nokia) Optimization such as joint consideration of UE capabilities can be discussed at later stage. (Apple, CMCC, HW, CTC) #### Issue 2-8: L1 filtering assumption in event evaluation Agreement: - RAN4 should start work assuming no L1 filtering. FFS whether to define requirement with L1 filtering assumed, taking into account the progress in RAN1/2.

RAN4 doesn’t need to consider to define requirements for two features developed in the same release.

Proposal 7: RAN4 to develop requirements for event triggered reporting for LTM and MIOM independently. No optimization is considered in this release.

In RAN1, there is no consensus to support specified filtering of L1-RSRP for LTM. Therefore, when RAN4 defining requirements, no L1 filtering should be considered.

Proposal 8: Not to consider L1 filtering in requirements for LTM.

# Summary

In this contribution, we further provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: For the first report after condition is met for periodic measurement reporting, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

Proposal 2: For aperiodic measurement reporting, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

- The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

Proposal 3: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements:

- No indicate TCI change during the evaluation period.

- No configuration change w.r.t measurement RS during the evaluation period

- After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Proposal 4: RAN4 to introduce requirements for reporting criteria for LTM and define a separate capability for supported number of L1 measurement reporting.

Proposal 5: Requirements for sharing the capabilities for reporting between L3 measurements and L1 measurements should also be considered.

Proposal 6: RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM firstly. For CSI-RS based L1 measurements for LTM, the reporting criteria can be discussed after critical issues are resolved.

Proposal 7: RAN4 to develop requirements for event triggered reporting for LTM and MIOM independently. No optimization is considered in this release.

Proposal 8: Not to consider L1 filtering in requirements for LTM.

- References

- R4-2502601	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple